3GPP TSG-SA2 Meeting #156E	S2-2304049

# Elbonia, April 17 – 21, 2023					       (revison of S2-XXXXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0330 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce Sidelink Based positioning procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  |  | Date: |  | 2023-04-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |  |
| Reason for change: |  | The study of Architecture Enhancement to support Ranging based services and sidelink positioning (FS_Ranging) has reached conclusions. It is conluded that “The LMF uses the location of Located UE(s) together with the ranging/SL positioning measurement data or result reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE.” This CR addresses this in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing Sidelink Positioning Procedure initiated by LMF for Target UE having NAS connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications to support Sidelink positioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11.Y(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

6.11.Y Sidelink Based Positioning Procedure with Target UE NAS connection

Figure 6.11.Y-1 shows a positioning procedure used by an LMF to support sidelink positioning and delivery of assistance data. The procedure is based on use of the LPP protocol defined in TS 37.355 [20] between the LMF and Target UE. The procedure is based on use of the SLPP protocol between the Target UE and Located UE. Procedure requires Target UE has NAS connection.

Editor's note:  SLPP protocol is developed currently by RAN. Procedure must be adjusted if it is necessary after RAN WGs complete SLPP protocol definition.

Figure 6.11.Y-1: Sidelink Positioning Procedure

1.	Steps 1-3 for UE Assisted and UE Based Positioning Procedure in Clause 6.11.1 are performed toward Target UE with the following differences:

- The LMF informs Target UE that ranging based positioning is required.

- The LMF may provide list of Located UE Application Layer ID(s).

NOTE 1: LMF may be provisioned with Located UEs or associated to PRUs with ranging capability.

2.	[Conditional] Target UE executes discovering and selection of Located UE based on method defined in clause 5.2.2 of TS 23.586 [40] if not discovered yet.

3.	[Conditional] Target UE may share any assistance data to Located UE provided in the Downlink Positioning message and performs any sidelink based positioning measurements and/or location computation with Located UE requested by the Downlink Positioning message.

4.	[Conditional] Steps 5-7 for UE Assisted and UE Based Positioning Procedure in Clause 6.11.1 executed with the following differences:

- Location information may contain ranging related location information.

- Location information may contain the identity and location of Located UE.

NOTE 2: The located UE Application Layer ID will be used in UE discovery. Details of located UE Application Layer ID is defined in TS 23.586 [40].

Editor's note: 	Security aspects will be determined in SA WG3.

* * * * End of Changes * * * *