3GPP TSG SA WG2 Meeting #156-E	S2-2304115

# Electronic meeting, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2084 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Differentiation for NAUN3 devices behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Release 18, differentiated QoS is provided for NAUN3 devices behind 5G-RG. The conclusion of the study for NAUN3 devices behind RG in TR 23.700-17 includes: - The NAUN3 devices may be grouped together - URSP may be sent to the 5G-RG to identify the mapping between the groups and DNN/S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | QoS differentiation for NAUN3 devices is presented in a new section 4.10b. In clause 9.5.2, it is added that the URSP rules may contain mapping between the Connectivity Group ID and the DNN/S-NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not capture features defined in R18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10b (new), 9.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 4.10b 	NAUN3 devices behind 5G-RG

A 5G-RG may provide connectivity for Non-Authenticable Non-3GPP (NAUN3) devices.

To provide differentiation between different types of NAUN3 devices, each NAUN3 device may connect to a (logical) port on the 5G-RG (e.g., Ethernet port or SSIDs), where each logical port is associated with a Connectivity Group. Connectivity Groups may be formed based on 5G-RG’s logical ports, and the devices that connect to a certain logical port are considered part of the same connectivity group. The 5G-RG is locally configured with the mapping between a (logical) port and an operator defined Connectivity Group ID. How this configuration is done is out of scope of this specification. The traffic to and from the devices in a Connectivity group traverse a single PDU Session established by the 5G-RG for a DNN/S-NSSAI, as illustrated in Figure 4.10b-1.

How the NAUN3 devices are configured to connect to a 5G-RG logical port (e.g. SSID) is out of scope of this specification.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG

QoS differentiation for different types of NAUN3 devices can be provided by e.g., associating different QoS information for different DNN, S-NSSAI.

**** Next Change ****

### 9.5.2	UE access selection and PDU Session selection related policy information

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

If the PCF provides the URSP policy to the 5G-RG, the PCF should neither include NSWO indication nor any ANDSP policies. The 5G-RG shall ignore any NSWO indication or any ANDSP policies if received from the 5GC. The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

URSP rules may indicate the association between the traffic from NAUN3 devices and DNN and S-NSSAI, where Connectivity Group ID, which identifies a type of device network behind 5G-RG (see 4.10b), may be used as Traffic Descriptor.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

### **** End of Changes ****