3GPP TSG SA WG2 Meeting #156-E	S2-2304119

# Electronic meeting, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2086 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for AF influence on TNAP ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions for UE behind RG in TR 23.700-17 states that: “(for Trusted access) To determine whether an UE is to be considered as a guest on a wireline access: - The TNAP Id of the 5G RG is provided by the 5G RG to the TNGF over Ta and then by the TNGF to the PCF of the UE as ULI carried via AMF and SMF; this is working as currently defined in TS 23.501 [2] / TS 23.502 [3]. - Policy subscription control data in UDR (as defined in TS 23.503 [4], Table 6.2-2: PDU Session policy control subscription information) may contain the TNAP Id(s) that is/are associated with a UE's subscription. - An AF may provide TNAP ID for a subscriber (GPSI) which is stored as Application data in UDR. - The PCF may use the TNAP Id of the 5G RG received over ULI and Policy subscription control data and Application data in UDR (as defined above).” i.e. the exposure framework should be enhanced to allow an AF to provide a TNAP ID for a subscriber (GPSI) which is stored as Application data in UDR. The PCF will read this data in UDR and use it for policy decisions when a UE connects via trusted non-3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the Service Specificv Parameter Provisioning procedure to allow an AF to provide TNAP ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications not aligned with study conclusions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.10.X	AF-based service parameter provisioning for TNAP ID

To support location dependent policies when a UE connects using trusted non-3GPP access procedures via a TNAP collocated with a 5G-RG, as described in Figure 4.10-1, an AF may provide one or more TNAP IDs for a UE or a group of UEs.

NOTE: 	A TNAP IDs provided by an AF can e.g. refer to a TNAP that is collocated with a 5G-RG where the UE is belonging to the same household.

The guidance sent by the AF is sent to the HPLMN of the UE and may apply to a single UE identified by GPSI or a group of UEs identified by a Group Id. The request cannot be sent with Any UE as a target.

For TNAP service parameter provisioning (i.e., creating, updating and deleting), the procedure defined in TS 23.502 [x], clause 4.15.6.7 is performed with the following considerations:

-	Service Description contains an AF service Identifier indicating that the request is for providing TNAP information.

-	Service Parameters include TNAP ID(s).

The PCF compares the TNAP ID provided by the AF with the TNAP ID received in the User Location Information when the UE connects via trusted non-3GPP access. The PCF may, e.g.y, apply different policies depending on whether UE is at the TNAP/RG indicated by the AF or not. In case PCF has both subscribed TNAP ID and AF-provided TNAP ID, the PCF decides based on configuraton whether to apply both or one of them.

**** End of Changes ****