3GPP TSG-SA WG2 Meeting #156 (e-meeting)	 S2-2304150

# Elbonia, April 17 – 21, 2023																		(rev of )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0208 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 Associating location dependent MBS Sessions delivering the same content during network sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are two ENs (see below from clauses 6.5.5 and 6.18) on how to address resource efficiency in network sharing for location dependent MBS service Editor’s Note: For location dependent broadcast service, whether the Associated Session ID is sufficient for the NG-RAN to identify multiple broadcast MBS Sessions via different CNs delivering the same content is FFS. Editor´s notes: How to support multiple broadcast MBS Sessions via different CNs to deliver the same content for location-dependent MBS sessions is FFS. Two main options were discussed in SA2#154AHe & SA2#155: Option-1: NG-RAN based solution, i.e., the NG-RAN correlates the multiple location dependent MBS sessions via different CNs delivering the same content based also on MBS Service Area Option-2: AF based solution, i.e. the NG-RAN correlates the multiple location dependent MBS sessions via different CNs delivering the content based also an additional Content ID assigned by the AF and included as part of Associated Session ID For Option-1, Additional collaboration effort is required among operators participating network sharing to ensure same configuration of list of cells and/or list of TAIs in the MBS service areas where the same content is delivered. Such collaboration effort is unnecessary and can be very challenging. Besides, even if the same configuration above could be ensured by the collaboration among operators, it is quite cumbersome for NG-RAN to correlate the location dependent sessions by checking whether there is an exact match of list of Cell IDs (with 36-bit NCI) and/list of TAIs (with 3-octet TAC) among the location dependent sessions. For Option-2, the AF includes an additional Content ID as part of the Associated Session ID. The solution is quite straightforward, and the impact to NG-RAN and to the AF is very limited. Besides, this option offers similar network sharing solution for both location dependent MBS and non-location dependent MBS. Based on the above, it is proposed to adopt Option-2. DP S2-2304149 may be referenced for more discussion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the ENs introducing additional Content ID assigned by AF and included as part of Associated Session ID to address resource efficiency in network sharing for location dependent MBS service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remain unresolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5, 6.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.5.5	Associated Session ID

In the case of network sharing, an Associated Session ID may be used as specified in clause 6.17. When the AF creates multiple broadcast MBS Sessions via different CNs to deliver the same content, it may provide the Associated Session ID which enables the NG-RAN to identify the multiple MBS Sessions delivering the same content.

Source Specific IP Multicast Address specified in clause 6.5.3 may be used as Associated Session ID. For location dependent broadcast service, the Associated Session ID includes additional Content ID assigned by the AF and representing the content to be distributed in a specific MBS Service Area.

* * * Next Changes * * *

## 6.18	Resource sharing across broadcast MBS Sessions during network sharing

In network sharing scenario as specified in clause 5.18 of TS 23.501 [2], the same MBS broadcast service may be delivered via multiple operators' CN participating in the network sharing to a shared NG-RAN, and the shared NG-RAN nodes may broadcast the MBS data only once for resource efficiency.

When the AF creates multiple broadcast MBS sessions via multiple CNs to deliver the same content, the shared NG-RAN allocates radio resource for one of broadcast MBS Sessions instead of allocating radio resource for all the broadcast MBS Sessions.

NOTE 1:	The same QoS requirements are assumed to be provided by the AF for the broadcast MBS Sessions via multiple CNs delivering the same content.

The NG-RAN determines the broadcast MBS sessions delivering the same content in one of the following ways:

-	Based on the Associated Session ID (see clause 6.5.X) provided by the AF to the NG-RAN via 5GCs when creating broadcast MBS sessions. or

-	Based on the association of MBS session identifiers (i.e. TMGIs) configured in NG-RAN, the shared NG-RAN nodes can determine that the multiple broadcast MBS sessions are transmitting the same content for the same MBS service. For the location dependent MBS session, the existing MBS session identifiers are used to identify multiple broadcast MBS Sessions via different CNs delivering the same content.

NOTE 2:	One possibility for configuring the association between TMGIs is configuring a range of TMGIs for each PLMN participating in network sharing, and associating TMGIs with the same offset in the ranges. Example for TMGI range mapping: If the PLMN 1 TMGI range x1 to xn is configured to be mapped to TMGI range y1 to yn in PLMN 2, x1 is mapped to y1, x2 is mapped to y2, and so forth.

Editor's note:	For the association of MBS session identifiers (i.e. TMGIs) configured in NG-RAN, it is FFS whether AFs can provide additional information (e.g. TMGI index) to request the allocation of a TMGI from a range of TMGIs for shared MBS services in an MB-SMF.

NOTE 3:	When the association of MBS session identifiers is configured in NG-RAN, there is no requirement on the AF to provide an Associated Session ID.

Illustrated in Figure 6.18-1 is an example that the AF creates broadcast MBS Sessions via 5GC Operators A, B and C respectively to deliver the same content and N3mb unicast transport is used from 5GC to the NG-RAN. Based on operator policy in the NG-RAN node, the N3mb tunnel may be established from the 5GC of only one operator (i.e., Operator A in Figure 6.18-1) to the shared NG-RAN, or the N3mb tunnels may be established from the 5GCs of all the operators to the shared NG-RAN. Over the Uu interface, the NG-RAN allocates radio resource for only one of the established broadcast MBS Sessions regardless of the number of N3mb tunnels established to deliver the MBS packets.

Figure 6.18-1: Example of Resource sharing across multiple broadcast MBS Sessions via different CNs to deliver the same content during network sharing

* * * End of Changes * * *