3GPP TSG-SA2 Meeting #156-e	S2-2304280

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0248 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provisioning for traffic type of L3 U2U Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For L3 U2U Relay, different traffic types(i.e. IP, Ethernet, Unstructured ) need be reflected in the U2U Relay discovery and U2U Relay communication procedures for each RSC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the traffic type used for each RSC in the provisioning for L3 U2U Relay and End UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Traffic types for the 1st hop and 2nd hop may not align. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

5.1.5.1	Policy/Parameter provisioning for 5G ProSe UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 End UEs accessing UE-to-UE Relays over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe UE-to-UE Relay also authorizes the use of 5G ProSe UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

Editor's note:	The policy/parameter provisioning corresponding to 5G ProSe UE-to-UE Relay Communication with integrated discovery is FFS.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and message and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 2:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay:

-	whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe End UE also authorizes the use of 5G ProSe UE-to-UE Relay discovery with Model A and Model B.

NOTE 3:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe UE-to-UE Relay Discovery as a 5G ProSe End UE and for enabling the UE to connect to the 5G ProSe UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	For 5G ProSe Layer-3 UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 4:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 5:	RAN WG2 will determine the default PC5 DRX configuration for 5G ProSe UE-to-UE relay scenario.

2)	Radio parameters for 5G ProSe UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

# * * * * End of changes * * * *