SA WG2 Meeting #S2-156-e	S2-2304351

# Elbonia, Apr 17 – 21, 2023, Electronic                                              (revision of S2-23XXXX)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4279 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN removal on KI#7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023--04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EN removal for KI#7 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor’s note removed and added normative text, removed duplicate text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open EN on KI#7 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Beginning of changes>>>

### 5.37.7	5GS Packet Delay Variation monitoring and reporting

#### 5.37.7.1	General

The 5GS Packet Delay Variation is the variation of packet delay measured between UE and PSA UPF. The AF may send the requirement for Packet Delay Variation monitoring to 5GS together with the requirement for packet delay measurement.

Based on the AF request for Packet Delay Variation monitoring, the PCF triggers the QoS monitoring procedure as described in clause 6.1.3.21 of TS 23.503[12], and gets the UL, DL or RT QoS Monitoring results from the SMF. The PCF derives the 5GS Packet Delay Variation based on the QoS Monitoring result(s) and then reports both packet delay measurements and Packet Delay Variation results to the AF/NEF. The PCF shall provision only one QoS Monitoring policy per PCC rule to get packet delay and derive Packet Delay Variation.

NOTE:	The derivation of 5GS Packet Delay Variation by PCF, based on QoS Monitoring, is implementation dependent. The Packet Delay Variation calculation method needs to be the same within the PLMN, based on operator policy.

## <<< End of changes >>>