3GPP TSG-WG SA2 Meeting #156E 	S2-2304386

April 17 – 21, 2023, Elbonia	(revision of S2-230xxxx)

Source:	Sony

Title:	Procedure for Ranging/SL Positioning service exposure though 5GC network.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper introduces a new Procedure for Ranging/SL Positioning service exposure though 5GC network.

# 1. Introduction/Discussion

This paper introduces a new Procedure for Ranging/SL Positioning service exposure though 5GC network.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

#### 6.6.1.2	Procedure for Ranging/SL Positioning service exposure though 5GC network

##### 6.6.1.2.1	General.

For Ranging/SL Positioning service exposure to a SL Positioning Client UE through network two alternative procedures may be used i.e., user plane-based procedure via NEF, or control plane-based procedure is used.

##### 6.6.1.2.2	Procedure for Ranging/SL Positioning service exposure though 5GC network via NEF.

Figure 6.6.1.2.2-1: Exposure procedure to Client UE via NEF

- The Client UE register and obtains the Onboarding Enrolment Information e.g., IP address and onboarding credentials and provides this to higher layer/application layer.

- The Application in Client UE performs the Onboarding procedure with the NEF as specified in TS 33.122 [11] clause 6.1 taking the role as API Invoker.

- The Application in Client UE invokes an Nnef_EventExposure_Subscribe service operation towards the NEF and includes the identification of the Target UE and Reference UE (e.g. SUPI or GPSI) and details of the Ranging/Sidelink location request such as whether a current or last know immediate location or a deferred location is requested, the location accuracy and response time, maximum age of location, LDR request information and other information applicable to the type of request.

- The NEF invokes an Ngmlc_Location_ProvideLocation Request service operation towards the GMLC. The service operation may include all of the information received in step 3.

- The GMLC triggers the SL-MT-LR procedure as specified in TS 23.273 [8] clause 6.y and obtains the Ranging/Sidelink positioning result.

- The GMLC invokes the Ngmlc_Location_ProvideLocation Response service operation towards the NEF to return the Ranging/Sidelink positioning result.

- The NEF returns the Ranging/Sidelink positioning result to the Application in the Client UE.

# * * * * End of changes * * * *