3GPP TSG-WG SA2 Meeting #156e	S2-2304438

Elbonia, April 17 – 21, 2023	(revision of S2-230xxxx)

# Title:	[DRAFT] LS Reply on Paging Policy Information for Network Triggered Connection Resume

# Response to:	LS (S2-2303936/C4-230533) on Paging Policy Information for Network Triggered Connection Resume from CT4

# Release:	Release 18

# Work Item:	NR_REDCAP_Ph2

Source:	[Huawei to be] SA2

To:	3GPP CT4, 3GPP RAN3

Cc:	-

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven dot wenham at huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.502 CR#4007

1. Overall Description:

SA2 thanks CT4 for the LS on Paging Policy Information for Network Triggered Connection Resume.

SA2 has agreed the CR as attached to introduce the PPI, the ARP and the 5QI of the QoS Flow of the PDU Session into the Namf_MT_EnableUERechability request which triggered the service operation and also include the above information into the N2 message when AMF triggers the RAN paging to enable the paging policy differentiation for the RAN paging.

SA2 believes it is in RAN3’s scope to determine what information is required, and would like to confirm feasibility, necessary and whether the information being provided is the right information with RAN3. SA2 asks RAN3 to provide their view.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 group to take the above into account.

To RAN3 group.

ACTION: 	SA2 kindly asks RAN3 group to take the above into account and provide their view.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157		May 22 – 26, 2023				Berlin, DE

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE