3GPP TSG-WG SA2 Meeting #156-e	S2-2304443

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4310 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Address EN for LADN aspects for SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To solve the EN: Editor’s note: The SMF does not need to know whether there is a LADN Service Area for a DNN/S-NSSAI, SMF does not need to know the content of the LADN Service Area. Whether this should be reflected in the data provided by UDM is FFS. There are two options to solve the EN: Option#1: the subscribed LADN service area delivered to the SMF is treated an indication for SMF to activate the LADN per DNN and S-NSSAI, i.e. subscription to "UE mobility event notification" event per DNN and S-NSSAI to receive the notification about the UE presence in the LADN service area, the SMF does not look into the content of the subscribed LADN service area. Option#2: UDM sends an indication that LADN service area per DNN and S-NSSAI is activated to the SMF, instead of sending the content of the subscribed LADN service area. With the explicit indication, the SMF subscribes to "UE mobility event notification" event per DNN and S-NSSAI to receive the notification about the UE presence in the LADN service area . Either option works, but option #1 is preferred considering the system impacts. Besides, the when the SMF subscribes to "UE mobility event notification" per DNN and S-NSSAI, the SMF also needs to be aware whether the AMF is excuting LADN Service Area per DNN and S-NSSAI for the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor’s note by clarifying that the SMF subscribes to "UE mobility event notification" per DNN and S-NSSAI with the knowledge of LADN service area per DNN and S-NSSAI, and the AMF is performing LADN per DNN and S-NSSAI for the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved issues remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20b.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.6.5a	Supporting LADN per DNN and S-NSSAI

The 5GS may support LADN per DNN and S-NSSAI, and the functions specified in clause 5.6.5 are used (with the extension to apply per DNN and S-NSSAI whenever applicable) with the following enhancements:

-	The UE indicates the support of LADN per DNN and S-NSSAI in the UE MM Core Network Capability of the Registration Request message.

-	The LADN Service Area can be provisioned for a group (e.g. 5G VN group) or an individual subscriber using UDM/NEF parameter provisioning service triggered by AF request as described in clause 4.15.6 of TS 23.502 [3].

-	The LADN Service Area is determined by AMF, based on the local configuration and/or the subscribed LADN service area received from UDM.

-	If UE indicates the support of LADN per DNN and S-NSSAI, the AMF provides the LADN Service Area Information per LADN DNN and S-NSSAI to the UE.

-	The AMF enforces the LADN Service Area per LADN DNN and S-NSSAI in the same way as is described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI and the AMF sends an indication to SMF that LADN Service Area Information per LADN DNN and S-NSSAI is activated for the UE.

-	The UE enforces the LADN Service Area per LADN DNN and S-NSSAI, if received from the AMF, in the same way as is described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI.

# * * * * Next change * * * *

### 5.20b.2	Support service area for a group

The procedure as defined in clause 4.15.6.2 of TS 23.502 [3] is applicable for provisioning of service area for a group with the following clarifications and enhancements:

-	The AF request additionally contains the LADN service area as part of DNN and S-NSSAI specific Group Parameters, and the service area is stored in UDR as subscription data and delivered to AMF as well as SMF.

-	If the AF provides the LADN service area in the form of geographical information, the NEF maps the geographical information to a list of TAs before sending the service area to the UDM.

LADN per DNN and S-NSSAI as defined in clause 5.6.5a is applicable for enforcement of LADN service area for a group with the following clarifications and enhancements:

-	If the group is provisioned with the LADN service area, the AMF configures the DNN of the group as LADN DNN and determines the LADN service area based on local configured LADN service area corresponding to the DNN (if any) and the service area received from UDM.

Editor's note:	If there is no overlapping between the LADN service area configured in AMF and service area received from UDM, how to handle this case is FFS.

-	If the group is provisioned with the LADN service area, the SMF configures the DNN of the group as LADN DNN. When the SMF receives a request for PDU Session Establishment with the LADN DNN and the indication that LADN Service Area Information per LADN DNN and S-NSSAI is activated for the UE, it shall subscribe to "UE mobility event notification" for reporting UE presence in Area of Interest by providing LADN DNN and S-NSSAI to the AMF as described in clauses 5.6.11 and 5.3.4.4

# * * * * End of changes * * * *