3GPP TSG-WG SA2 Meeting #156E	S2-2304466

# E-meeting, April 17–21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4320 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN Removing and Clarifications of PDV for KI#7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 5.37.7 of TS 23.501, there is an EN to resolve about QoS monitoring. Also some changes are needed for clarification and text alignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the EN and make some text updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN not resolved and text unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

5.37.7	5GS Packet Delay Variation monitoring and reporting

The 5GS Packet Delay Variation (PDV) is the variation of packet delay measured between UE and PSA UPF. The AF may send the requirement for PDV monitoring to PCF directly or via NEF.   The 5GS can utilize packet delay measurement mechanism, described in clause 5.33.3, to derive 5GS PDV.  Based on the QoS monitoring mechanism, the 5GS PDV is derived by the PCF and then reported to the AF.

Upon AF request for Packet Delay Variation monitoring, the PCF triggers the QoS monitoring procedure as described in clause 6.1.3.21 of TS 23.503[12], and gets the UL, DL or RT QoS Monitoring results from the SMF. The PCF derives the 5GS PDV based on the QoS Monitoring results and then reports to the AF/NEF both packet delay measurements and 5GS PDV.

The PCF only use one QoS monitoring policy per PCC rule for packet delay and PDV.

NOTE:	The derivation of 5GS Packet Delay Variation by PCF, based on QoS Monitoring, is implementation dependent. The Packet Delay Variation calculation method needs to be the same within the PLMN, based on operator policy.

# * * * * End of changes * * * *