3GPP TSG-WG SA2 Meeting #156-e 	S2-2304509

Elbonia, April 17 – 21, 2023	(revision of S2-2202665)

Source:	Huawei, HiSilicon

Title:	Procedure for Ranging/SL Positioning

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to add the description on procedure for Ranging/SL Positioning based on the conclusion in TR 23.700-86.

1. Introduction/Discussion

This paper proposes to add the description on procedure for Ranging/SL Positioning based on the conclusion in TR 23.700-86.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change (all new)* * * *

## 6.X	Procedure for Ranging/SL Positioning

This clause describes the basic procedure for ranging/SL Positioning, used for the determination of the distance and/or the direction between two UEs (i.e. Target UE and Reference UE).

Figure 6.X-1: Procedure for Ranging/SL Positioning

1.	UE1 and UE2 may get the ranging authorization policy and parameters from PCF during the registration procedure. The ranging authorization policy and parameters may include whether the UE is authorized as Reference UE or Target UE.

2.	UE1 gets the ranging request from the application layer, and the ranging request includes Application Requirements (e.g., one time or periodic ranging, ranging for distance or direction measurement or both).

3.	UE1 performs Ranging/Sidelink Positioning device discovery, as described in clause 6.4.

4.	UE1 and UE2 perform the RSPP related procedures to exchange the coordination information, e.g., Ranging/Sidelink Positioning capability, Ranging/Sidelink Positioning assistant data.

Editor's note:	It is FFS whether additional procedure between step 3 and step 4 to initiate the ranging session is needed.

5.	The Ranging/SL Positioning layer of each UE provides the ranging configuration to the AS layer. The ranging configuration includes the Ranging role (Reference UE or Target UE), one time or period ranging, ranging for distance or direction measurement or both.

6.	The ranging operation is performed in the AS layer of each UE.

7.	UE1 or UE2 performs the ranging result calculation, and the ranging result calculation may be shared between the UEs.

# * * * * End of changes * * * *