3GPP TSG-WG SA2 Meeting #156-e	S2-2304538

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0762 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AnLF Accuracy Monitoring Description and Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#1 in TR 23.700-81 define that NWDAF can have the accuracy checking capability and consumers of NWDAF can request the accuracy information to such NWDAFs with such capability. Such definitions need to be included in NWDAF capabilities, procedures, and services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the description of NWDAF providing accuracy information about analytics IDs and/or ML models to consumers. It has been agreed that NWDAF has the accuracy checking capability. Therefore the descriptions of NWDAF capabilities as well as how NWDAFs are discovered are updated. Updating NWDAF services with the accuracy related information in the subscription and notification services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 agreements on KI#1 will not be properly captured in R18 NWDAF description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 5.2, 5C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.1	General

The NWDAF (Network Data Analytics Function) is part of the architecture specified in TS 23.501 [2] and uses the mechanisms and interfaces specified for 5GC in TS 23.501 [2] and OAM services (see clause 6.2.3.1).

The NWDAF interacts with different entities for different purposes:

-	Data collection based on subscription to events provided by AMF, SMF, PCF, UDM, NSACF, AF (directly or via NEF) and OAM;

-	[Optionally] Analytics and Data collection using the DCCF (Data Collection Coordination Function);

-	Retrieval of information from data repositories (e.g. UDR via UDM for subscriber-related information);

-	Data collection of location information from LCS system;

-	[Optionally] Storage and retrieval of information from ADRF (Analytics Data Repository Function);

-	[Optionally] Analytics and Data collection from MFAF (Messaging Framework Adaptor Function);

-	Retrieval of information about NFs (e.g. from NRF for NF-related information);

-	On demand provision of analytics to consumers, as specified in clause 6.

-	Provision of bulked data related to Analytics ID(s).

-	Provision of Accuracy information about Analytics ID(s).

-	Provision of ML model accuracy degradation about a ML Model.

A single instance or multiple instances of NWDAF may be deployed in a PLMN. If multiple NWDAF instances are deployed, the architecture supports deploying the NWDAF as a central NF, as a collection of distributed NFs, or as a combination of both. If multiple NWDAF instances are deployed, an NWDAF can act as an aggregate point (i.e. Aggregator NWDAF) and collect analytics information from other NWDAFs, which may have different Serving Areas, to produce the aggregated analytics (per Analytics ID), possibly with Analytics generated by itself.

NOTE 1:	When multiple NWDAFs exist, not all of them need to be able to provide the same type of analytics results, i.e. some of them can be specialized in providing certain types of analytics. An Analytics ID information element is used to identify the type of supported analytics that NWDAF can generate.

NOTE 2:	NWDAF instance(s) can be collocated with a 5GS NF.

# * * * * Second change * * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with Accuracy checking capability, the consumer may query NRF providing also the accuracy checking capability in the discovery request.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types, the consumer may query NRF providing the NF Set IDs or NF types in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	an NWDAF containing MTLF shall include the ML model provisioning services (i.e. Nnwdaf_MLModelProvision, Nnwdaf_MLModelInfo) as one of the supported services during the registration in NRF when trained ML models are available for one or more Analytics ID(s). The NWDAF containing MTLF may provide to the NRF a (list of) Analytics ID(s) corresponding to the trained ML models and possibly the ML Model Filter Information for the trained ML model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. For each Analytics ID, if the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF may also include, in the registration to the NRF, an ML Model Interoperability indicator.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (i.e. an NWDAF containing AnLF) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required, ML Model Interoperability indicator and NF consumer information. The NRF returns one or more candidate for instances of NWDAF containing MTLF to the NF consumer and each candidate for instance of NWDAF containing MTLF includes the Analytics ID(s) and possibly the ML Model Filter Information for the available trained ML models, if available.

NOTE 5:	NF consumer information such as Vendor ID is defined in stage 3.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with accuracy checking capability, the consumer may query NRF also providing the accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Federated Learning (FL) capability via NRF:

- An NWDAF containing MTLF supporting FL as a server shall additionally include FL capability type (i.e. FL server), Time interval supporting FL as FL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include FL capability type (i.e. FL client), Time interval supporting FL as FL capability information during the registration in NRF, and it may also include, NF type(s) where data can be collected as input for local model training.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) includes in the request the FL capability type as FL server, Time Period of Interest and ML model Filter information for the trained ML model(s) per Analytics ID(s), if available. The NRF returns one or more candidate for instances of NWDAF containing MTLF as FL server to the consumer.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) includes in the request FL capability type as FL client, Time Period of Interest, ML model Filter information for the trained ML model(s) per Analytics ID(s), a list of NF type(s). The NRF returns one or more candidate for instances of NWDAF containing MTLF as FL client to the consumer.

NOTE 6:	An NWDAF containing AnLF does not support discovering an NWDAF containing MTLF with FL capability.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming entry capability to request analytics or input data is discovered via the NRF. An NWDAF discovers the NWDAF in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

Editor's note:	Its is FFS if the NWDAF with roaming entry capability will be slice-specific, and if so how slices are used in the discovery.

# * * * * Third change * * * *

5C.1	General

A NWDAF has the accuracy checking capability of Analytics and/or ML Model, where the NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes.

Input data is collected from Data Producer NF when there is a request for inference/prediction per analytics ID in NWDAF for specific time period in future while ground truth data are collected from Data Producer NF corresponding to analytic ID requested at the time which the prediction refers to.

NOTE:	The ground truth data is the actual measured data observed at the time which the prediction refers to.

Analytics/ML Model Accuracy Monitoring is to be achieved by comparing the predictions using the current trained ML model and its corresponding ground truth data i.e. the corresponding true observed events.

The NWDAF (containing AnLF/MTLF) with accuracy checking capability decides to initiate analytics accuracy monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

The AnLF with accuracy checking capability as defined in clause 6.2D is able to provide or notify the accuracy information of Analytics IDs to the analytics consumers of such service and when the analytics accuracy does not meet the analytics consumer's requirements, the analytics consumer may stop using analytics for a period of time or obtain new analytics. In addition, updated analytics for the provided Analytics IDs may be provided to analytics consumers as requested, if the updated analytics is able to be generated within the correction time period. The AnLF with accuracy checking capability as defined in Clause 6.2D.1 is able to determine analytics accuracy information based on e.g.,:

-	Comparing predictions and its corresponding ground truth data

-	Determining accuracy by comparing analytics accuracy using multiple ML models

The MTLF with accuracy checking capability as defined in clause 6.2E is able to determine ML Model degradation based on e.g.:

-	the data including input data, analytics output and the ground truth data either collected from various data source NFs, DCCF, AnLF, ADRF or configured by OAM;

-	or AnLF providing notifications of the accuracy feedback or analytics feedback information of the analytics generated by the ML model.

The NWDAF containing MTLF may reselect a new ML model or retrain the existing ML model and consequently notify the ML model accuracy degradation to the ML Model consumer(s). In addition, the NWDAF containing MTLF may consider the rating of untrusted AF(s) when used as data sources.

# * * * * End of changes * * * *