3GPP TSG-SA2 Meeting #156-e	S2-2304547

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4340 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for local routing of PIN traffic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The clause session management enhancements for PIN currently has the following Editor’s Note Editor's note: How and whether to handle the case where PINs share a PDU session and local switching is FFS Also, the session management related policy control clause refers to the communication between PINEs via 5G core network when PINEs are connected to different PEGCs. However, there is a need to clarify how this is realized by reusing local routing procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes the following: i). PIN that requires local routing of traffic has a dedicated PDU Session and removes the EN ii). URSP rules guide the PIN traffic to the dedicated PDU Session iii). SMF may preferentially select the same UPF for PDU Sessions from PEGCs that require local routing iv). The N4 rule attribute Network Instance is used in addition to UPF internal interface for local routing and it is set to the value of PIN internal identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The local routing of PIN traffic in 5GC is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1, 5.44.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * 1st change * * * *

#### 5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC. One PIN may be served by more than one PDU sessions in the PEGC. URSP rules are configured for guiding PIN traffic that requires local switching to a dedicated PDU Session.

Editor's note:	One PIN served by more than one PDU sessions in PEGC is FFS.

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

# * * * 2nd change * * * *

#### 5.44.3.2	Session management related policy control

For PIN traffic via PEGC/5GC with PDU session, the 5GC supports the policy control. The policy control is based on session management procedures as specified in TS 23.502 [3] and TS 23.503 [45].

A PEGC or an AF may provide QoS flow parameters to 5GC. An AF supports PIN service may also influence traffic routing for PDU sessions for PIN traffic. The PIN traffic can be categorised into following types:

-	Between two PINEs, which is via 5G core network when the two PINEs connect to different PEGCs.

-	Between PINE and PEMC via a PEGC and 5G core network.

-	Between PINE and DN via a PEGC and 5G core network.

-	Between PEGC and DN via 5G core network.

The SMF may prefer to select a single PSA UPF for PDU Sessions from PEGCs of a PIN to implement local switch on the UPF. The local routing for traffic is supported as described in clause 5.8.2.13.0. To support multiple PINs in a PLMN, the N4 rule attribute Network Instance is used in addition to the UPF internal interface and set to a value corresponding to the PIN internal identifier.

Editor's note:	The handling of PIN deletion/deactivation/activation is FFS.

# * * * * End of change * * * *