3GPP TSG-SA WG2 Meeting #156e	S2-2304582

# April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0227 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility Registration Update and Service Request procedures for location dependent multicast service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Mobility Registration Update procedure and Service Request procedure for location dependent multicast session need to be enhanced taking into account of mobility of UE in RRC_INACTIVE state receiving multicast MBS session data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add enhancements to Mobility Registration Update procedure and Service Request procedure for location dependent multicast session taking into account of mobility of UE in RRC_INACTIVE state receiving multicast MBS session data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Fail to support location dependent multicast service in Mobility Registration Update procedure and Service Request procedure, with mobility of UE in RRC_INACTIVE state receiving multicast MBS session data. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4.2.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.2.4.2.y	Mobility Registration Update and Service Request procedures

For mobility of UE in RRC_INACTIVE state receiving multicast MBS session data, the Mobility Registration Update procedure and Service Request procedure are performed as defined in clause 7.2.3.8 with the following additions:

-	upon receiving Nsmf_PDUSession_UpdateSMContext Request from the AMF, the SMF checks whether the UE is in MBS service areas of the location dependent MBS session by comparing the User Location Information of the UE provided by the AMF with the MBS service areas received from the NRF.

-	If the UE is in a service area of the MBS session:

-	the SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast MBS session information which additionally includes the Area Session ID.

-	the SMF may subscribe at the AMF using the Namf_EventExposure service to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. The SMF supplies all MBS service areas of the location dependent MBS session as Area Of Interest.

-	the shared delivery (if not already established) is established if the NG-RAN serving the UE supports MBS, or the individual delivery is established if the NG-RAN node does not support MBS, as specified in clause 7.2.4.2.1.

-	If the UE is out of all the MBS service areas of the MBS session:

-	If the NG-RAN node does not support MBS, the SMF removes the associated QoS flow information (if any) from the associated PDU session context which is provided to the NG-RAN using the Nsmf_PDUSession_UpdateSMContext response.

-	Per operator's policy (e.g. when a local configured timer expires since the UE left the whole MBS service area of the MBS session) the SMF may remove the UE from the MBS session as defined in clause 7.2.2.3. When the UE is removed from the location dependent MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event.

* * * End of Changes * * * *