3GPP TSG-WG SA2 Meeting #156-e	S2-2304609

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0772 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refinement for FL procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Uniformed terms (i.e. FL server NWDAF and FL client NWDAF) are defined to call FL entities in other paragraghs, but in clause 6.2C.2, wording alignment is missing, where server NWDAF and client NWDAF are used. - In the past meetings, it has been discussed and approved that NWDAF containing MTLF can be consumer of ML model, and a new service, i.e. Nnwdaf_MLModelTraining is defined for this purpose. however, this is missed in procedure of 6.2C.2.2; - A new concept model metric was brought in, but it is not clear what is it. Except for the accuracy, the other two parameters precision and recall have not been discussed before. For this reason, it is proposed to keep the accuracy as the interpretaion for model metric and delete the other ones,meanwhile, the “accuracy” is improved to “ML model accuracy” to keep alignment with the name defined in KI#1. - In the step 0 of clause 6.2C.2.2, the information that “time when analytics information is needed” will not be provided to FL server when requesting for an ML model, “time when ML model is needed” should be provided instead. In this case, “the consumer of the NWDAF containing AnLF sholud be deleted” from the text, because it will not be aware of ML modle retrieval. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Terminology alignment: Server NWDAF is changed to FL Server NWDAF, Client NWDAF is changed to FL Client NWDAF; - Clarify NWDAF containing MTLF can also be the consumer to retrieve a ML model from the FL server NWDAF; - Clarify “ML model accuracy” is the interpretation of model metric, and delete the other parameters(precision, recall) which have not been discussed and defined; - Delete the scenario that the consumer of the NWDAF containing AnLF provides the time when the ML model to the FL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned terms and wrong wording may lead to misunderstanding of the FL process. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2, 6.2C.2.1, 6.2C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2C.2	Procedures

#### 6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2C.2.1-1: Registration and Discovery procedure for Federated Learning

1-3.	NWDAF containing MTLF as FL Server NWDAF or FL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability information including FL capability Type (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

Editor's note:	Definition of FL capability and whether FL capability includes computation capability is FFS.

4-6.	NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as FL Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), check if the selected FL Server NWDAF is currently executing an Federated Learning procedure for the Analytics ID, Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as FL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, Data availability by the FL Client NWDAF, Time Period of Interest, Interoperability Indicator.

Editor's note:	Additional criteria for FL server to discover FL clients from the NRF (e.g. data availability) is FFS.

7.	FL Server NWDAF sends Federated Learning preparation request to the FL Client NWDAF(s) with Interoperability information. In the preparation request, the following parameters may be added: Available data requirement, availability time requirement.

Editor's note:	Other parameters in the preparation request and the definition of "Available data requirement" and "availability time requirement" are FFS.

8.	FL Client NWDAF(s) decides whether to join the Federated Learning process based on implementation. Example criteria used by a FL Client NWDAF(s) may be based on its availability, computation and communication capability, and Interoperability information.

9.	FL Client NWDAF(s) sends response to FL Server NWDAF indicating if it will join the FL procedure.

10.	FL Server NWDAF conducts selection of FL Client NWDAF(s).

# * * * * Next change * * * *

#### 6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF or NWDAF containing MTLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, using Nnwdaf_MLModelProvision service as defined in clause 7.5 or Nnwdaf_MLModelTraining_Subscribe Service as defined in clause 7.10, including Analytics ID, model metric (i.e. ML model accuracy), pre-determined status (e.g. requested ML Model Accuracy and/or total training time) and/or periodically reporting condition (e.g. training round/time).

NOTE 1:	The requested ML Model Accuracy can be used to indicate the target ML Model Accuracy of the training process, and the FL FL Server NWDAF may stop the training process when the requested ML Model Accuracy is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF) provides the time when the ML model is needed, the FL Server NWDAF can take this information into account to decide the maximum response time for its FL Client NWDAFs.

1.	FL Server NWDAF sends a request to the selected NWDAF containing MTLF (FL Client NWDAF) that participates in the Federated learning to perform the local model training for Federated Learning, including model metric. The request also includes the maximum response time before which the FL Client NWDAF has to report the interim local ML model information to the FL Server NWDAF.

2.	Each FL Client NWDAF collects its local data by using the current mechanism in clause 6.2.

3.	During Federated Learning training procedure, each FL Client NWDAF further trains the retrieved ML model from the FL Server NWDAF based on its own data, and reports interim local ML model information to the FL Server NWDAF. Each FL Client NWDAF also computes local model metric and includes it in the local training status report sent to the FL Server NWDAF, which optionally also contains other information about local training in the FL Client NWDAF, e.g. characteristics of local training dataset (e.g., area, sampling ratio, max/min of value of each dimension of data), etc.

NOTE 2:	The parameters in characteristics of local training dataset are up to the implementation.

The ML model, which is sent from the FL Client NWDAF(s) to the FL Server NWDAF during the FL training process, is the information needed by the FL Server NWDAF to build the aggregated model based on the locally trained models.

Editor's note:	The services (i.e. Nnwdaf_MLModelTraining, Nnwdaf_MLModelTrainingInfo) in steps 1, 3, 5a and 6 will be updated in the Figure 6.2C.2.1-1.

If the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the FL Server NWDAF, the FL Client NWDAF notifies the FL Server NWDAF on the delay event with proper cause information (e.g. local ML model training failure, more time necessary for local ML model training) before the maximum response time elapses.

Editor's note:	Further information on how the server NWDAF behaves based on the maximum response time from client NWDAF is FFS.

4.	The FL Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model. The FL Server NWDAF may also compute the global model metric, e.g. based on the local model metric(s) or by applying the global model on the validation dataset (if available).

If the FL Server NWDAF provides the maximum response time for the FL Client NWDAFs to provide the interim local ML model information in step 1, the FL Server NWDAF decides either to wait for the FL Client NWDAFs which have not yet provided their interim local ML model within the maximum response time or aggregates only the retrieved local ML model information instances to update global ML model. The FL Server NWDAF makes this decision, considering the notification from the FL Client NWDAF or, if the notification is not received, based on local configuration.

Editor's note:	It is FFS whether Server NWDAF solicits a response from an FL clients which has not reported the FL status within the maximum response time.

5a.	[Optional] Based on the consumer request, the FL Server NWDAF updates the global model metric to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the requested ML Model Accuracy is achieved or training time expires).

5b.	[Optional] Consumer decides whether the current model can fulfil the requirement, e.g. global model metric is satisfactory for the consumer. The consumer modifies subscription (stops or continues the training process) according to the consumer decision.

5c.	[Optional] According to the request from the consumer, FL Server NWDAF updates or terminates the current FL training process.

6.	If the FL procedure continues, FL Server NWDAF sends the aggregated ML model information to each FL Client NWDAF for next round model training.

7.	Each FL Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the FL Server NWDAF at step 6.

NOTE 3:	The steps 2-7 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the FL Server NWDAF may send the globally optimal ML model information to the consumer.

# * * * * End of changes * * * *