3GPP TSG-WG SA2 Meeting #156e	S2-2304672

# April 17th – 21st, 2023, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0023 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New shape to support hollow area |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current TS 23.032 has defined several shapes in subclause 5 such as circle, polygon, ellipsoid arc and so on. They can be used to describe a certain area by LCS client/AF. However, with the development of location accuracy and request granularity, LCS client/AF or UE may describe an area under the complex scenario. Some areas can not be covered by the existing shapes, e.g., hollow area as shown in the following figure. The area C contains all points inside A but outside B. (Area C = area A – area B). In order to address the above shape to describe a certain area, It is recommended to introduce a new shape. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify a new shape to describe the hollow area in the TS 23.032. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified how to describe a complex hollow area as the target area in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 7.2, 5.x(new), 7.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change  * * * *

5	Shapes

The intention is to incorporate a number of different shapes, that can be chosen according to need.

-	Ellipsoid Point;

-	Ellipsoid point with uncertainty circle;

-	Ellipsoid point with uncertainty ellipse;

-	Polygon;

-	Hollow;

-	Ellipsoid point with altitude;

-	Ellipsoid point with altitude and uncertainty ellipsoid;

-	Ellipsoid Arc;

-	High Accuracy Ellipsoid point with uncertainty ellipse;

-	High Accuracy Ellipsoid point with scalable uncertainty ellipse;

-	High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid;

-	High Accuracy Ellipsoid point with altitude and scalable uncertainty ellipsoid.

-Shapes relevant to Local Co-ordinates:

-	Local 2D point with uncertainty ellipse (only in 5GS);

-	Local 3D point with uncertainty ellipsoid (only in 5GS).

Each shape is discussed individually.

# * * * * Next change  * * * *

7.2	Type of Shape

The Type of Shape information field identifies the type which is being coded in the Shape Description. The Type of Shape is coded as shown in table 2a.

Table 2a: Coding of Type of Shape

| Bits |  |
| --- | --- |
| 4 3 2 1 |  |
| 0 0 0 0 | Ellipsoid Point |
| 0 0 0 1 | Ellipsoid point with uncertainty Circle |
| 0 0 1 1 | Ellipsoid point with uncertainty Ellipse |
| 0 1 0 1 | Polygon |
| 0 1 1 0 | Hollow |
| 1 0 0 0 | Ellipsoid point with altitude |
| 1 0 0 1 | Ellipsoid point with altitude and uncertainty Ellipsoid |
| 1 0 1 0 | Ellipsoid Arc |
| 1 0 1 1 | High Accuracy Ellipsoid point with uncertainty ellipse |
| 1 1 0 0 | High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid |
| 1 1 0 1 | High Accuracy Ellipsoid point with scalable uncertainty ellipse |
| 1 1 1 0 | High Accuracy Ellipsoid point with altitude and scalable uncertainty ellipsoid |
| other values | reserved for future use |

# * * * * Next change (all texts new)* * * *

5.x	Hollow

A hollow is an arbitrary shape described by one or more inner shapes and one outer shape with a certain inclusion relationship (in the example pictured in the drawing, points inside A but outside B). The inner shape is the proper subset of the outer shape and has no overlapping among several inner shapes. The minimum number of inner shapes allowed is 1, and the maximum number of inner shapes allowed is 15. The inner and outer shapes can be described as the circle (see clause 5.2), ellipse (see clause 5.3), or polygon (see clause 5.4).

Figure X: Description of a Hollow

# * * * * Next change (all texts new)* * * *

7.3.x	Hollow

Figure 7: Shape description of a hollow

The number of inner shape field encodes in binary on 4 bits in the description, and range from 1 to 15. The shape description field of a hollow consists of one outer shape field and at least one inner shape field, which are encoded as described in clause 7.1.

# * * * * End of changes * * * *