3GPP TSG-SA WG2 Meeting #156E	S2-2304797

# E-meeting, April 17 - 21, 2023													 (revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0263 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Multi-path |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As stated for the multipath pinciples usage: - The determination for multipath including the L3 relay without N3IWF establishment, the enhanced RSD “ProSe Multi-path Preference” and the path availability should be taken into account, this principle shoule be reflect in the clause 5.17.2 and 6.9.2. - For multipath including the L3 relay with N3IWF, only the ProSe Layer-3 Remote UE is ATSSS-capable, the multi-path transmission can be achieved using MA PDU Session in ATSSS features specified in clause 5.32 of TS 23.501 [4], this restriction should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that: - For L3 relay without N3IWF multipath establishement, RSD with "ProSe Multi-path Preference" as specified in 6.5.4 and the path availability should be taken into account by UE. - For L3 relay with N3IWF multipath establishement, it should emphasize that the L3 remote UE is ATSSS-capable. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current description on multipath pinciples usage for L3 relay are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.2, 6.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.17.2	Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay

A 5G ProSe Layer-3 Remote UE may access the network via a 5G ProSe Layer-3 UE-to-Network Relay with or without N3IWF.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, the multi-path establishment takes the enhanced RSD with "ProSe Multi-path Preference" as specified in 6.5.4 and the path availability into account, and the multi-path transmission via direct Uu path and via the UE-to-Network Relay is decided by the application layer mechanisms.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the Layer-3 Remote UE connection via Layer-3 UE-to-Network Relay with N3IWF is considered as "untrusted non-3GPP access to 5GC via N3IWF", if the 5G ProSe Layer-3 Remote UE is also ATSSS-capable, the multi-path transmission can be achieved using MA PDU Session in ATSSS features specified in clause 5.32 of TS 23.501 [4].

NOTE:	MA PDU Session requires ATSSS Information in SM subscription data as specified in TS 23.502 [5].

* * * * Start of Next Change * * * *

6.9.2	Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, if the UE decides to establish the multi-path via direct Uu path and via the UE-to-Network Relay by evaluating the matched URSP rule for application traffic and the path availability as specified in clause 6.5.4, the following is performed:

-	The UE sets up path over Uu by establishing a new PDU Session as specified in clause 4.3.2 of TS 23.502 [5] or modifying an existing PDU session as clause 4.3.3 of TS 23.502 [5].

-	The Remote UE establishes 5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF as specified in clause 6.5.1.1.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, if the UE decides to establish the multi-path via direct Uu path and via the UE-to-Network Relay, the UE establishes a MA PDU Session as specified in clause 4.22 of TS 23.502 [5].

* * * * End of Changes * * * *