3GPP TSG-SA WG2 Meeting #156e 	S2-2304812

April 17 – 21, 2023, e-meeting	(was)

Source:	vivo

Title:	TS 23.586: Procedures for SL positioning without NAS under no NF involvement

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to implement Procedures for SL positioning for target UE without NAS connection with located UE under NO NF involvement case.

1. Introduction/Discussion

This paper tries to implement Procedures for SL positioning for target UE without NAS connection with located UE under NO NF involvement case.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

6.5.2.2	Procedures for SL positioning without NAS connection when 5GC NF is not involved

Figure 6.5.2.2-1 illustrates the general Procedures for SL positioning without NAS connection and 5GC NF involvement.

Figure 6.5.2.2-1: Target UE derived positioning with Ranging involved Procedure

1)	When Target UE cannot establish NAS connection with AMF and to obtain its own location, the Target UE determines to use the Located UE’s assistant, then the Target UE performs the Located UE discovery and selection.

2)	Target UE sends a Ranging service request to the selected Located UE(s) to request for the location of Located UE(s).

3)	After receiving the Ranging service request, if the Located UE doesn’t have the available location of itself, the Located UE perform the MO-LR procedure as described in clause 6.2 of TS23.273[8].

4)	The Located UE(s) returns its location to the Target UE.

5)	The Target UE performs the Ranging and sidelink procedure with the Located UE(s), this step can be initiated after the step1.

6)	The Target UE derives its own location based on Ranging and Sidelink positioning measurement data/result and the location of the Located UE(s).

# * * * * End of changes * * * *