3GPP TSG-SA WG2 Meeting #156e 	S2-2304813

April 17 – 21, 2023, e-meeting	(was)

Source:	vivo

Title:	TS 23.586: EN resolve for NW assistant SL positioning without NAS

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper tries to implement the procedures for Ranging/Sidelink Positioning device discovery based on the conclusions of KI#3 in TR23.700-86.

1. Introduction/Discussion

Last meeting, Network assisted SL positioning has captured the case that the Target UE is not able establish NAS connection for Target UE postioning. Following FFS are left to be resolved

#1 Editor's note:	It is FFS if an enhanced 5GC-MO-LR message is sent by the Located UE.

#2 Editor's note:	It is FFS when AMF does not maintain target UE's context or GMLC cannot get target UE AMF address because of deregistration, whether and how location report from Located UE on behalf of Target UE is supported.

#3 Editor's note:	How LMF associates the ranging/SL positioning measurement or estimation result from different Located UEs is FFS.

#4 Editor's note:	It is FFS whether the pending 5GC-MT-LR also includes the 5GC-MT-LR.

For the first EN, this paper proposes to enhance the 5GC-MO-LR request message to include a new type of type of location service: Network assisted SL positioning, which indicates to response the Network assisted SL positioning result (i.e., the location of Target UE)

For the EN#2 and #4, this paper proposes to not consider the MT-LR case including the deferred MT-LR case, as the Target UE is out of coverage and unable to establish the NAS, the network will not keep the UE context, so it is hard to use the Located UE’s network report the Target UE position to the server, so these 2 ENs can be removed.

For the EN#3, each LMF is only responsible for its Located UE reported measurement data and returns the result to the Located UE, the results are gathered to the Target UE from the different Located UE(s), and the Target UE determines the final Ranging result based on the gathered results.

2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

# * * * * First change * * * *

### 5.5.3	Network assisted SL positioning without NAS connection

When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage the following principles are applied for Target UE to obtain its location:

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE may transmit its ranging measurements/results to the Located UE(s).

-	The Located UE may initiate the 5GC-MO-LR procedure with a new type of location service: Network assisted SL positioning, which indicates to response the Network assisted SL positioning result (i.e., the location of Target UE) .

-	The LMF performs the UE positioning procedure for Located UE if there is not available location of Located UE.

-	Located UE(s) reports the ranging/SL positioning measurement result to the LMF. This may include ranging measurements/results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

-	The LMF uses the location of Located UE together with the ranging/SL positioning measurement data or estimation results reported by the Located UEs to estimate the location of the Target UE and provides the resulting location to the Located UE

-	The Located UE(s) forwards the location received form the LMF to the Target UE.

-	The Target UE determines its final location if there are multiple locations received from the Located UE(s) based on implementation.

Editor's note:	Security aspects require confirmation from SA WG3.

# * * * * End of changes * * * *