3GPP TSG-SA WG2 Meeting #156E	S2-2304852

# E-meeting, April 17 - 21, 2023													 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4419 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PEGC operation related to PDU Session for PIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There may be the case that PEGC cannot establish a PDU Session to serve a PIN due to the NAS level congestion control. The congestion control related back-off timer provided by 5GC may be a bit long resulting in bad experience/performance for the PIN. In this case, it is considered beneficial from PIN management perspective to let the PEMC and AF of the PIN know that the PEGC cannot serve the PIN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.44.3.1 - Describe about how to handle the case that PEGC cannot establish a PDU Session for a PIN due to the NAS level congestion control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to handle the case that PEGC cannot establish a PDU Session for a PIN due to the NAS level congestion control is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC. One PIN may be served by more than one PDU sessions in the PEGC.

Editor's note:	How and whether to handle the case where PINs share a PDU session and local switching is FFS.

Editor's note:	One PIN served by more than one PDU sessions in PEGC is FFS.

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

If the PEGC cannot establish the PDU Session to serve a PIN due to the NAS level congestion control as specified in clause 5.19.7, the PEGC tries other URSP rules for the PIN in the order of Rule Precedence, if available, in order to establish a PDU Session, as specified in clause 6.6.2.3 of TS 23.503 [45]. If the PEGC fails to establish the PDU Session to serve the PIN, the PEGC informs the PEMC for the PIN that it cannot serve the PIN and the back-off time based on the back-off timer received from the AMF or the back-off timer received from the SMF. In this case, the PEGC may also provide the above information to the AF for the PIN if AF deployed.

NOTE:	When the PEMC/AF receive the information about that the PEGC cannot serve the PIN, how to manage the PIN is up to application logic, e.g. providing service parameters related to URSP rules for other PEGC or with other DNN/S-NSSAI by the AF to 5GC, reselecting a PEGC, switching to other PEGC, etc.

* * * * End of Changes * * * *