3GPP TSG-WG SA2 Meeting #156-e	S2-2304860

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0347 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF service operation to support UP Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align with the procedure of UP positioning between UE and LMF, the new LMF service operations need to be added in clause 8.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LMF service operations to support UP positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not support of UP positioning between UE and LMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.2.1, 8.3.2.x (new), 8.3.2.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3	LMF Services

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPNotify | Subscribe/Notify | AMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |

# * * * * Second change * * * *

8.3.2	Nlmf_Location service

8.3.2.1	General

Service description: This service enables an NF to request location determination for a target UE. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request the current geodetic and optionally local and/or civic location of a target UE.

-	Allow the consumer NF to subscribe/unsubscribe the geodetic and optionally local and/or civic location of a target UE for some certain events.

-	Allow the consumer NF to get notified about the geodetic and optionally local and/or civic location of a target UE when some certain events are detected.

-	Allows the consumer NF to cancel location event reporting for a target UE.

-	Allows the consumer NF to transfer location context information for location event reporting for a target UE.

-	Allows the consumer NF to request to set up, modify or terminate a secure LCS-UP connection for a target UE.

-	Allows the consumer NF to get notified about status or modification of a secure LCS-UP connection for a target UE.

The events to trigger location estimation notification are defined in clause 4.1a.5.1

# * * * * Third change * * * *

8.3.2.x	Nlmf_Location_ UPConfig service operation

Service operation name: Nlmf_Location_UPConfig

Description: Allow the consumer NF to request to set up or terminate a secure LCS-UP conncection for a target UE.

Input, Required: Notification Target Address, UE identifier (SUPI and if available GPSI).

Input, Optional: LCS-UP connection set up request indication, LCS-UP connection termination indication, AMF reallocation indication.

Output, Required: Success/Failure indication.

Output, Optional: None.

See clause 6.18.1, 6.18.2 and 6.18.3 for an example of usage of this service operation.

# * * * * Third change * * * *

8.3.2.y	Nlmf_Location_UPNotify service operation

Service operation name: Nlmf_Location_UPNotify

Description: Allow the consumer NF to subscribe and get notified about status or modification of a secure LCS-UP conncection for a target UE.

Input, Required: Notification Target Address, UE identifier (SUPI and if available GPSI).

Input, Optional: None.

Output, Required: LCS-UP status (established, released or modification indication).

Output, Optional: Target LMF identifier, modification reason.

See clause 6.18.1 and 6.18.3 for an example of usage of this service operation.

# * * * * End of changes * * * *