3GPP TSG-WG SA2 Meeting #156-e	S2-2304988

# Elbonia, April 17 – 21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1310 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Trigger and Control of DC Media Negotiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE can establish IMS data channel(s) in parallel to the establishment of an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to add IMS data channel(s) for the session. There is a concern that IMS data channel(s) established simultaneously with an IMS audio/video session may have impact to the IMS session setup time. Some data channel applications however are used at the alerting state and therefore require that the bootstrap data channel(s) and application data channel(s) are established before the IMS session is connected. Both situations need to be considered and settled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A solution is provided to address the concern on impact to the IMS session setup time and meet the requirement on supporting data channel applications used in alerting stage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Call setup time may be unnecessarily extended if IMS data channel is always established in parallel with the IMS audio/video session. On the other hand, some IMS data channel applications that require to be activated before the IMS audio/video session is connected will not work correctly if the IMS data channel is established after the IMS audio/video session is connected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.4	IMS DC Channel Setup

The following principles apply when an IMS Data Channel is established:

-	After UE completes the IMS DC capability negotiation with the IMS network as specified in Annex AC.8.1 of this document and receives the 200 OK response from the IMS network with the data channel capability indication in the Feature-Caps header field, it shall include data channel media description(s) alongside audio/video media desciptions in the initial INVITE request. Based on the operator’s policy, the IMS AS can proceed with the media negotiation procedure to setup the audio/video session with or without performing data channel bootstrapping.

NOTE 1:	For the network which need to provide data channel applications to its subscribers before the IMS audio/video session is connected, the IMS AS can proceed with the media negotiation procedure with performing data channel bootstrapping. For the network which concerns the IMS session setup time and has no data channel application provided to its subscribers, the IMS AS can only proceed with the audio/video media negotiation procedure and discard the data channel media.

-	After the audio/video session is connected, UE shall initiate a bootstrap data channel establishment request if any data channel media is not successfully negotiated in the initial INVITE procedure.

-	If a UE that has not subscribed to IMS Data Channel establishes an IMS Data Channel simultaneously with an IMS audio/video session, the IMS AS shall discard the Data Channel request and proceed with the audio/video IMS session establishment.

-	The DCSF provides a URL to the DCMF or MRF via the IMS AS during data channel media resource reservation for the bootstrap data channel that enables downloading a subscriber specific graphical user interface to the UE via the Bootstrap Data Channel. Once the DCMF or MRF, acting as HTTP proxy, receive a HTTP GET Request containing the root ("/") URL through the bootstrap data channel, the DCMF or MRF replaces the root ("/") URL in the HTTP GET Request with the subscriber specific URL and forward the request to the DCSF for downloading the subscriber specific graphical user interface to the UE. The UE shall maintain the HTTP session state when interacting with the DCMF.

# * * * * End of changes * * * *