3GPP TSG-SA WG2 Meeting #156e		S2-2304992

# E-meeting, April 17th – 21st, 2023									                     (revison of S2-2300xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0809 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FL Server NWDAF behaviour related to the maximum response time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes to address the following editor’s note: Editor's note: Further information on how the server NWDAF behaves based on the maximum response time from client NWDAF is FFS. It was agreed that the Server NWDAF decides either to wait for the Client NWDAFs which have not yet provided their interim local model or to aggregate only the retrieved local model information instances to update global model based on the notification from the Client NWDAF as in the step 3/4 of the general procedure in clause 6.2C.2.2. In this regard, to address the above EN, it is proposed to introduce the following further clarification on the behaviour of the Server NWDAF as follows: If the Client NWDAF notifies the Server NWDAF of local model training failure in step 3, the Server NWDAF does not wait for the Client NWDAF and decides to proceed to compute the global model metric. If the Client NWDAF notifies the Server NWDAF of more time necessary for local ML model training, the Server NWDAF can decides whether to wait for the Client NWDAF’s local model training completion or not based on local configuration and/or the time when the analytics information is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN on Server NWDAF behaviour related to the maximum response time, Editorial correction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Server NWDAF operations related to FL maximum response time remains unclarified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a subscription request to NWDAF containing MTLF to retrieve a ML model, using Nnwdaf_MLModelProvision service as defined in TS 23.288 [5], including Analytics ID, model metric (e.g., accuracy, precision, recall, etc), pre-determined status (e.g. requested accuracy level and/or total training time) and/or periodically reporting condition (e.g. training round/time).

NOTE 1:	The requested accuracy level can be used to indicate the target accuracy level of the training process, and the FL server NWDAF may stop the training process when the requested accuracy level is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF or the consumer of the NWDAF containing AnLF) provides the time when the analytics information is needed, the Server NWDAF can take this information into account to decide the maximum response time for its Client NWDAFs.

1.	Server NWDAF sends a request to the selected NWDAF containing MTLF(Client NWDAF) that participates in the Federated learning to perform the local model training for Federated Learning, including model metric. The request also includes the maximum response time before which the Client NWDAF has to report the interim local ML model information to the Server NWDAF.

2.	Each Client NWDAF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3.	During Federated Learning training procedure, each Client NWDAF further trains the retrieved ML model from the Server NWDAF based on its own data, and reports interim local ML model information to the Server NWDAF. Each Client NWDAF also computes local model metric and includes it in the local training status report sent to the Server NWDAF, which optionally also contains other information about local training in the Client NWDAF, e.g. characteristics of local training dataset (e.g., area, sampling ratio, max/min of value of each dimension of data), etc.

NOTE 2:	The parameters in characteristics of local training dataset are up to the implementation.

The ML model, which is sent from the Client NWDAF(s) to the Server NWDAF during the FL training process, is the information needed by the Server NWDAF to build the aggregated model based on the locally trained models.

Editor's note:	The services (i.e. Nnwdaf_MLModelTraining, Nnwdaf_MLModelTrainingInfo) in steps 1, 3, 5a and 6 will be updated in the Figure 6.2C.2.1-1.

If the Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the Server NWDAF, the Client NWDAF notifies the Server NWDAF on the delay event with proper cause information (e.g. local ML model training failure, more time necessary for local ML model training) before the maximum response time elapses.

4.	The Server NWDAF aggregates all the local ML model information retrieved at step 3, to update the global ML model. The Server NWDAF may also compute the global model metric, e.g. based on the local model metric(s) or by applying the global model on the validation dataset (if available).

If the Server NWDAF provides the maximum response time for the Client NWDAFs to provide the interim local ML model information in step 1, the Server NWDAF decides either to wait for the Client NWDAFs which have not yet provided their interim local ML model within the maximum response time or aggregate only the retrieved local ML model information instances to update global ML model. The Server NWDAF makes this decision, considering the notification from the Client NWDAF or, if the notification is not received, based on local configuration. For example, if the Client NWDAF notifies the Server NWDAF of local ML model training failure in step 3, the Server NWDAF does not wait for the Client NWDAF and decides to proceed to compute the global model metric. If the Client NWDAF notifies the Server NWDAF that more time is necessary for local ML model training, the Server NWDAF decides whether to wait for the Client NWDAF to complete the local training and provide the interim local ML model information or not based on local configuration and/or the time (provided from the NWDAF containing AnLF or the consumer of the NWDAF containing AnLF) when the analytics information is needed.

Editor's note:	It is FFS whether Server NWDAF solicits a response from an FL clients which has not reported the FL status within the maximum response time.

5a.	[Optional] Based on the consumer request, the Server NWDAF updates the training status (e.g. global model metric) to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the requested accuracy level is achieved or training time expires).

5b.	[Optional] Consumer decides whether the current model can fulfil the requirement, e.g. global model metric is satisfactory for the consumer. The consumer modifies subscription (stops or continues the training process) according to the consumer decision.

5c.	[Optional] According to the request from the consumer, Server NWDAF updates or terminates the current FL training process.

6.	If the FL procedure continues, Server NWDAF sends the aggregated ML model information to each Client NWDAF for next round model training.

7.	Each Client NWDAF updates its own ML model based on the aggregated ML model information distributed by the Server NWDAF at step 6.

NOTE 3:	The steps 2-7 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

After the training procedure is complete, the Server NWDAF may send the globally optimal ML model information to the consumer.

* * * * End of Changes * * * *