SA WG2 Meeting #156E(e-meeting)	S2-2305065

April 17 – 21, 2023, Electronic Meeting	(revision of S2-22xxxxx)

Source:	CATT

Title:	Discussion on Query of Serving LMF of PRU

Document for:	Discussion

Agenda Item:	9.6.2

Work Item / Release:	5G_eLCS_Ph3 / Rel-18

Abstract of the contribution: this discussion paper is proposed to discuss the possible solution of querying serving LMF of PRU and propose the way forward.

1. Introduction

It is possible that the serving LMF of the PRU and the serving LMF of the target UE are different LMFs, for example, the target UE and/or the PRU locates in border areas shared by two or more LMFs.

To support the scenario above, the step 7 and step 10 which are interaction between the two LMFs are introduced in clause 6.17.4 in TS 23.273 as shown in Fig 1.

Fig 1: Location of a target UE using PRUs

The pre-condition of the steps 7 and 10 in Fig 1 is that the serving LMF of target UE can find the serving LMF of the PRU. To support this pre-condition, NRF impact is introduced to store PRU information in LMF profile stored in NRF. The PRU information includes the PRU location, PRU existence indication and so on. Then the serving LMF of target UE can discover the serving LMF of PRU by querying the NRF.

Based on the discussion in SA2#155 meeting, it was commented that the NRF can only store static information. Based on the discussion in pre-meeting CC, a new solution with the pre-condition of introducing the terminology of stationary PRU was proposed. But the comment on the pre-condition was received during the discussion, i.e. there was a long discussion in CT1 that whether to introduce the concept of stationary UE or not, but the conclusion is no. So it seems that the new solution cannot work well.

To solve the issue, this discussion paper is proposed to discuss possible solutions to solve the issue and propose the way forward.

2. Possible Solutions

2.1	Sol#1: storing PRU information in UDR

Considering UDR is unified data repository used by NF service consumers to store and retrieve data from it. And there is no limitation that the UDR can only store static information. So it is proposed that the serving LMF can store PRU information in the UDR, e.g. for PRU location in border areas shared by two or more LMFs.

To support this way forward, the following existing UDR service operations can be re-used, no new service operation needs to be defined.

- Nudr_DM_Create Request/Response.

- Nudr_DM_Update Request/Response.

- Nudr_DM_Delete Request/Response.

- Nudr_DM_Query Request/Response.

To support this way forward, the impacts to UDR is to store PRU information. The extension to Table 5.2.12.2.1-1 in TS 23.502 is as follow:

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMF Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMS Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  |  |  | Serving PLMN ID and optionally NID |
|  | Slice Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Group Data (NOTE 5) | Internal Group Identifier or External Group Identifier | - |
|  | Identifier translation | GPSI |  |
|  |  | SUPI | Application Port ID, MTC Provider Information, AF Identifier |
|  | Intersystem continuity Context | SUPI | DNN |
|  | LCS privacy | SUPI | - |
|  | LCS mobile origination | SUPI | - |
|  | UE reachability | SUPI | - |
|  | Group Identifier Translation | Internal Group Identifier or External Group Identifier | - |
|  | UE context in SMSF data | SUPI | - |
|  | V2X Subscription data | SUPI | - |
|  | ProSe Subscription data | SUPI | - |
|  | User consent | SUPI | Purpose |
|  | ECS Address Configuration Information (See Table 4.15.6.3d-1) | SUPI, Internal group identifier or external group identifier or any UE | DNN, S-NSSAI |
|  | MBS Subscription data | SUPI | - |
|  | Shared data | Shared Data ID | - |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
|  | EAS Deployment Information (See clause 7.1 of TS 23.548 [74]) | DNN and/or S-NSSAI | Application Identifier and/or Internal Group Identifier |
|  | AM influence information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 of TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 of TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
|  | Network Slice Specific Control Data (See clause 6.2.1.3 of TS 23.503 [20]) | S-NSSAI |  |
|  | Operator Specific Data | SUPI or GPSI |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
| PRU Data | Serving LMF ID, Cell ID, PRU location | TAI | PRU identifier |

When the serving LMF of target UE receives the PRU Data from UDR, the behaviors of the LMFs are as follows:

- The serving LMF of target UE discovers the PRU(s) located in the serving cell of the target UE based on the PRU Data received from UDR.

- For each PRU discovered in step 1, the serving LMF of target UE sends the Nlmf_Location_MeasurementData Request (i.e. step 7 in fig 1) to the serving LMF of the PRU and receives Nlmf_Location_MeasurementData Response (i.e. step 10 in fig 1).

2.2	Sol#2: add a new capability in the LMF capability stored in NRF

In this solution, the LMF capability is extended to include a new capability of supporting PRU feature. To avoid impacts to NRF, the serving LMF of target UE provides TAI to the NRF, NRF returns the profile(s) of LMF(s) covering the TA.

When the serving LMF of target UE receives the LMF profile(s), the behaviors of the LMFs are as follows:

- The serving LMF of target UE discovers the LMFs supporting PRU feature based on the LMF capability in the LMF profile.

- For each LMF decided in step 1, the serving LMF of target UE sends the Nlmf_Location_MeasurementData Request (i.e. step 7 in fig 1) and receives Nlmf_Location_MeasurementData Response (i.e. step 10 in fig 1). The response includes failure in the case that there is no PRU in the cell ID included in the Nlmf_Location_MeasurementData Request.

3. Analysis

In sol#1, the UDR is used to store the PRU information. The serving LMF of target UE can obtain the exact/right LMF of the PRU located in the same TA of the target UE and obtain PRU measurements from the LMF above.

Sol#2 avoids storing PRU related information which is dynamic in NRF and reduces the impacts to NRF. After the serving LMF of target UE obtains the LMF(s) from NRF, it has to try with each LMF to request the PRU measurements. It is possible that the try is useless, for example the LMF returned by NRF supports the PRU feature and covers the serving cell of target UE, but there is no PRU in the TA/cell of the target UE. In this case, the signaling is wasted and the positioning latency is increased to find the right serving LMF of PRU.

4. Proposal

Based on the analysis in clause 3, it is proposed to take sol#1 as the way forward. The related CR (S2-2305074) is submitted to SA2#156E meeting accordingly.