3GPP TSG-WG SA2 Meeting #156-e	S2-2305120

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0366 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for interaction with NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Feature description for “Location estimate assisted by NWDAF” has been documented. However, the procedures are not yet proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TS 23.288 has described from NWDAF perspective how to derive the Location Accuracy Analytics. The proceudre demonstrated in TS 23.288 can be reused with some clarification. In TS 23.288 Figure 6.17.4-1 the Analytics consumer is replaced by LMF. Pre-condition of the procedure is NWDAF has a trained supervised ML model for deriving Location Accuracy Analytics. When LMF receives the Location Accuracy Analytics from NWDAF, it may establish a mapping for “location accuracy – applicable geographic area”. An example how LMF uses the mapping for “location accuracy – applicable geographic area” is when receiving a location request with a requested accuracy in the LCS QoS parameter, LMF can determine the streignet location accuracy in the UE’s serving cell by checking the mapping, and determine to accept the location request if the requested location accuracy is less streignet than the achievable accraucy in a certain area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure showing “Location estimate assisted by NWDAF” has not been documented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(new), 6.X.1(new), 6.x.1.1(new), 6.x.1.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.X	Interaction with NWDAF

6.x.1	Location Accuracy Analytics Retrieval by LMF

6.x.1.1	General

LMF may request Location Accuracy Analytics from NWDAF to retrive the location accuracy and its applicable geopraphical area. LMF only requests Location Accuracy Analytics in a sandalone procedure as defined in clause 6.x.1.2, but not during the procedure of MT-LR, MO-LR or deferred MT-LR.

6.x.1.2	Procedures

Procedure to request Location accuracy analytics by LMF is referred to TS 23.288[x], clause 6.17.4. In Figure 6.17.4-1 the Analytics consumer is replaced by LMF. Pre-condition of the procedure is NWDAF has a trained supervised ML model for deriving Location Accuracy Analytics.

When LMF receives the Location Accuracy Analytics from NWDAF, it may establish a mapping for “location accuracy – applicable geographic area”. An example how LMF uses the mapping for “location accuracy – applicable geographic area” is when receiving a location request with a requested accuracy in the LCS QoS parameter, LMF can determine the streignet location accuracy in the UE’s serving cell by checking the mapping, and determine to accept the location request if the requested location accuracy is less streignet than the achievable accraucy in a certain area.

Additional information in the Location Accuracy Analytics are indoor/outdoor indication may also be used by LMF for UE location determination.

Editor’s note: other parameters listed in TS 23.288, i.e. Indication on the ratio of LOS/NLOS measurements, (requested) confidence, are FFS whether and how LMF uses them.

# * * * * End of changes * * * *