3GPP TSG-WG SA2 Meeting #156E	S2-2305147

# April 17 – 21, 2023		(revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4503 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for network exposure for XR services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-02 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The description in clause 5.37.4 needs to be clarified and aligned with other clauses. - It is proposed to solve the following EN: Editor's note: It is FFS whether all QoS monitoring requests can be addressed with the limitation of one QoS Monitoring control information per PCC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.37.4, it clarifies the congestion information, data rate informaiton and round trip delay for multiple QoS flows. Other related updates are made in clause 5.45.2 and 5.45.3. - Regards to the EN, it is proposed that the PCF may include multiple QoS parameters to be measured for the QoS flow within the PCC rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related description is not clear, and the EN remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.37.4, 5.45.2, 5.45.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.4	Network Exposure of 5GS information

5GS and XR/media services cooperate to provide a better user experience using External Network Exposure.

Based on the AF request, the 5GS can report per QoS Flow information based on the QoS Monitoring as defined in clause 5.33.3 and/or clause 5.45:

-	For the congestion information (i.e. a percentage of congestion level for UL and/or DL), based on the PCC rule from PCF, the SMF requests the NG-RAN to report the information via GTP-U header to PSA UPF as described in clause 5.45.3 (this NG-RAN information can be provided to support ECN marking for L4S in PSA UPF as described in clause 5.37.3.3). The PSA UPF exposes the UL and/or DL congestion information via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18. It can be applied to a Non-GBR or GBR QoS flow.

-	For QoS Notification Control for GBR QoS Flow as defined in clause 5.7.2.4, upon SMF request, the NG-RAN may additionally support indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U to UPF and the PSA UPF exposes this information via Nupf_EventExposure or via SMF/PCF/NEF as described in clause 5.8.2.18.

-	Data rate information of UL and/or DL for the QoS flow may be measured and exposed to the AF based on SMF request as one of the following:

-	measured and reported by PSA UPF via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18.

-	measured by RAN and reported via SMF/PCF/NEF.

Editor's note:	It is for RAN WGs to confirm whether providing QoS Notification Control for GBR QoS Flow and data rate information can be included in Release 18.

-	For round trip delay for multiple QoS Flows of the XR service (e.g. the UL and DL are separated into two flows) in the same PDU Session, it is determined based on the QoS Monitoring for packet delay of individual QoS Flow  as described in clause 5.33.3. The AF includes in the request the necessary information for the PCF to derive the associated QoS monitoring requirements for each PCC rule. The PCF provides the QoS Monitoring policies in the PCC rules for the XR service data flows. The PSA UPF reports the delay information per QoS Flow to the SMF. SMF reports to PCF. The PCF derives round trip delay information for the XR service data flows and exposes the information to the AF directly or via NEF.

NOTE:	How PCF calculates the requested round trip delay for multiple QoS flows from delays of individual QoS Flows is not defined in this specification.

The PCF may include multiple QoS parameters to be measured for the QoS flow within the PCC rule.

The AF may provide the Alternative QoS parameter set requirements and Averaging Window to the NEF/PCF for the GBR QoS Flow as specified in clause 4.15.6.6 of TS 23.502 [3].

Estimated bandwidth for 5QI may be exposed by NWDAF (according to information described in clause 6.9.2 of TS 23.288 [86]) to AF.

# * * * * 2nd changes * * * *

5.45.2	Packet delay monitoring

QoS Monitoring for packet delay allows for the measurement of UL packet delay, DL packet delay or round trip packet delay between UE and PSA UPF. The details of the QoS Monitoring for packet delay are described in clause 5.33.3.

For round trip delay for multiple QoS Flows in the same PDU Session, it is determined by the PCF based on the QoS Monitoring report for packet delay of individual QoS Flow as described in clause 5.37.4.

5.45.3	Congestion information monitoring

The NG-RAN may be required to provide the UL and/or DL QoS Flow congestion information (i.e. a percentage of congestion level for exposure). The UPF may be required to monitor the UL and/or DL QoS Flow congestion information reported from the NG-RAN. The QoS Monitoring on congestion information can be applied to a Non-GBR or GBR QoS flow.

QoS monitoring request to the NG-RAN and NG-RAN reporting for UL and/or DL QoS Flow congestion information to PSA UPF is as defined in 5.37.3. The PSA UPF reports the received UL and/or DL QoS Flow congestion information to the target NF as instructed by the QoS Monitoring request from the SMF.

# * * * * End of changes * * * *