3GPP TSG-WG SA2 Meeting #156E	S2-2305148

# April 17 – 21, 2023		(revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4504 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for PDU Set handling support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-02 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - It is proposed to add a NOTE in clause 5.7.7.4 to solve the following EN: Editor's note: Usage of PSIHI is FFS. - RAN2 has provided reply LS on PSER as follows: - RAN2 suggests to use PSER definition as “the PSER defines an upper bound for a rate of non-congestion related PDU Set losses”. - RAN2 thinks that the use of PSER is up to network implementation. The definition of PSER in TS 23.501 can be further updated. - If the RAN can’t support PDU Set QoS Parameters, it would be a waste of UPF resource to perform the PDU set information marking. So the PSA UPF needs to get the information when the RAN can’t support the PDU Set QoS Parameters |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is proposed to add the NOTE on how to use the PSIHI in clause 5.7.7.4 to solve the EN about the PSIHI. - Update defintion of PSER in clause 5.7.7.3. - It is proposed to clarify how to solve the case when the RAN can’t support PDU Set QoS Parameters for the QoS flow. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The ENs are not solved. - The PSER definition is not aligned with RAN WG. - If the RAN can’t support PDU Set QoS Parameters, it would be a waste of UPF resource to perform the PDU set information marking. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.7.7.1, 5.7.7.3, 5.7.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.7.7	PDU Set QoS Parameters

5.7.7.1	General

PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. The followings are the PDU Set specific QoS characteristics:

1.	PDU Set Delay Budget (PSDB).

2.	PDU Set Error Rate (PSER).

3.	PDU Set Integrated Handling Information (PSIHI).

For a QoS Flow supporting PDU Set based QoS handling, the QoS Profile includes the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile.

If the NG-RAN receives PDU Set QoS Parameters and supports them, it applies PDU Set QoS Parameters as described in this clause. If the NG-RAN can’t support the PDU Set QoS Parameters, it sends an indication to the SMF. Based on this indication, the SMF disables the PDU Set marking in PSA UPF for the QoS flow.

Editor's note:	[XRM] The applicability and details of PDU Set handling in uplink direction is pending RAN WG's progress.

# * * * * Second change * * * *

5.7.7.3	PDU Set Error Rate

The PDU Set Error Rate (PSER) defines an upper bound for a rate of non-congestion related PDU Set losses..

NOTE X:	In this Release, how a PDU Set is considered as successfully deliveredis based on PSIHI indication and RAN implementation as specified in clause 5.7.7.4.

NOTE Y:	Usage of PSER e.g. for appropriate link layer protocol configurations, is left for network implementation.

A QoS Flow is associated with only one PDU Set Error Rate. If the PSER is available, the usage of PSER supersedes the usage of PER. The value of the PDU Set Error Rate is the same in UL and DL.

5.7.7.4	PDU Set Integrated Handling Information

The PDU Set Integrated Handling Information (PSIHI) indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side.

NOTE:	If PSIHI indicates that all PDUs are needed for the usage of the PDU Set, the PDU Set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully. If PSIHI indicates that not all PDUs are needed for the usage of the PDU Set, how the PDU Set is considered as successfully delivered is left for RAN implementation in this Release.

# * * * * End of changes * * * *