3GPP TSG-WG SA2 Meeting #156-e	S2-2305180

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4516 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The update of Policy control enhancements to support multi-modal services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | S2-2303238 was agreed during SA2#155 meeting. The new clause about Policy control enhancements to support multi-modal services is introduced in 501. update related description of the feature based on the discussion and the TR concltuion, as follow: For the update of the Multi-modal Service ID, add the clarification for the feature supporting, based on the online and offline discussion. For the URSP reusing, add the description for the single UE case, based on TR conclusion. For the multiple PCFs involved, there is no support for policy coordination among the multiple PCF(s). SP-230233 is agreed during the SA#99, which objective is to support the service continuity and connectivity for XR under high UE mobility. Add the clarification that no coordination between NFs in this release to avoid ambiguity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarifications of multi-modal service supporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The realted descrption of the feature is uncomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.501 CR ... TS 23.503 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

5.37.2	Policy control enhancements to support multi-modal services

Multi-modal services consist of several data flows (named as multi-modal flows) that related to each other and may come from different sources. Each data flow (single-modal data) may be seen as one type of data (for example audio, video, positioning, haptic data) associated with the same communication service. Data flows that comprise a multi-modal service may come from a single UE, either via a single device or via multiple devices connected to the single UE that can access the 5GS, or from multiple UEs.

For the single UE case, the expected multi-modal application behaviour is that those data flows that are closely related and require strong application coordination for correct delivery of the multi-modal application data, are transmitted in a single PDU session.

-	The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, service requirements, for each media that comprise the multi-modal service, a Multi-modal Service ID and QoS monitoring requirements for multiple IP data flows associated to a multi-modal service.

-	The AF may provide to the PCF a Multi-modal Service ID, as an explicit indication that the application traffics are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and apply QoS policies for data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirement for data flows associated to a multi-modal service to the PCF (either directly or via NEF) with in a certain period. The PCF generates the authorized QoS Monitoring policy for these service data flows.

NOTE 1:	In order to start the monitoring of the packet delay results for data flows associated to a multi-modal service within a certain period, the PCF needs to receive the data from AF accordingly.

-	The multi-modal services are carried in single PDU session for one UE. In order to ensure that the correct DNN/S-NSSAI combination for the XRM traffic is selected, a traffic descriptor (e.g. an FQDN) for the session can be used during URSP rule.

In addition to the features that are provided for the case that the flows are associated with a single UE, the following features are provided for the case where the flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modal service should be selected among the multiple UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

-	The AF may allocate the same Multi-modal Service ID to all UE PDU sessions that compose a multi-modal service. The PCF may take this information into account (e.g. to allocate a specific QoS profile) when processing each PDU session independently. The data flows that contribute to the service experience, but may still be valid stand-alone, may be transmitted over separate PDU sessions from multiple UEs.

-	In case multiple PCFs are involved, the PCF(s) take policy decisions according to the input provided by the AF. There is no support for policy coordination among the multiple PCF(s) in this release of the specification. Policy decisions are taken by each PCF separately on a per PDU session basis.

NOTE 2:	For both the single UE and multi-UEs case, the update of the Multi-modal Service ID is not supported in this release of the specification.

# * * * * End of changes * * * *