SA WG2 Meeting #S2-156E	S2-2305190

17 - 21 April, 2023, Electronic

Source:		Interdigital

Title:	TS 23.586: Procedures for UE only operation and NW assisted operation

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract:  This paper proposes the Procedures for UE only operation and NW assisted operation.

# 1. Introduction/Discussion

The procedures for UE only operation and NW assisted operation shall be specified.

# 2. Text Proposal

It is proposed to capture the following changes to TS 23.586.

* * * Start of Change * * *

## 6.7	Procedures for UE only operation and NW assisted operation

Overall procedure of UE only operation is depicted in clause 6.7.1 and procedures of NW assisted operations are depicted in clause 6.5

### 6.7.1	Procedures for UE only operation

NOTE : Detail procedures of UE only operation are specified in TS 38.XXX [X].

Editor’s Note: The procedures may be further updated to align with procedures defined in RAN.

All signalling messages for UE only operation are transferred over PC5 channels among target UE, SL reference UEs, and SL positioning server UE.

Figure 6.7.1-1: procedures for UE only operation

1.	Among target UE, SL reference UEs and SL positioning server UE, capability negotiation is performed.

2.	SL positioning server UE decides SL positioning method.

3. 	Assistance data for the SL positioning method may be shared among target UE, SL reference UEs and SL positioning server UE.

4.	The target UE and SL reference UEs perform Ranging/SL positioning operation.

5.	The result of Ranging/SL positioning operation is shared to the SL positioning server UE.

6.	SL positioning server UE calculates location result e.g. location of target UE, ranging result between target UE and SL reference UE.

7.	location report including location result is shared to the target UE and/or SL reference UEs.

* * * End of Change * * *