3GPP TSG-SA2 Meeting #156-e	S2-2305261

# Online, , 17th Apr 2023 - 21st Apr 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0185 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for MBS multicast session release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 09/02/2023 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Two options for the removal of a UE without activated UP from an MBS multicast session (e.g. due to a termination of the MBS session) are currentley documented: 1. (Group) paging of related UEs to trigger those UEs to transition to connected state to inform them that they are removed from the MBS session. 2. Waiting for related UEs to become connected and then inform them them that they are removed from the MBS session. Option 1 has the drawback that this can lead to unnecessary load and possible congestion in RAN. It also has possible forward compatibility problems with Rel-18, where RRC-inactive UEs do not need to transition to RRC connected state when receiving group paging as the SMF may not be aware that NG-RAN applies the transmission mode for RRC-inactive UEs and may regard a lacking response from the UE as error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (Group) paging of UEs to trigger those UEs to transition to connected state to inform them that they are removed from the MBS session is depreceated and the alternative option that the SMF waits until a UE establishes the UP before informing it that it is removed from the MBS session is recommended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary load and possible congestion in RAN. Forward compatibility problems with Rel-18, where RRC-inactive UEs do not need to transition to RRC connected state when receiving group paging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.2.2.3	Multicast session leave requested by the network or MBS session release

This procedure applies to the following scenarios:

1.	When the MB-SMF decides to release an MBS Session:

-	based on a request from the AF (directly or via the NEF/MBSF);

In this scenario, the MB-SMF notifies the SMF of multicast session release, and the SMF initiates procedures to remove all joined UEs from the MBS session.

2.	When the SMF decides to remove a UE from an MBS session:

-	based on a request from the UDM (subscription change); or

-	due to local and location dependent MBS service is described in clause 7.2.4; or

-	due to network internal reasons.

For the active MBS session, to release radio resources as early as possible, the MB-SMF may trigger Multicast Session Deactivation towards the NG-RAN as specified in steps 5-9 of clause 7.2.5.3, prior to or in parallel with triggering MBS Session Release to the SMF.

Figure 7.2.2.3-1: MBS Session Release or Multicast session leave requested by the network

1a.	For MB-SMF triggered MBS session release, the SMF receives Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session release) from the MB-SMF with MBS Session ID. The SMF checks all joined UEs and perform step 2 to step 9 for each UE.

1b.	The SMF decides to remove a UE from the MBS session without MBS session release (e.g. due to UE moving out of MBS service area for local or location dependent MBS service as described in clause 7.2.4).

2.	For UEs without activated UP,

the SMF should not trigger message to the AMF, instead the SMF should mark that the UE is to be informed of the MBS Session release. In this case, the SMF should initiate PDU Session Modification to inform the UE of the MBS Session release at next UP activation of the associated PDU Session.

3.	For the joined UEs with UP activated, the SMF invokes Namf_Communicate_N1N2MessageTransfer to the AMF. The N1 SM container indicates UE removed from MBS session with appropriate cause (e.g. MBS session release, out of MBS service area, etc.). In N2 SM information, the SMF informs the NG-RAN to remove the UE from the MBS session. If there are associated QoS Flow(s) for individual delivery, the SMF also releases those QoS Flow(s) as specified in TS 23.502 [6] clause 4.3.3.2.

4.	The AMF sends N2 Request to the NG-RAN.

5.	The NG-RAN transports the N1 SM container (PDU Session Modification Command (MBS Session ID, UE removed from MBS session with appropriate cause)) to the UE.

6.	The NG-RAN performs radio resource modification. If there are no joined UEs in the MBS session, the NG-RAN releases the radio resources.

7.	If there are no joined UEs in the MBS session, for unicast transport of N3mb, the NG-RAN initiates the DL tunnel release towards MB-UPF via AMF and MB-SMF. For multicast transportation of N3mb, the NG-RAN performs IGMP/MLD Leave for the MBS session. See clause 7.2.2.4 for details.

8.	The NG-RAN sends N2 Response to the AMF. If there are no joined UEs in the MBS session, the MBS Session Context is removed from the NG-RAN.

9.	The AMF transfers the N2 message received in step 8 to the SMF via the Nsmf_PDUSession_UpdateSMContext service operation. The SMF removes the UE from the MBS Session.