3GPP TSG SA WG2 Meeting #156e 	S2-2305321

Electronic, 17-21 April 2023

Source:	InterDigital Inc.

Title:	Using the Traffic Descriptor for UPF Selection

Document for:	Agreement

Agenda Item:	9.12.2

Work Item / Release:	XRM/Rel-18

Abstract: The paper discusses how to use the eUEPO feature of reporting URSP enforcement to help with UPF selection for XR traffic.

# 1. Background

In addition to the features that are provided for the case that the flows are associated with a single UE, the following features are provided for the case where the flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modal service should be selected among the multiple UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

TS 23.501 includes the following text:

Observation 1: If the applications of the UEs that are associated with the same multi-modal service provide the same traffic descriptor (e.g., FQDN), the URSP Rule evaluation framework can be used to ensure that the same DNN/S-NSSAI is selected.

Observation 2: Although the existing URSP framework can be used to ensure that the UEs use the same DNN/S-NSSAI combination, using the same DNN/S-NSSAI combination does not guarantee that the same slice and/or network functions will be selected to serve the UEs.

Observation 3: If the traffic descriptor (e.g., FQDN) was provided to the SMF during PDU Session Establishment or Modification, the traffic descriptor could be used during UPF selection and reselection, thus the SMF would be able to easily ensure that all of the multi-modal flows use the same UPF.

For example, UEs of the same multi-modal service could be provisioned with the same FQDN and use FQDN as a traffic descriptor. The SMF could then use the FQDN during slice and/or network function (i.e., UPF or PCF) selection.

If UE supports to report URSP rule enforcement to network, the UE may provide Connection Capabilities as described in clause 6.6.2.4 of TS 23.503 [20].

In eUEPO, the PDU Session Establishment and Modification procedures were updated so that the UE can report a traffic descriptor to the network, however, the specifications only allow the traffic descriptor to be reported if the traffic descriptor is a connection capability. For example, the following text has been added to the PDU Session Establishment and Modification procedures in TS 23.502.

Observation 4: The UE can already report the traffic descriptor, but only when it is a Connection Capability.

# 2. Proposal

It is proposed to agree to allow the UE to report other types of traffic descriptors besides Connection Capabilities. For example, allow the UE to report an FQDN if the traffic descriptor is an FQDN. The reported FQDN can then be used by the SMF in UPF selection.

The following CRs implement the above proposal.

- S2-2305322 for TS 23.501

- S2-2305323 for TS 23.503

- S2-2305324 for TS 23.502