3GPP TSG-SA2 Meeting #156E	S2-2305347

# Elbonia, 17 – 21 April, 2023		                                                        (revision of S2-XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4557 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Session Establishment for PIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, PEGC establishing single/multiple PDU session and routing the traffic to these PDU session based on the required PINE QoS is not defined. In this CR, we propose to establish single/multiple PDU session by PEGC on behalf of PINE with the intended QoS and route the packets from a PINE to the appropriate PDU session with the intened QoS. Though PEGC will establish a PDU session with the 5G network based on the existing procedure defined in clause 4.3.2.2 of TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PEGC establishing a PDU session and serving as gateway for all the PINE with the intended QoS. PEGC establishes single/multiple PDU session based the QoS requested by the PINE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature as defined in the study conclusion is not currently supported by 3GPP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.44.3.1	PDU Session Establishment for PIN

PIN will configure one its PINE as PEGC and PIN’s default gateway, so that all the traffic is sent to the 5G network via PEGC relay.	PEGC acts as a Network Address Translator (NAT) between 5G network and the PIN by representing a unique IP address to all the PINE(s).

PINE in the PIN will request PEGC for 5G session establishment using request-response queries. The PEGC will establish a PDU session with the 5G network based on the existing procedure defined in clause 4.3.2.2 of TS 23.502 [3].

PINE requesting for a desired QoS will request the PEGC to setup a PDU session with the intended QoS, and PEGC will setup appropriate PDU session. All the traffic from the PIN will be forwarded to the PDU session with the desired QoS.

PINE will use DSCP markings on the IP packet in the PIN to indicate the desired QoS and PEGC will setup an appropriate 5QI PDU session with the DNN, and further PEGC will reset the QoS marking on the IP packet and forward it to the established PDU session.

PEGC will also provide the necessary QoS as indicated by the PINE on the IP packets in the PIN network.

PINE in the PIN will request PEGC for 5G session release using request-response quires. The PEGC will release the respective PDU associated with the PINE - PEGC uses PDU ID and PINE FQDN or IP address of the PINE in the local PIN.

* * * End of Changes * * *