SA WG2 Meeting #156e	S2-2305367

# April 17-21, 2023, Electronic Meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4155 | rev |  | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIMLsys: KI#7 – UP Security filtering criteria for UE Member Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Europe, Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | User Plane Security Status: User plane security status is an indicator that is important to consider for the purpose of UE member selection. The AF may be interested in knowing whether the session the UEs are using are integrity protected and / or confidentiality protected, e.g., as this affects data rates. The UP-integrity protection is related to the maximum supported data rate per UE for integrity protection for the DRBs which can be very useful for the application function |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add User Plane Security Status as a filtering critera. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide a 5GC mechanism to support the member selection to assist the Application AI/ML operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2 and 4.15.13.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.502 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is adding one row to the table in clause 4.15.13.2, and a specific procedure in 4.15.13.X |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 4.15.13.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

Table 4.15.13.2-1 provides a summary of the UE member filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria |  | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time | Clause 6.18 of TS 23.288 [50]. |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.3 |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.3 |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.3 |
| User Plane Security Status | Indicate the User Plane Security Status of the selected UE | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI | 4.15.13.X |

* * * * Start of 2nd Change * * * *

#### 4.15.13.X	Specific procedure for the 5GC assistance to member selection based on User Plane Security Status

Figure 4.15.13.X: 5GS assistance for UE member selection based on User Plane Security Status

1.	AF subscribes the member selection assistance functionality by sending Nnef_UEMemberSelectionAssistance_subscribe request including the Application Identity, Flow Description(s) and the User Plane Security status filtering criteria that includes one or more of UL packet delay, UL packet delay and round-trip packet delay.

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the UP Security status filtering criteria provided by the AF.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI and DNN to which this Application ID have access. Due to the absence of a time window in the request from the AF, NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application Id or AF Id).

4.	For each target SMF, NEF sends an Nsmf_EventExposure_Subscribe request (Event: 'UP Security status', target UEs, Application ID or Flow Description(s), S-NSSAI, DNN, etc.).

5a.	The SMF many need to interact with UPF for requesting the QoS monitoring for certain flows. The SMF sends a Nudm_SDM_Get message to the UDM including target UEs SUPI, S-NSSAI, DNN, and the Session Management Subscription data type (User Plane security policy).

5b. The UDM provides UP Security policy to the SMF.

6.  The SMF determines User Plane Security enforcement information for the PDU session(s).

7.	SMF sends an Nsmf_EventExposure_Notify request including an Event Exposure notification.

8.	Based on the Analytics report received from the SMFs, NEF consolidates the results received from the SMFs and derives the list(s) of candidate UE(s) and additional information which fulfil the UP Security status filtering criteria in the AF request.

9.	NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * * End of Changes * * * *