3GPP TSG-SA WG2 Meeting #156-e	S2-2305391

# Electronic meeting, 17th – 21st April, 2023

# Title:	Reply LS on Handling of the Allowed PDU session status IE in Non-allowed service area

# Response to:	LS on Handling of the Allowed PDU session status IE in Non-allowed service area (S2-2303916/S2-2302183/S2-2300029/C1-227197)

# Release:	Rel-18

# Work Item:	5GProtoc18

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Peng Tan

E-mail Address:	v-tanpeng@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

# 1. Overall Description:

SA2 thanks CT1 for the LS in S2-2303916/S2-2302183/S2-2300029/C1-227197. SA2 discussed the scenario described in the LS and agreed on the following answers to CT1 questions.

Question 1: Does the UE need to include the Allowed PDU session status IE also if it responds to the paging when it is in a non-allowed area?

SA2 Answer:

First, the UE in a Non-Allowed Area shall respond to paging request with Service Request, via non-3GPP access, as specified in Clause 5.3.4.1 of TS 23.501, if the UE is in CM-IDLE state in both non-3GPP and 3GPP accesses.

As further specified in clause 4.2.3 of TS 23.502, the UE Triggered Service Request procedure shall contain the List of Allowed PDU Sessions, if they can be re-activated over the 3GPP access based on UE policies and NSSAI policies. Otherwise, the UE includes an empty List of Allowed PDU Sessions.

So, in summary, for the scenario described in this LS, the UE shall include the Allowed PDU session status IE. However, as the UE is currently in a non-allowed area, and the PDU session(s) activated via non-3GPP access cannot be transferred to 3GPP access, UE includes an empty “List of Allowed PDU Sessions”.

SA2 would like to clarify that as specified in TS 23.502 the list of Allowed PDU sessions is provided by the UE according to UE policies and whether the S-NSSAIs of these PDU Sessions are within the Allowed NSSAI for 3GPP access.

Question 2: Is the UE allowed to include the Uplink data status IE if it has uplink data pending if it responds to the paging when it is in a non-allowed area?

SA2 Answer:

As there are no available PDU sessions to send uplink IP packets, the UE is not allowed to include the uplink data status IE for the scenario described in this LS.

# 2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take above into account.

# 3. Date of Next SA2 Meetings:

SA2#157	22-26 May 2023	Berlin, DE