3GPP TSG-WG SA2 Meeting #156E 	S2-2305742

April 17 – 21, 2023, Elbonia	(revision of S2-2304370)

Source:	Sony

Title:	Procedure MO-LR NW based UE positioning using SL positioning.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper introduces a new procedure for MO-LR using SL positioning.

# 1. Introduction/Discussion

This paper introduces a new procedure for MO-LR using SL positioning.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

#### 6.5.1.2	Procedure MO-LR Procedure for UE with NAS connection using SL positioning.

Figure 6.5.1.2-1: MO-LR procedure for UE with NAS connection using SL positioning.

- If the UE is in CM-IDLE state, UE instigates the UE triggered Service Request as defined in clause 4.2.3.2 of TS 23.502 [3] in order to establish a NAS signalling connection with the AMF.

- The UE sends an MO-LR Request message included in a UL NAS TRANSPORT message specified in step 2 clause 6.2 in TS 23.273 [8]. UE may include indication that SL positioning is requested.

- The AMF selects an LMF as described in TS 23.273 [8] clause 5.1.

- The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF as specified in TS 23.273 [8].

- The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Downlink (DL) Positioning message to the UE. The DL Positioning message may include a list of candidate Located UE(s) as assistance data if SL positioning is requested in MO-LR request in step 2.

- The AMF forwards the Downlink Positioning message to the UE in a DL NAS TRANSPORT message. The AMF includes a Routing identifier.

- SL-MO-LR procedure defined in clause 6.x of TS 23.273 [8] is performed with the following adaptations:

- UE1 is the Target UE, and UE2/…/UEn is the located UE and/or SL Reference UE.

- It is indicated in step 8 that location calculation assistance is needed and absolute location is required

# * * * * End of changes * * * *