3GPP TSG-WG SA2 Meeting #156E	S2-2306088

Elbonia, April 17-21, 2023																	was S2-2304075r01

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0727 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data storage in ADRF using DSC indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update TS23.288 to support the usage of a Data Synthesis and Compression (DSC) information to store and fetch data to and from ADRF, as concluded for KI#4 in TR23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TS23.288, clause 6.2B – introduce a Data Synthesis and Compression (DSC) information in the ADRF procedures. Clause 10.2 – introduces the Data Synthesis and Compression (DSC) information in the appropriate service procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not updated for Rel. 18 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2, 10.2.2, 10.2.5, 10.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 6.2B.2	Historical Data and Analytics storage

The procedure depicted in figure 6.2B.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

The consumer may include in the storage request the DataSetTag attribute which the data records are to be associated to when stored by ADRF. The DataSetTag attribute is defined in Table 6.2B-1. Data records can be associated to multiple DataSetTag attributes.

Table 6.2B-1: DataSetTag attribute

| Information | Description |
| --- | --- |
| DataSetTag | Identifies the data set. |
| DataSetDescription | Provides human-readable information about the characteristics of the data set. |

The consumer may include in the storage request the Data Synthesis and Compression (DSC) information. The detail of DSC information is up to implementation, which is out of 3GPP scope.

NOTE:	DCS information can include the following information:
indication that the data have been generated using a data synthesis tool;
indication that the data have been generated using a data compression tool;
the information about the data synthesis and/or compression technique.

Figure 6.2B.2-1: Historical Data and Analytics storage

0a-c.	NWDAF, DCCF or ADRF are configured with default operator storage policies as described in clause 5B.1.

1.	The consumer sends data and/or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest (collected data with timestamp, analytics with timestamp, Service Operation, Analytics Specification or Data Specification, Storage Handling information, optionally DataSetTag, optionally DSC information) service operation. The NWDAF or DCCF may provide notification endpoint information to the ADRF for use by the ADRF to send notifications (implicit subscription) alerting the DCCF or NWDAF that data are about to be deleted (see step 6).

2.a-c	Based on Storage Handling information (if available) and Storage Policy, the ADRF, DCCF or NWDAF determines the Storage Approach (life-time for storing data and whether consumer is notified prior to data deletion).

3.	The ADRF stores the data and/or analytics sent by the consumer. The ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer NF and, if the data and/or analytics is already stored or being stored in the ADRF, the ADRF decides to not store again the data and/or analytics sent by the consumer. If the DataSetTag attribute is included for data and/or analytics already stored or being stored, then ADRF associates the data records with such DataSetTag.

4.	The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating that data and/or analytics is stored, whether the ADRF determined at step 3 that data or analytics is already stored and the Storage Approach.

Conditional on ADRF Managing the Storage Approach

5	The ADRF determines that the life-time of the stored data or analytics has expired (according to the Storage Approach).

6	If indicated by the Storage Approach, the ADRF sends a notification alerting the DCCF or NWDAF that data are about to be deleted.

NOTE:	This is an implicit subscription.

7.	The DCCF or NWDAF indicate in the response to the ADRF whether they will retrieve the Data or Analytics

8	The DCCF or NWDAF may retrieve the Data or Analytics from the ADRF

Conditional on the DCCF or NWDAF Managing the Storage Approach

9	The NWDAF or DCCF determine that the life-time of the stored data or analytics has expired (according to the Storage Approach).

10. The NWDAF or DCCF optionally retrieve the data or analytics from the ADRF

11.	The NWDAF or DCCF request that the data or analytics be deleted from the ADRF

12.	The ADRF deletes the data or analytics if:

-	the Storage Approach in the ADRF indicates alerting the consumer is not required prior to data or analytics deletion;

-	in step 7 the DCCF or NWDAF indicated data or analytics will not be retrieved prior to deletion;

-	data or analytics retrieval in step 8 has completed; or

-	A request to delete data or analytics was received in step 11.

If the ADRF received a response from the NWDAF or DCCF in step 7 indicating data or analytics will be retrieved but retrieval is not initiated before an adequate fixed time has elapsed, the ADRF may autonomously delete the data or analytics.

# * * * * 2nd change * * * *

### 10.2.2	Nadrf_DataManagement_StorageRequest service operation

Service operation name: Nadrf_DataManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store data or analytics. Data or analytics are provided to the ADRF in the request message.

Inputs, Required: Data with timestamp or Analytics with timestamp to be stored, Service operation, Analytics Specification or Data Specification.

"Service Operation" identifies the service used to obtain the data or analytics from a Data Source (e.g. Namf_EventExposure_Subscribe or Nnwdaf_AnalyticsSubscription_Subscribe).

"Analytics Specification or Data Specification" is the "Service Operation" specific required and optional input parameters that identify the data that was stored (e.g. Analytics ID(s) / Event ID (s), Target of Analytics Reporting or Target of Event Reporting, Analytics Filter or Event Filter, etc.). Service Operations and input parameters are defined in clause 7 for NWDAF and in clause 5.2 of TS 23.502 [3] for the other NFs.

Inputs, Optional: DataSetTag, DSC information.

Outputs Required: Result Indication.

### Outputs, Optional: Storage Transaction Identifier, DataSetTag(s).

# * * * * 3th change * * * *

### 10.2.5	Nadrf_DataManagement_RetrievalRequest service operation

Service operation name: Nadrf_DataManagement_RetrievalRequest

Description: The consumer NF uses this service operation to retrieve stored data or analytics from the ADRF. The Nadrf_DataManagement_RetrievalRequest response either contains the data or analytics, or provides instructions for fetching the data or analytics. The Nadrf_DataManagement_RetrievalRequest may be unsolicited (e.g. when the consumer itself has known "Storage Transaction Identifier") or sent in response to a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify.

Inputs, Required: one of the following:

-	Storage Transaction Identifier; or

-	Fetch Correlation ID(s) if the RetrievalRequest is in response to a Fetch Instruction received from the ADRF in an Nadrf_DataManagement_RetrievalNotify; or

-	DataSetTag.

Inputs, Optional: None.

Outputs Required: Result Indication.

Outputs, Optional: Data or Analytics, DSC information.

# * * * * 4th change * * * *

### 10.2.8	Nadrf_DataManagement_RetrievalNotify service operation

Service operation name: Nadrf_DataManagement_RetrievalNotify

Description: This service operation provides consumers with either data or analytics from an ADRF, or instructions to fetch the data or analytics from an ADRF. The notifications are provided to consumers that have subscribed using the Nadrf_DataManagement_RetrievalSubscribe service operation. Historical data or analytics may be retrieved from ADRF storage and data received in the future be sent when obtained by the ADRF.

Inputs, Required: Notification Correlation Information, time stamp representing time when ADRF completed preparation of the requested data.

Inputs, Optional: Requested Data or Analytics, Fetch Instructions, Termination Request, DSC information.

Fetch Instructions indicate whether the data or analytics are to be fetched from the ADRF by the Consumer. If the data or analytics are to be fetched, the fetch instructions include an address from which the data may be fetched, one or more Fetch Correlation IDs. and a deadline to fetch the data (Fetch Deadline).

Data or Analytics are fetched using the Nadrf_DataManagement_RetrievalRequest service operation.

Termination Request indicates that the ADRF requests to terminate the subscription, i.e. ADRF will not provide further notifications related to this subscription, e.g. when all data or analytics requested by the consumer have been provided to the consumer.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *