SA WG2 Meeting #156e	S2-2306214

# Elbonia, April 17th – 21st, 2023               	  			  			        (revision of 2305014r01)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0137 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HR SBO and DNS security |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Solve following EN Editor's note: It is FFS if and how DNS with security (i.e. DNSSEC, DoT and DoH) can be supported when using IP replacement (step 4C and step 1b of clause 6.7.2.3). Editor's note: It is FFS how the Local DNS server can detect which FQDN(s) are allowed for HR-SBO offloading. Usage of this configuration may not fit with HR-SBO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure in clause 6.7.2.4 assumes that the UL/CL is able to detect FQDN(s) in traffic sent to the IP address of the EASDF or of the IP address of a local DNS Server. It is thus incompatible with usage of DoT (DNS over TLS) or DoH to protect the DNS traffic exchanged between the UE and the PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aspects related with the EN are not solved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.2, 6.7.2.3, 6.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

### 5.2.1	Functional Description

The Edge DNS Client (EDC) functionality is a 3GPP functionality in the UE that ensures that DNS requests from applications are sent to the DNS Server's (e.g. EASDF/DNS resolver) IP address received from the SMF in the ePCO. The EDC functionality in the UE is a UE capability that ensures the usage of the EAS discovery and re-discovery functionalities defined in clause 6.2.

NOTE 1:	A UE without EDC functionality can use the EAS (re-)discovery functionalities provided by EASDF, but the usage of the EASDF cannot be ensured since it may use a different DNS server from the DNS server provided by the operator.

Figure 5.2-1 depicts the Edge DNS Client (EDC) functionality in the UE.

Figure 5.2-1: EDC functionality in the UE

NOTE 2:	Whether EDC functionality is provided to the consumer directly or via/by the UE's operating system is implementation specific. The APIs between EDC consumer and EDC functionality are out of scope of 3GPP.

A UE that hosts the EDC functionality indicates its capability in the PCO during the PDU Session Establishment and the PDU Session Modification procedures. The EDC functionality includes the following functionalities:

-	Configures the DNS Client with the DNS server configuration (e.g., EASDF/DNS resolver) received from the SMF in ePCO according to TS 23.501 [2] (clause 5.6.10.1).

-	DNS Client:

-	Provides the capability to the consumer in the UE to resolve FQDN using DNS Queries towards the DNS Server (e.g., EASDF/DNS resolver) indicated by the SMF.

-	Sends DNS Queries towards the DNS Server indicated by the SMF via the related PDU session.

-	Forwards EAS IP addresses and other relevant information included in the DNS responses received from the DNS Server to the consumer in the UE.

-	Provision of DNS settings (Optional):

-	Provides to the consumer in the UE the IP address of the DNS Server (e.g., EASDF/DNS resolver) received from the SMF. The consumer in the UE can explicitly request the DNS Server's IP address and/or can subscribe/unsubscribe to receive updates of the DNS Server's IP address.

When the UE performs an FQDN resolution request for an application, the UE shall use the EDC functionality to perform the DNS resolution in one of the following cases:

-	the application mapped onto the PDU Session explicitly requests the use of the EDC functionality and the SMF indicated, at PDU Session Establishment or at PDU Session Modification, that the use of the EDC functionality is allowed for that PDU session; or

-	the SMF indicated, at PDU Session Establishment or at PDU Session Modification, that the use of the EDC functionality is required for the PDU Session for the specific DNN. In this case, the UE shall use the EDC functionality for all the applications mapped onto that PDU Session.

NOTE 3:	Whether the specific DNN(s) is applied is based on the agreement between the MNO and the application provider.

NOTE 4:	User preferences at OS level related to the use of DNS server do not apply when the EDC functionality is used.

NOTE 5:	It is subject to local regulatory requirements whether the MNO can force the UE to use EDC functionality.

2nd CHANGE

#### 6.7.2.2	PDU Session establishment for supporting HR-SBO in VPLMN

Figure 6.7.2.2-1: Procedure for PDU Session establishment supporting HR-SBO in VPLMN

1.	During the registration procedure, the AMF receives the HR-SBO allowed indication per DNN/S-NSNAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

2.	During the PDU Session establishment procedure for Home-routed roaming as in clause 4.3.2.2.2 of TS 23.502 [3], if the AMF had received in SMF selection data from UDM the HR-SBO allowed indication for the DNN/S-NSSAI in the step 1, the AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the V-SMF in the step 2 and the step3a of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

Editor's note:	It is FFS how to route the DNS traffic between the UE and the V-EASDF where multiple DNN networks with the same IP address range are deployed in different HPLMNs or in the same HPLMN.

If the V-SMF supporting the HR-SBO receives the HR-SBO allowed indication from AMF, the V-SMF may:

-	select UL-CL and PSA functionalities based on UE location information and this indication in the step 4 of the Figure 4.3.2.2.2-1 of TS 23.502 [3].

NOTE 1:	The UL-CL UPF and PSA UPF can be co-located in the single V-UPF.

-	select a V-EASDF,

-	obtain the V-EASDF IP address based on local configuration, or invoke Neasdf_DNSContext_Create Request including the DNN, S-NSSAI, HPLMN ID and the UE IP address set to unspecified address as specified in clause 7.1.2.2 to obtain the V-EASDF IP address, and

-	send to the H-SMF the request for the establishment of the PDU Session supporting HR-SBO in VPLMN and the V-EASDF IP address to in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

The H-SMF authorizes the request for HR-SBO based on SM subscription data (i.e. HR-SBO authorization indication) in the step 7 of the procedure in the clause 4.3.2.2.2-1 of TS 23.502 [3].

Once the HR-SBO is authorized, the H-SMF requests and retrieves the optional VPLMN Specific Offloading Policy from H-PCF.

If HR-SBO is authorized for the PDU session, the H-SMF provides in the Nsmf_PDUSession_Create Response in the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] with the following information:

-	optional VPLMN Specific Offloading Information

-	the V-EASDF IP address (corresponding to clause 6.7.2.3) or DNS server IP address of HPLMN (corresponding to clause 6.7.2.5) as DNS server address to be sent to the UE via PCO and

-	the DNS server address of HPLMN (to be used for DNS requests related with traffic not to be subject to HR-SBO)

-	the HR-SBO authorization result (i.e. whether HR-SBO request is authorized or not).

The H-SMF may indicate to the UE either that for the PDU Session the use of the EDC functionality is allowed or that for the PDU Session the use of the EDC functionality is required.

If the request for HR-SBO is not authorized and DNS context has been created, the V-SMF delete the DNS context from the selected V-EASDF, and the subsequent steps related to the EASDF in this procedure are skipped.

The detailed information of VPLMN Specific Offloading Policy is described in clause 6.4 of TS 23.503 [4].

NOTE:	The VPLMN Specific Offloading Policy can be prior configured in HPLMN based on the service level agreement between the VPLMN and HPLMN.

3.	The V-SMF configures the V-EASDF with the DNS handling rules using the received VPLMN specific offloading policy and may configure the V-EASDF with the DNS server address of HPLMN as default DNS server, after the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] if they are received from H-SMF in the step 2.

If the V-SMF has interacted with the V-EASDF in step 2, then the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The V-SMF configures the V-UPF selected in the step 2 to forward DNS messages to V-EASDF.

4A.	EAS Discovery procedure with V-EASDF is performed as described in clause 6.7.2.3.

4B.	EAS Discovery procedure with Local DNS Server/Resolver is performed as described in clause 6.7.2.4.

4C.	EAS discovery procedure with V-EASDF using IP replacement mechanism as described in clause 6.7.2.5.

#### 6.7.2.3	EAS Discovery Procedure with V-EASDF for HR-SBO

Figure 6.7.2.3-1: Procedure for EAS Discovery with V-EASDF for HR-SBO roaming scenario

1.	The DNS query sent by the UE reaches the V-EASDF via an UPF controlled by the V-SMF.

If the target FQDN of the DNS query is not part of the FQDN authorized by the H-SMF in step 2 of Figure 6.7.2.2-1,

a)	The V-EASDF proceeds to step 12 of Figure 6.2.3.2.2-1 where it sends the DNS request to the DNS server address of HPLMN received in step 2 of Figure 6.7.2.2-1 (to be used for DNS requests related with traffic not to be subject to HR-SBO) through N6. Upon receiving the DNS response, the procedure proceeds immediately to step 5.

Editor's note:	It is FFS how to route the DNS traffic between the V-EASDF and HPLMN DNS server when the HPLMN DNS is deployed in a private IP address range.

The rest of the procedure assumes the target FQDN of the DNS query is part of the FQDN authorized by the H-SMF in step 2 of Figure 6.7.2.2-1.

2.	The step 8 to 15 of the procedure in the Figure 6.2.3.2.2-1 by replacing SMF and EASDF with V-SMF and V-EASDF respectively.

3.	The V-SMF selects UL CL/BP and local PSA in VPLMN based on the V-EASDF notification and UE location. The V-SMF may perform insertion or change of UL CL/BP and local PSA in VPLMN .

Editor's note:	It is FFS how the VPLMN Specific Offloading Information is used by V-SMF in VPLMN.

Editor's note:	How to retrieve QoS parameter from HPLMN to control local PSA is FFS.

If there is no other V-UPF between the selected UL CL/BP in this step and H-UPF, the V-SMF sets up user plane between this UL CL/BP and H-UPF via the interaction with H-SMF. Otherwise, the V-SMF sets up user plane between this ULCL/BP and the existing V-UPF.

The V-SMF sets up user plane between the selected UL CL/BP in this step and RAN (if no other V-UPF exists between RAN and this UL CL/BP) or the V-UPF (if exists between this UL CL/BP and RAN).

NOTE:	In the home routed roaming scenario, the V-UPF can be deployed at a central area within VPLMN. In this case, the V-UPF is located in the user plane path between ULCL/BP UPF in VPLMN and PSA-UPF in HPLMN. In some deployments, the ULCL/BP UPF can be collocated with the V-UPF.

4.	The steps 17 to 18 of the procedure in clause 6.2.3.2.2 by replacing SMF and EASDF with V-SMF and V-EASDF respectively.

5.	V-EASDF sends the DNS Response to the UE.

#### 6.7.2.4	EAS Discovery Procedure with Local DNS for HR-SBO

Figure 6.7.2.4-1: Procedure for EAS Discovery with local DNS for HR-SBO roaming scenario

The procedure in this clause 6.7.2.4 assumes that the UL/CL is able to detect FQDN(s) in traffic sent to the IP address of the EASDF or of the IP address of a local DNS Server. It is thus incompatible with usage of DoT (DNS over TLS)  or DoH to protect the DNS traffic exchanged between the UE and the PLMN.

EAS Discovery procedure with local DNS are corresponding with the procedure Figure 6.2.3.2.3-1. The steps 1 to 5 are the same as the steps 1 to 6 of Figure 6.2.3.2.2-1 with following differences:

-	SMF is replaced with V-SMF.

-	UE, (R)AN, AMF, ULCL/BP UPF, L-PSA UPF, V-SMF, Local DNS Resolver/Server are located in VPLMN.

-	UPF, H-SMF, C-DNS are located in HPLMN.

0.	The HR-SBO PDU Session is established. See the procedure in clause 6.7.2.2.

1,	ULCL/BP insertion. See the step 1 of the procedure in Figure 6.2.3.2.3.

2,	After ULCL/BP insertion is performed, the V-SMF sends new local DNS server address to the UE by performing PDU Session Modification procedure as in clause 4.3.3.3 of TS 23.502 [3] with following additions:

-	V-SMF sends Local DNS Server/Resolver to the H-SMF in the step 1a of the procedure as in clause 4.3.3.3 of TS 23.502 [3].

-	H-SMF sends the Local DNS Server/Resolver to be sent to the UE via PCO to the V-SMF in the step 3 of the procedure in clause 4.3.3.3 of TS 23.502 [3].

3.	See the step 4 of the procedure in Figure 6.2.3.2.3. If the target FQDN of the DNS query is not part of the FQDN authorized by the H-SMF in step 2 of Figure 6.7.2.2-1, the UL CL/BP UPF sends the DNS request to the DNS server address of HPLMN via V-UPF (if exists) and H-UPF (through N9), by modifying the packet's destination IP address (corresponding to V-EASDF or local DNS server IP address) to that of the DNS server of HPLMN

4.	See the step 5 of the procedure in Figure 6.2.3.2.3.

5.	See the step 6 of the procedure in Figure 6.2.3.2.3.

NEXT CHANGE (2)

END OF CHANGES