SA WG2 Meeting ##157	S2-2306291

22 - 26 May, 2023, Berlin, Germany

3GPP TSG-CT WG1 Meeting #141-e	C1-232736

# Online, 17 – 21 April 2023

Title:	Clarifications on location validity information

Release:	18

Work Item:	eNPN_Ph2

Source:	CT1

To:	SA2

Cc:	none

Contact person:	Amer Catovic

amerc@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT1 is discussing how to implement the requirements in TS 23.501 related to location validity information in the automatic SNPN selection when the UE supports localized services in SNPN and access for localized services in SNPN is enabled. CT1 has detected the following inconsistency in the requirements: in some places, the location validity information restricts selection of an SNPN:

Validity information consists of

-	Time validity information, i.e., time periods (defined by start and end times) when access to the SNPN is allowed; and/or,

-	Location validity information

…

…the UE shall select and attempts registration on available SNPN in the following order:

(a)	if the UE supports access to an SNPN using Credentials from a Credentials Holder then the UE continues by selecting and attempting registration on available and allowable SNPNs which broadcasts the indication that access using credentials from a Credentials Holder is supported in the following order:

i	the SNPN with the validity information the UE was last registered with (if the validity information is met) …

while in other places, the location validity information does not restrict the selection of an SNPN:

NOTE 2: The location validity information is used to aid the UE where to search for the SNPNs in the Credentials Holder controlled prioritized list of SNPNs and GINs and is not used for any area restriction enforcement.

Question 1: Is the location validity information (a) used by the UE to restrict the selection of SNPN or (b) used only to aid the UE in the search for SNPN?

CT1 kindly asks SA2 to clarify SA2 specifications based on the answer to Question 1.

# 2	Actions

To SA2

ACTION: 	CT1 kindly asks SA2 to answer the above question and clarify SA2 specification(s).

# 3	Dates of next TSG CT WG 1 meetings

CT1#142	22-26 May 2023		              Bratislava, SK

CT1#143              21-25 August 2023                          Goteborg, SE