3GPP TSG-SA WG2 Meeting #157	S2-2306365

# Berlin, Germany,  May 22-26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0820 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enforcing periodic reporting for ML model accuracy monitoring in case of multiple subscriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In S2-2306074, it is approved that when AnLF notifies MTLF for ML model accuracy (which is requested by MTLF via Nnwdaf_MLModelMonitor_Subscribe), it also includes the number of inferences used to measure the accuracy. Currently, in the subscribe request, both "Reporting Threshold(s)" and "Reporting Period" are optional inputs, which is problematic if MTLF subscribes to multiple AnLFs for monitoring the same Analytics and ML model; for example if (i) MTLF subscribes to two AnLFs A and B for monitoring the same ML model and only specifies Reporting Threshold, (ii) AnLF A performs many inferences where the output predictions do not deviate significantly from the ground truth data, hence the measured accuracy by this AnLF is high and above the Reporting Threshold, (iii) AnLF B performs very few inferences, but the accuracy is lower than the Reporting Threshold, then in this case, AnLF A never notifies MTLF because its measured accuracy is higher than the Reporting Threshold, but AnLF B may send multiple notifications indicating low accuracy (but just for very few inferences). Therefore, the MTLF will be biased with this feedback from AnLF B and may retrain the model (using the data sent back by AnLF B) while unaware of the good performance of the model in AnLF A. To address this issue, this CR proposes that "Reporting Period" be made a mandatory input of Nnwdaf_MLModelMonitor_Subscribe if MTLF subscribes to multiple AnLFs to monitor the same Analytics ID and ML model. And to reduce the volume of data sent from AnLF to MTLF, AnLF includes ground truth and network data (which can be used by MTLF to retrain the model) in the notification only if deviation of the analytics by the model from the ground truth data is greater than the threshold. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Making the "Reporting Period" as a mandatory input if MTLF subscribes to multiple AnLFs to monitor the same Analytics ID and ML model. Making sending ground truth and network data from AnLF to MTLF optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Biased model retraining by infrequent low accuracy inferences. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.3.3, 7.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

#### 6.2E.3.3	Procedures for monitoring the analytics accuracy of an ML model

An NWDAF containing MTLF, due to the registration of monitoring of the analytics accuracy of an ML model received from NWDAF containing AnLF and local policies, subscribes to the NWDAF containing AnLF for receiving notifications of the accuracy of the ML Model. The NWDAF containing MTLF may get the Subscription endpoint address of the NWDAF containing AnLF from the information received in a previous registration or through a service discovery procedure at the NRF.

Figure 6.2E.3.3-1 illustrates the procedure for monitoring the analytics accuracy of an ML model. Nnwdaf_MLModelMonitor_Subscribe and Nnwdaf_MLModelMonitor_Notify service operations are used for that purpose. A service consumer, i.e. an NWDAF containing MTLF, subscribes at a service producer, i.e. an NWDAF containing AnLF, to be notified when the analytics accuracy of the previously provisioned ML model is not sufficient or the reporting period is reached.

Figure 6.2E.3.3-1: Procedure for monitoring the analytics accuracy of an ML model

0.	Upon the reception of an Nnwdaf_MLModelMonitor_Register request and based on local policy, the NWDAF containing MTLF determines to subscribe to the analytics accuracy monitoring for the ML model.

1.	The NWDAF containing MTLF sends an Nnwdaf_MLModelMonitor_Subscribe request (Analytics ID(s), unique identifier(s) of the ML model(s) to be monitored, accuracy metrics to be monitored, optionally Reporting Threshold(s) or Reporting Period) to an NWDAF containing AnLF subscription endpoint.

NOTE: Reporting Period is mandatory input if NWDAF containing MTLF subscribes to multiple NWDAFs containing AnLF for monitoring the same Analytic ID and ML model.

2.	The NWDAF containing AnLF sends a response to the NWDAF containing MTLF.

3.	The NWDAF containing AnLF starts monitoring the analytics accuracy of the ML model(s), if it not started yet.

NOTE 1:	The NWDAF containing AnLF can monitor the analytics accuracy in many ways: e.g. comparing predictions of ML model and its corresponding ground truth data, comparing changes in internal configuration for the analytics ID generation, previous existent records of analytics accuracy information etc.

4-5. When the analytics accuracy of the ML model is insufficient i.e. deviation of the output analytics using the trained ML model from ground truth data (which are collected from Data Producer NF corresponding to analytic ID requested at the time which the prediction refers to) is greater than the Reporting Threshold(s) (which are locally configured or received in the Subscribe request), or the Reporting Period indicated in the Subscribe request is reached, the NWDAF containing AnLF sends an Nnwdaf_MLModelMonitor_Notify request to the notification endpoint (e.g. the NWDAF containing MTLF). The Notify request includes the analytics accuracy information of the ML model (e.g. a Deviation value which indicates the deviation of the predictions generated using the ML model(s) from the data, optionally the network data when the deviation occurs (which can be used by the NWDAF containing MTLF for possible ML model retraining) if deviation of the output analytics from ground truth data is greater than the Reporting Threshold(s), and optionally an indication that the analytics accuracy of the ML model does not meet the requirement of accuracy for the ML model.

6.	The NWDAF containing MTLF sends a response.

7.	When the NWDAF containing MTLF has received the multiple analytics accuracy information, from one or more NWDAFs containing AnLF, it may consider that the ML model is degraded/to be updated (i.e. enough number analytics accuracy information received from one or more NWDAFs containing AnLF, indicating insufficient analytics accuracy).

NOTE 2:	The actual mechanism for the NWDAF containing MTLF for determining the degradation of the ML model degradation is an internal procedure of the NWDAF containing MTLF, e.g. the NWDAF containing MTLF calculate a global accuracy based on the analytics accuracy information and the number of inferences received from multiple NWDAFs containing AnLF.

8.	When an ML model is considered degraded / to be updated, the NWDAF containing MTLF re-trains the existing ML model or selects a new ML model. If the network data was not included in the Nnwdaf_MLModelMonitor_Notify request of step 5, the NWDAF containing MTLF may request data from the NWDAF containing AnLF, ADRF and/or other 5GS entities as specified in clause 6.2, and use the collected data for ML model retraining. The NWDAF containing MTLF notifies the NWDAF(s) containing AnLF with the updated trained ML Model Information by invoking Nnwdaf_MLModelProvision_Notify service operation, as described in clause 6.2A.

********** Next Change **********

### 7.9.4	Nnwdaf_MLModelMonitor_Notify service operation

Service operation name: Nnwdaf_MLModelMonitor_Notify.

Description: NWDAF notifies the ML Model accuracy information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, Set of:

-	the tuple (Unique ML model identifier, ML Model accuracy information).

ML Model accuracy information may include a deviation value which indicates the deviation of the predictions generated using the ML model(s) from the ground truth data, and the number of inferences that were performed during the time interval between Nnwdaf_MLModelMonitor_Register request and the Notify request or between the time of last Notification message and the time of the current Notification message, Accuracy metrics as requested in Subscribe service operation.

Inputs, Optional: Validity period, Network data when the deviation occurs (which can be used by the NWDAF containing MTLF for possible ML model retraining).

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

********** End of Changes **********