3GPP TSG-SA2  Meeting #157	S2-2306372

# Berlin, Germany, May 22 - 26, 2023, 	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4165 | rev | - | Current version: | 18.1.1 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Management NSAC Slices. |
| --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Unspecified behavior if the operator wants to update the quota below the already admitted UEs, and PDU sessions. This can be due to contractual obligations which have to take effect immediately. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NSACF may also have to deregister a number of already registered UEs. This requires a new procedure. The existing EAC notification mechanism can be extended to enable that. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The operator may not be abe to fulfill contactual obligations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.X (new Clause), 4.2.11.Y (new Clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 4.2.11.X 	Configuration of Admission Information in NSACF

For S-NSSAI(s) subject  to NSAC, the maximum number of registered UEs to be admitted, and maximum number of allowed PDU sessions to be admitted are configured in the NSACF NF(s) handling the S-NSSAI(s), via  O&M system or other means. This configuration information is adjusted over time to adapt to dynamic conditions, e.g. increasing or decreasing the admitted numbers.

Dynamic adaptation for the number of registered UEs and/or number of PDU sessions for any S-NSSAI requires that the updated numbers provided to the NSACF NF handling the S-NSSAI do not go below the already admitted number of registered UEs and/or admitted number of PDU sessions which is known to the O&M system. However, in certain cases this cannot be avoided. In this case, the NSACF may need to deregister a certain number of registered UEs.

To deregister UEs, NSACF NF reuses the existing EAC notification mechanism as depicted in clause 4.2.11.Y. The selection of UEs to be deregistered is based on local policy in AMF.

# * * * * Next change * * * *

##### 4.2.11.Y NSACF initiated UE deregistration

If the NSACF decides to deregister a UE because of the expiration of the S-NSSAI deregistration timer or other reasons, it reuses the notification mechanism in clause 4.2.11.3 to notify the AMF about the UE ID(s)  to be deregistered. The mechanism is extended, as depicted in Figure 4.2.11.Y-1 to support sending the required information to AMF.

Figure 4.2.11.Y-1: NSACF UE Dergistration procedure

The NSACF triggers Nnsacf_NSAC_EACNotify operation including the S-NSSAI(s) , and UE IDs or a number of UEs for which the AMF initiates deregistration using existing procedures.

# * * * * End of changes * * * *