3GPP TSG SA WG2 Meeting #157	S2-2306377

# Berlin, Germany, May 22-26, 2023 												(revision S2-23cccccc)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1317 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture alignment to replace DCMF with MF Annex AC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is an alignment CR to replace DCMF with MF in the architecture where applicable, and provide a description for MF services |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace DCMF in the architecure with MF. The term DCMF will be used when a clause exclusively discusses DCMF services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.1, AC.2.1, AC.2.2.1, AC.2.2.3, AC.2.2.4, AC.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# AC.1	General

This annex describes IMS architecture enhancements to support data channel services.

Data channels are always established in the context of an IMS MMTel session.

Service based interfaces applicable to data channel services are specified in AA.2.4, and AA.2.5.

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A network function for data channel management signalling, Data Channel Signalling Function (DCSF), is specified. The network function to handle data channel media can be provided in two ways:

-	by enhancing the existing IMS MRF (called MRF) to perform media functions related to DC, which interacts with the IMS AS via enhanced Mr'/Cr interfaces; and

-	by introducing a new network function called Media Function (MF), which interacts with the IMS AS via the service-based interface DC2.

NOTE:	In this release of the specification the IMS AS supporting data channel services is collocated with the TAS.

Editor's note:	It is FFS whether and how IMS Data Channel is supported in roaming cases.

**** Next Change ****

## AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture option of IMS supporting DC usage with MF

Figure AC.2.1-2 shows the data channel architecture when using the MRF.

Figure AC.2.1-2: Architecture option of IMS supporting DC usage with MRF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this release.

NOTE 5:	Enhancements to the N33 reference point for support of data channel services are not specified in this release.

Editor's note:	Enhancements to other interfaces (e.g., N71/Sh, N70/Cx, Gm, Mw, ISC, Iq, Mb) for support of IMS data channel service are FFS.

**** Next Change ****

### AC.2.2.1	Data Channel Signalling Function (DCSF)

The DCSF is the signalling control function that provides data channel control logic. The DCSF is not involved in SIP signalling. The DCSF supports the following functionalities:

-	The DCSF receives event reports from the IMS AS and decides whether data channel service is allowed to be provided during the IMS session;

-	The DCSF manages bootstrap data channel and (if applicable) application data channel resources at the MF or MRF via the IMS AS;

-	The DCSF supports HTTP web server functionality to download data channel applications (bootstrapping) via MF and/or MRF to the UE based on UE subscription.

-	The DCSF downloads data channel applications from the Data Channel Application Repository;

NOTE 1:	The detailed procedures to download data channel applications to the UE are out of scope of this specification.

-	The DCSF interacts with NEF for data channel capability exposure via N33;

NOTE 2:	Interaction with NEF is not specified in this release of the specification.

-	The DCSF interacts with the DC Application Server for DC resource control via DC4/DC3, and for traffic forwarding via MDC3/MDC2.

NOTE 3:	Interaction with the DC Application Server for resource control is not specified in this release of the specification.

**** Next Change ****

### AC.2.2.3	Media Function (MF) / MRF

The MF and MRF provide the media resource management and forwarding of media traffic. DCMF is  one media capability  supported by MF for data channel. The MF and MRF provide the following functionalities:

-	The MF and MRF manage the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the IMS AS;

-	The MF and MRF terminate the bootstrap data channel from the UE and forward HTTP traffic between the UE and DSCF via MDC1;

-	The MF and MRF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs;

is-	The MF and MRF relays traffic on the A2P/P2A application data channel between the UE and the DC Application Server via MDC2.

**** Next Change ****

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

**** Next Change ****

## AC.2.3	Reference points

The following service-based reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and MF.

-	DC3:	Reference point between the DCSF and NEF.

NOTE:	DC3 reference point is not specified in this Release.

-	N72/Sc:	Reference point between the DCSF and HSS.

Editor's note:	It is FFS whether a new reference point between DCSF and HSS is required or N71/Sh reference point can be re-used.

The following service- based reference points are updated to support data channel signalling control in IMS:

-	N70/Cx/Dx:	Reference point between the CSCF and HSS.

-	N71:	Reference point between the IMS AS and HSS.

The following reference points are defined for data channel media handling:

-	MDC1:	Reference point for transport of data channel media between media function (either MF or MRF) and DCSF.

-	MDC2:	Reference point for transport of data channel media between media function (either MF or MRF) and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

The following reference point is updated to support data channel media handling:

-	Mr'/Cr:	SIP based reference point between IMS AS and MRF.

**** End of Changes ****