3GPP TSG SA WG2 Meeting #157	S2-2306380

# Berlin, Germany, May 22-26, 2023 													revision S2-23cccccc

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1318 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HTTP Application multiplexing within a single SCTP association. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current revision states that a downloaded HTTP application (singular) only may use the SCTP association as in which it was downloaded, for communication with its application server. This phrasing limits the possibility to establish a separate SCTP association for applications, Different from the bootstrap association. It should also be possible to start multiple local and/or remote HTTP applications in paralell. Since SCTP is perfectly suited for multiplexing, it should be mentioned that mulitple HTTP applications can be multiplexed within the same SCTP association using separate data channel streams. Replace DCMF with MF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that HTTP applications may use a separate SCTP association different from bootstrap and that multiple applications can be multiplexed on the same association. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Block the possibility to start multiple HTTP applications in paralell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# AC.6	Data Channel Media Setup

MF and the MRF provides media anchoring when needed for the IMS Bootstrap Data Channel, and the IMS Application Data Channel. To that effect, they support the following capabilities:

-	HTTP Proxy: In this configuration, MF/MRF supports terminating DTLS with HTTP traffic being transparent to MF/MRF. This mode is deployed both for Bootstrap Data Channel, and HTTP Application Data Channels. Figure AC.6-1 illustrates the protocol stack for the HTTP proxy configuration mode for the Bootstrap Data Channel case.

When used with HTTP Application Data Channel, the downloaded Data Channel application will in this case communicate with the DC Application Server, using basic HTTP signalling, within either the same bootstrap SCTP association as in which this application was downloaded or using a different SCTP association. Multiple HTTP applications can be multiplexed within the same SCTP association (using separate data channel streams) if sharing the same HTTP Proxy MF instance.

Figure AC.6-1: MF/MRF "HTTP Proxy" Media Configurations

-	UDP Proxy: In this configuration, MF/MRF transparently proxies HTTP traffic to its target. This mode is deployed only for Application Data Channels. Figure AC.6-2 illustrates the protocol stack for the UDP proxy configuration mode for the Application Data Channel case, providing a Person2Application/Application2Person/Person2Person Data Channel Application.

Figure AC.6-2: MF/MRF "UDP Proxy" Media Configurations

**** End of Changes ****