SA WG2 Meeting #157	S2-2306389

22 - 26 May, 2023, Berlin, Germany

Title:	[Draft]LS reply on TSCAI for XR

Response to:             LS on TSCAI for XR (S2-2306323/R2-2304569)

Release:	Release 18

Work Item:	XRM, NR_XR_enh-Core

Source:	SA2

To:	RAN2

Cc:	RAN3

Contact Person:

Name:	Xiaowan Ke

E-mail Address:	Xiaowan.Ke@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks RAN2 for their LS in (S2-2306323/R2-2304569) on TSCAI for XR and would like to provide the following feedback:

Question: RAN2 has discussed the provision of assistance information from CN to RAN for XR services and is wondering whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR.

SA2 Answer: in case of XR, the assistance information in TSCAI presents the traffic characteristic and is not specifically relative to GBR or non-GBR. Also, there is no requirement to allocate GBR only or non-GBR only QoS flow for XR traffic. In summary, for either GBR or non-GBR QoS flow, it is possible to provide TSCAI to the RAN when the XR traffic has related characteristics. Further, as defined in 23.501 clause 5.37.8, the following traffic assistance information may be provided by the CN to NG RAN via TSCAI:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity.

Besides, SA2 understands PDU Set handling in uplink direction may need the cooperation of UE, NG RAN and CN. But SA2 has assumed that the applicability and details of PDU Set handling in uplink direction is pending RAN WG's progress. In order to promote the corresponding SA2 work in the future meetings, SA2 kindly requests RAN2 to notify SA2 the corresponding functionality definition for PDU Set handling in uplink at RAN side when it is ready.

2. Actions:

To RAN2 group:

ACTION: 	SA2 respectfully asks RAN2 to take the above information into consideration and provide the corresponding functionality definition for PDU Set handling in uplink at RAN side when it is ready.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157	from 2023-05-22	to 2023-05-26		Berlin, Germany

TSG-SA2 Meeting #158	from 2023-08-21	to 2023-08-25		Goteborg, Sweden