3GPP SA WG2 Meeting #157		S2-2306443

Berlin, May 22 – 26 2023		(revision of S2-22xxxx)

# Source:	Ericsson

# Title:	Study on IMS event exposure framework

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on IMS event exposure framework

Acronym:	FS_IMS-EF

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X | AF |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

# 3	Justification

There are currently no standard defined mechanisms in IMS for an external (to the PLMN trust domain) API invoker to subscribe to specific events related to a specific IMS subscriber / group of IMS subscribers.

The serving IMS NFs (providing / producing said events / data) cannot be dynamically located by the API invoker.

An example of a specific event that can be subscribed to is “IMS user call-state” (e.g., idle, busy, registered etc.). This specific event may be used by an external AF (e.g., a backend bank fraud system) to obtain the call-state of an IMS user. This information can be input to fraud algorithms (e.g., suspicious activity on User-X’s bank account plus user X call state = busy indicates a heightened risk of fraud).

An IMS subscriber may be identified by for example an IMPU/MSISDN.

A group of IMS subscribers may be identified by for example a group identifier (list of IMPUs/MSISDNs).

# 4	Objective

The objective of the study is to update the IMS reference architecture (i.e., TS. 23.228) to provide a framework whereby external (to the PLMN trust domain) API invokers can dynamically subscribe to IMS events related to a specific IMS subscriber or group of IMS subscribers via subscribe / notify mechanism.

The study shall investigate framework to support:

- event subscription mechanisms via subscribe / notify procedures to external API invokers,

- identification of IMS events that can be subscribed to,

- locator functionality whereby the IMS NF / IMS NFs serving the specific subscriber(s) for which the specific event was requested is found,

- subscribe / notify procedures from the IMS NF / IMS NFs serving the specific subscriber(s) for the requested event.

- synergies with the Network Exposure Framework defined in 5GS (e.g., exposure of events to external API invokers via the network exposure function (NEF) from the IMS NF / IMS NFs).

Time Estimate : TBD

# 5	Expected Output and Time scale

| New specifications / TR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | - | Study on IMS event / data exposure framework | SA 104 TBD (June 2024) | SA 106 TBD | Foti, George, Ericsson |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Foti, George, Ericsson

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Collaboration with other groups (e.g. SA3) as part of the study will be considered on a need basis.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Ericsson

Telefonica?

Verizon??

Vodafone?