SA WG2 Meeting #S2-157	S2-2306476

22 - 26 May, 2023, Berlin, Germany

Source:	NTT DOCOMO

Title:	Alignment of Routing ID description with RAN3 incoming LS

Document for:	Discussion

Agenda Item:	6.8

Work Item / Release:	5GS_Ph1 / Rel-16

Abstract:	This contribution discusses stage 2 decsription about Routing ID regarding incoming LS from RAN3 on the maximum length of Routing ID (S2-2306313/R3-232170).

- Background

RAN3 clarified the Routing ID of LMF in NRPPa transport messages is UUID and its maximum length is 16 octs according to incoming LS from RAN3 on the maximum length of Routing ID (S2-2306313/R3-232170).

Current version of stage 2 document TS23.273 mentions that AMF includes a Routing identifier such as a global address of LMF in the transport message in step 3 of clause 6.11.2.

The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF (e.g. a global address of the LMF).

Since UUID is not included in the global address, RAN3 clarification and TS23.273 description is inconsistent.

- Way Forward

There are three options/alternatives SA2 can take.

- Modifying TS23.273 from the new version.

  - Less impact on TS23.273, but less alignment with RAN3 clarificatioin

  - CR draft for this option is S2-23xxxxx

  - Reply LS to incoming LS from RAN3

- Modifying TS23.273 from Rel-16 (FASMO) to align with RAN3 modification

  - More impact on TS23.273, full alignment with RAN3 clarification

  - CR drafts for this option are S2-230xxxx, 230xxxx, 230xxx

  - Reply LS to incoming LS from RAN3

- No modification on TS23.272 and ask RAN3 to align stage 2 specificaion

  - No impact, no alignment with RAN3 clarification

  - Reply LS to incoming LS from RAN3 asking RAN3 and in addition CT4 to use global address

- Proposal

It is proposed to discuss the above and select a way foward.