3GPP TSG-SA2 Meeting #157	S2-2306497

22-26 May 2023, Berlin, Germany	(revision of S2-2306246)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3954 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs in the Procedures for Consolidated-MBR Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson, Huawei, ETRI, OPPO, Samsung], Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solving the ENs in clause 4.15.6.13.2: - Proper reference to the UPF Event Exposure needs further clarification. - Whether and how to leverage direct or indirect UPF event notification is FFS. - Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS. Solving the ENs in clause 4.15.6.13.3: - Whether and how to leverage direct or indirect UPF event notification is FFS. - Proper reference to the UPF Event Exposure needs further clarification. - Whether and how to activate the Consolidated MBR monitoring during the create procedure is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To solve the ENs, the following changes are made: - For "Proper reference to the UPF Event Exposure needs further clarification", the reference for UPF Event Exposure is added to step 12 and 13 in clause 4.15.6.13.2, and step 10 in clause 4.15.6.13.3. - For "Whether and how to leverage direct or indirect UPF event notification is FFS.", descriptions are added to steps 1, 5, 12 and 13 in clause 4.15.6.13.2, and steps 1 and 2 in clause 4.15.6.13.3. - For "Whether and how to activate the Consolidated-MBR monitoring during the create procedure is FFS.", descriptions are added to steps 1, 2, 5 and 13 in clause 4.15.6.13.2, and steps 1,2 and 11 in clause 4.15.6.13.3. - Modified the Table 4.15.6.13.1-1 in clause 4.15.6.13.1 for mapping the NEF service requests of the multi-member AF sessions and the NEF requests to PCF. - Modified the descriptions in clause 4.15.6.13.1 correspondly to the above changes. Revision proposed in SA2#157 meeting: - AF provides may port number(s) corresponding to the UE IP address to NEF in creating and updaging procedure. - NEF get the private UE IP address(es) before interating 5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for Consolidated-MBR Monitoring are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.13.1, 4.15.6.13.2, 4.15.6.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision for SA2#157 meeting: As discussed in S2-2306496, in order to support NAT applied in UPF, it is proposed that AF sends the port numbers corresponding to the list of UE IP address to NEF. NEF request the private UE IP address from UPF before sending request to PCF. |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.6.13	Multi-member AF session with required QoS for a set of UEs identified by a list of UE addresses

##### 4.15.6.13.1	General Descriptions

This clause describes the procedure to request QoS and to perform QoS monitoring for the traffic flows for the communication between an AF and a set of UEs, identified by the list of UE address(es).

Editor's note:	It is FFS whether to converge with GMEC to leverage common service interface to also support AF session with required QoS for a list of UEs.

The NEF receives the Multi-member AF session request with required QoS for a set of UEs identified by their addresses. The NEF then maps the AF session to individual AF sessions with QoS (i.e., one individual AF session per UE address) and interacts with each of the UE's serving PCFs on a per AF session basis. The interaction follows the AF session with required QoS procedure as described in clause 4.15.6.6 and 4.15.6.6a, except for the involvement of the TSCTSF and the provisioning of TSCTSF related information.

The NEF receives the outcome of the individual AF session with required QoS corresponding to each UE's address and consolidates them into single response before forwarding it to the AF.

When the AF subscribes to QoS Monitoring for the set  of UEs identified by their respective addresses, the AF may provide a Consolidated MBR threshold that is stored in the NEF. If the AF requests to monitor the Consolidated MBR, the NEF subscribes to QoS Monitoring to request Data Rate monitoring for each individual AF session with required QoS parameter(s). The list of the QoS Monitoring for Data Rate reports will be sent to the AF by NEF, only if the aggregated bit rate exceeds the Consolidated MBR threshold. Otherwise, the list of QoS Monitoring for Data Rate report will be sent to the AF, by NEF, according to the reporting frequency. When the AF requests to monitor the Consolidated MBR, the NEF may include an indication of direct event notification to indicate that QoS Monitoring reports shall be sent by the UPF directly to the NEF.

The following describes the 4 types of requests for the AF session and the and the NEF requests to PCF using Npcf_PolicyAuthorization service. There may be a different PCF serving the PDU Sessions of each of the UE addresses in the Nnef_AFSessionWithQoS Request.

Table 4.15.6.13.1-1

| Nnef_AFSessionWithQoS Request | Npcf_PolicyAuthorization request |
| --- | --- |
| Create | Npcf_PolicyAuthorization Create request for each of UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization Subscribe to request QoS Monitoring request for each QoS Monitoring requests received in the Nnef_AFSessionWithQoS Request. |
| Update | Npcf_PolicyAuthorization Update request to update the QoS and/or monitoring request which may include the Consolidated MBR monitoring received in the Nnef_AFSessionWithQoS_Update Request. Npcf_PolicyAuthorization Create request for each new UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization Delete request for each removed UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization Subscribe to request QoS Monitoring request for each new QoS Monitoring requests received in the Nnef_AFSessionWithQoS Request. More than one of the above operations can be requested at the same time. |
| Revoke | Npcf_PolicyAuthorization Delete request for each removed UE address received in the Nnef_AFSessionWithQoS Request. |
| Notify | Npcf_PolicyAuthorization Notify to report the events that NEF subscribed to. |

The Consolidated Maximum Bit Rate  threshold is described as follows:

-	The Consolidated MBR threshold defines the upper bound of the aggregate bit rate across all GBR QoS Flows corresponding to the list of PDU Sessions of the UEs who participate in a list of transmissions concurrently (e.g. Federated Learning (FL) operation) with active user planes. The NEF aggregates bit rate across the set of selected QoS Flows against the Consolidated MBR threshold.

In the case of the QoS monitoring for a list of traffic flows, Consolidated MBR monitoring or concurrent reporting for the list of QoS flows' data rate or both could be supported at the same time by the NEF according to the AF request. When the AF provides the Consolidated MBR threshold, the NEF subscribes to QoS Monitoring for reporting data rates to NEF and may request direct reporting.

* * * Next Changes * * *

##### 4.15.6.13.2	Procedure for creating a Multi-member AF session with required QoS for a set of UEs

Figure 4.15.6.13.2-1: Procedures for creating Multi-member AF sessions with required QoS for a list of UEs

1.	The AF sends a request to reserve resources for the traffic flows for the communication between a set of UEs and an AF, using Nnef_AFsessionWithQoS_Create request message (a list of UE addresses, Port numbers corresponding to the UE IP addresses, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of local event notification, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20].  The AF may also provide the Consolidated MBR threshold and optionally, together with the list of UE addresses subject to Consolidated MBR monitoring and also subscribe to QoS Monitoring to monitor the UL and/or DL data rate monitoring described in clause 5.45 of TS 23.501 [2] .

Editor's note:	The name of the service is FFS.

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 8) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. The NEF generates the Transaction Reference ID and associates the list of UE addresses received from the AF to the Transaction Reference ID. The NEF determines to do Consolidated MBR monitoring, when both the Consolidated MBR threshold and a request to do QoS Monitoring on data rate are provided by the AF as described in step 1.

2a.	If the NEF has received Port Number(s) corresponding to the UE IP address(es) from AF in step 1, based on configuration, the NEF may recognize the address(es) received are the IP address(es) which are different from the actual private UE IP address(es) assigned by 5GC, i.e, the UE(s) are behind a NAT in UPFs. If so, the NEF performs steps 3 to 6 as defined in clause 6.25.10 for each UE IP address with port number.

3-4.	The NEF finds BSF serving the UE address(es) using NRF and then for each UE address, the NEF uses Nbsf_Management_Discovery service operation, providing the UE address, to discover the responsible PCF for each of the PDU Sessions.

Steps 5-7 and 9-10 apply for each UE address in the list of UE addresses.

5.	The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Create request. In the request, an indication of direct event notification may also include in the request to indicate that QoS Monitoring reports shall be sent by the UPF directly to the NEF.

6.	Step 4 in Figure 4.15.6.6-1 applies. The PCF generates authorized QoS Monitoring policy according to the QoS Monitoring information if received from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.8.2.18 of TS 23.501 [2].

7.	The PCF sends the Npcf_PolicyAuthorization_Create response message to the NEF.

8.	The NEF aggregates the authorization responses from the PCFs and sends a Nnef_AFsessionWithQoS_Create response message (Transaction Reference ID, Result for list of UE addresses) with the aggregated authorization responses to the AF.

9.	Step 6 in Figure 4.15.6.6-1 applies.

10.	Step 7 in Figure 4.15.6.6-1 applies.

11.	The NEF aggregates the notifications from the PCFs and sends a Nnef_AFsessionWithQoS_Notify message (Transaction Reference ID, Result for list of UE addresses) with the aggregated resource allocation status events to the AF. Result for list of UE addresses includes whether the resources are fully allocated, partially allocated (according to one of the Alternative Service Requirements) or not allocated for every UE address in the list. The NEF updates the list of UE addresses and the resource allocation status for every UE address in the list accordingly.

NOTE T: 	The AF may check the notification from the NEF, and for those UE address(es) that did not get any resources, the AF may request resource reservation again, by adding them to the list of UE address(es) as described in clause 4.15.6.13.3.

12.	If direct event notification is requested based on the parameters received in step 5, the QoS Monitoring events are reported by the UPF to the NEF using Nupf_EventExposure service as described in clause 5.2.26.2.

13.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the aggregated QoS Monitoring events to the AF. If the NEF monitors the Consolidated MBR threshold, the NEF aggregates the bitrate received from step 10 or step 12 which correspond to UE addresses that are in the list of UE addresses or the list of UE addresses subject to Consolidated MBR monitoring if available, and if the aggregated data rate exceeds the Consolidated -MBR threshold, the NEF sends the event report for Consolidated MBR Monitoring to the AF If the Reporting Frequency received is periodic, the NEF periodically sends the event report for Consolidated MBR Monitoring to the AF.

The AF may send Nnef_AFsessionWithQoS_Revoke request containing the Transaction Reference ID to the NEF in order to revoke the AF session related to the Transaction Reference ID. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for every UE in the list of UE address(es) associated with the Transaction Reference ID.

* * * Next Changes * * *

##### 4.15.6.13.3	Procedure for updating a Multi-member AF session with required QoS for a set of UEs

Figure 4.15.6.13.3-1: Procedure for updating a Multi-member AF session with required QoS for a set of UEs

1.	The AF which controls the operation for a multi-member AF Session invokes the Nnef_AFSessionWithQoS_Update Request to update the list of UE addresses and/or to update the resource allocations and/or to update the required QoS monitoring for a selected set of UEs to support a specific Application operation (e.g. FL operation).

When the AF requests to update the UE(s) in the list and/or to update the resource allocation and/or to update the type of QoS monitoring, the AF initiates the Nnef_AFSessionWithQoS_Update request which includes the list of UE addresses, Port numbers corresponding to the UP IP addresses, the AF Identifier, Transaction Reference ID, [Flow description information], [QoS Reference or individual QoS parameters], [Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20])]) to NEF for updating the reserved resources and/or to update the type of QoS monitoring. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned by the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The Consolidated MBR threshold for Consolidated MBR monitoring (optional); in addition, the list of UE addresses subject to Consolidated MBR monitoring may be provided. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as specified in clause 6.1.3.22 of TS 23.503 [20]. If new UE IP address is provided in this message, AF may also provide the corresponding port number to NEF.

NOTE 1:	If Consolidated MBR threshold is provided, the data rate monitoring has to be activated by the AF, i.e. the QoS parameter(s) to be measured have to indicate the data rate.

NOTE 2:	When the Consolidated MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated MBR monitoring is also provided together with the Consolidate MBR threshold, then such list has to be the subset of the list of UE addresses.

NOTE 3:	If AF wants to terminate the Consolidated MBR monitoring for the list of AF sessions, AF does not include the Consolidate MBR threshold in the AF request.

2.	NEF authorizes the Multi-member AF request and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all following steps are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

2a.	If the NEF has received new UE IP address(es) with Port Number(s) corresponding to the UE IP address(es) from AF in step 1, based on configuration, the NEF may recognize the addresses received are the IP addresses which are different from the actual private UE IP addresses assigned by 5GC, i.e, the UEs are behind a NAT in UPFs. If so, the NEF performs steps 3 to 6 as defined in clause 6.25.10 for each UE IP address with port number.

3.-5.		When the AF requests for Nnef_AFSessionWithQoS_Update in order to update the list of UE addresses, to update the QoS and/or to update the QoS monitoring, the NEF refers to the locally stored set of UEaddresses and determines which new UE address is to be added to the list, which of the existing UE address is to be removed from the list and/or which of the existing UE address is to be updated. Then, the NEF continues by invoking one of the following Npcf_PolicyAuthorization procedures for every affected UE address:

a)	If the Multi-member AF session update request involves a new UE is to be added to the list the NEF interacts with the BSF as described in clause 4.15.6.13.2 steps 3 to 4, to find out the UE's serving PCF, and then the NEF interacts with this PCF as described in clauses in steps 5 to 10 of clause 4.15.6.13.2  excluding TSCTSF related procedure towards the PCF for the corresponding UE.

b)	If the Multi-member AF session update request involves an existing UE is to be removed from the list, the NEF initiates the Npcf_PolicyAuthorization_Delete as described in clause 5.2.6.9.4 excluding TSCTSF related info towards the PCF for the corresponding UE.

NOTE X:	If an AF session update request includes both a new UE addresses(es) to be added to the list and the Consolidated-MBR threshold, The NEF updates the Consolidated MBR threshold, requests QoS for the traffic flows for the UE to AF communication and request to monitor the data rate.

c)	If the Multi-member AF session update request involves the update of the QoS or the QoS monitoring or both for existing UE addresses, the NEF initiates the Npcf_PolicyAuthorization_Update to the respective UE's serving PCFs on a per AF session basis.

NOTE Y:	What to be updated is determined based on the combination of inputs. If the AF request includes a new UE in the optional inputs, the new UE is considered to be a new UE to the list of UE(s). If the AF request does not include the existing UE in the optional inputs, the existing UE is to be removed from the list of UE(s). If the AF request includes an existing UE in the optional inputs with inputs for the resource allocation or QoS Monitoring type, the resource allocation or QoS Monitoring type of the existing UE is considered to be updated.

6.	NEF aggregates the authorization responses from the PCFs and sends the Nnef_AFSessionWithQoS Update response (Transaction Reference ID, Result for list of UE addresses) with the aggregated authorization results  to the AF. Result for list of UE addresses includes whether the request is granted or not for every UE address in the list. The NEF removes the UE address(es) to be removed from the locally stored list of UE addresses.

7.	If NEF requested Npcf_PolicyAuthorization_ Update is authorized by the PCF as described in c) of step 5 above, the PCF processes the Npcf_PolicyAuthorization_Update request according to the actions described in step 4 of clause 4.15.6.6 for SM policy Association Modification procedure and/or QoS Monitoring as described in clause 6.1.3.21 of TS 23.503 [20]. The NEF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status, QoS Monitoring for reporting data rate and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

8.-9.	When the establishment or update of transmission resources occurs , the UE's serving PCFs send Npcf_PolicyAuthorization_Notify messages to the NEF about the Resource allocation status. The NEF aggregates the notifications from the respective UEs' serving PCFs before notifying the AF with a Nnef_AFSessionWithQoS_Notify message (Transaction Reference ID, Result for list of UE addresses). Result for list of UE addresses includes whether the resources are fully allocated, partially allocated (according to one of the Alternative Service Requirements) or not allocated for every UE address in the list. The NEF updates the list of UE addresses and the resource allocation status for every UE address in the list accordingly.

NOTE Z: The AF may check the notification from the NEF, and for those UE address(es) that did not get any resources, the AF may request resource reservation again, by adding them to the list of UE address(es) as described in clause 4.15.6.13.3.

10.	If the QoS Monitoring event for Data Rate Monitoring was subscribed by the PCF, and direct event notification is requested, the SMF requests UPF to report the data rate for the target QoS flow periodically to the NEF using Nupf_EventExposure service as described in clause 5.2.26.2; otherwise, if the QoS Monitoring event for Data Rate Monitoring is unsubscribed as indicated by the PCF, then the SMF requests the UPF to terminate the QoS Monitoring reporting for the QoS Flow.

11.	UPF provides Data Rate Monitoring reporting, using Nupf_EventExposure_Notify.

12.	When NEF receives the Data Rate Monitoring report of the QoS Monitoring event for specific QoS flow from UPFs, and the Consolidated -MBR threshold was provided by the prior Nnef_AFSessionWithQoS request or updated by the AF in step 1 , the NEF sums up the data rates of all the active QoS flows according to the set of UEs which are subjected to Consolidate -MBR monitoring through the duration of the Reporting Frequency. If the given Application AI/ML traffic is asymmetric, the bit rate aggregation is done per uplink and downlink separately; otherwise, the bit rate aggregation is done per either uplink or downlink. NEF compares the aggregated bit rate of the uplink or downlink against the Consolidated MBR threshold for the uplink or downlink, accordingly.

13.	In the case of event reporting for Consolidated -MBR monitoring, only if the aggregate bit rate exceeds the Consolidated MBR threshold, the NEF will notify the AF via Nnef_AFSessionWithQoS_Notify about the status of the Consolidated MBR monitoring; otherwise, the NEF notify the AF periodically with the collected UPF QoS monitoring reports together with the status of the of the Consolidate MBR monitoring, if requested.

The UPF QoS Monitoring event for Data Rate Monitoring continues until AF terminates the Consolidated MBR monitoring. The UPF continues to perform the QoS Monitoring event for Data Rate Monitoring for the given QoS flow to the NEF periodically corresponding to the Reporting Frequency as described in steps 11-12 above.

* * * End of Changes * * *