SA WG2 Meeting #157	S2-2306499

# 22-26 May 2023, Berlin, Germany	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4173 | rev |  | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update API to support NAT applied use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys, UPEAS |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As the discssion in S2-2306496, it is propsed to have a complete solution to support NATed UPF in Rel-18. The Nnef_MultiMemberAFSessionWithQoS service which defined currently should be further updated to include the Port number. If the Service is further merged into the Nnef_AFSessionWithQoS service, it is proposed to merge the update in this CR into the Nnef_AFSessionWithQoS service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add the Port number in the Nnef_MultiMemberAFSessionWithQoS_Creat service operation. - add the Port number in the Nnef_MultiMemberAFSessionWithQoS_Creat service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure can not work if NAT is applied in UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.31.2; 5.2.6.31.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | It is related to the single API discussion between AIMLsys and GMEC, if the Nnef_MultiMemberAFSessionWithQoS_Creat and Nnef_MultiMemberAFSessionWithQoS_Creat service operation are removed and merged in to the Nnef_MultiMemberAFSessionWithQoS service, the proposal in the CR will apply to the Nnef_MultiMemberAFSessionWithQoS service. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 5.2.6.31.2	Nnef_MultiMemberAFSessionWithQoS_Create service operation

Service operation name: Nnef_MultiMemberAFSessionWithQoS Create

Description: The consumer requests the network to allocate QoS dedicated resources for a specific group of AF sessions and potentially to perform QoS monitoring (e.g. delay, jitter, bit rate or Consolidated-MBR).

Inputs, Required: AF Identifier, a list of UE addresses (i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: Port number(s) corresponding to the UE IP address(es), Time Period, Traffic Volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Consolidated-MBR Threshold, a list of UE addresses subjected to Consolidated-MBR monitoring, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available.

NOTE 1:	If Consolidated-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE 2:	When the Multi-member AF request is for Consolidated-MBR monitoring is set for event reporting, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Consolidated-MBR threshold is exceeded. NOTE-3: When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided, then such list has to be the subset of the list of UE addresses.

Outputs, Required: Transaction Reference ID, result for the success or failure of the request corresponding to individual UE(s) in the list.

Output (optional): None.

# >>>> NEXT CHANGES<<<<

##### 5.2.6.31.5	Nnef_MultiMemberAFSessionWithQoS_Update service operation

Service operation name: Nnef_MultiMemberAFSessionWithQoS_Update

Description: The consumer requests the network to update list of UEs (identified by IP addresses) of the list of AF sessions, the Service Requirement(s) and/or additional Alternative Service Requirement(s) for a list of AF sessions and/or the type of QoS monitoring operation.

Inputs, Required: Transaction Reference ID. AF Identifier, a list of UE addresses (i.e. IP addresses), Flow description information as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier.

Inputs, Optional: Port number(s) corresponding to the UE IP address(es), QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Consolidated-MBR Threshold, a list of UE addresses subjected to Consolidate-MBR monitoring, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], flow direction.

NOTE 1:	If Consolidated-MBR Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE 2:	When the AF request is for Consolidated-MBR monitoring is set for event reporting, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group-MBR threshold is exceeded.

NOTE 3:	When the Consolidated-MBR threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subjected for Consolidated-MBR monitoring is also provided, then such list has to be the subset of the list of UE addresses.

NOTE 4:	If AF wants to terminate the Consolidated-MBR monitoring, AF does not include the Consolidate-MBR threshold in the AF request.

Outputs, Required: Transaction Reference ID, result for the success or failure of the operation execution result.

Output (optional): None.

# >>>>END OF CHANGES<<<<