SA WG2 Meeting #157	S2-2306510

# Berlin, Germany, May 22nd – 26th, 2023           	  			  			       (revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4592 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of TL Containers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Editor’s Notes in Annex M.1 were resolved with the approved CRs defined in S2-2306230 and S2-2306231. Based on these approvals this CR provides a proposal for the TL Container content with a complete list of parameters and respective information on references and notes. Learning of TL support based on the Feature flag. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of TL container content. Usage of Feature Flag to learn the capability of support of TL at SMF/CUC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 work is postponed due to missing information on TL-Container handling. Stage 3 has the capability to use Feature Flag to learn the support of TL, which would be discrepancy if not defined in SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.28a.2	Transfer of TL-Container between SMF/CUC and AN-TL and CN-TL

When NG-RAN and UPF has signalled the support of AN-TL and CN-TL, the SMF/CUC may use the TL-Container to send a:

1)	get-request.

2)	set-request: submits the following elements of the Status group to the AN-TL or CN-TL:

-	list of InterfaceConfiguration(s) (one InterfaceConfiguration is associated with each QFI in the N3 tunnel)

submits the following elements of Talker group to the AN-TL or CN-TL, as derived by the SMF/CUC:

-	Interval.

-	EarliestTransmitOffset.

-	LatestTransmitOffset.

The AN-TL or CN-TL may use the TL-Container to send a:

1)	get-response: indicates the following elements of the Talker or Listener group from the AN-TL or CN-TL:

-	EndStationInterfaces: list of InterfaceIDs, one InterfaceID is associated with each Local F-TEID.

-	InterfaceCapabilities.

-	Buffer capability: maximum possible buffer duration supported by the Talker.

2)	set-response: acknowledges the reception of the Status group to the SMF/CUC.

Details on the TL-Container information are provided in table [K.3.X], Annex K.

The SMF may request the UPF to report AN-TL or CN-TL information by including a TL-Container with a get-request to the AN-TL or CN-TN, respectively. The get-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 of TS 23.502 [3].

If the NG-RAN/UPF supports AN-TL/CN-TL, the NG-RAN/AN-TL or UPF/CN-TL responds with a TL-Container including the elements defined for the get-response. The elements are associated with the corresponding N3 Tunnel Info (Local F-TEID) in the NG-RAN or UPF.

The SMF may submit TL-Container including a set-request the elements defined for the set-request to NG-RAN/AN-TL or UPF/CN-TL. The set-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 of TS 23.502 [3]. The InterfaceConfiguration is associated with the corresponding QFI in the N3 tunnel in the NG-RAN or UPF, respectively. The AN-TL or CN-TL uses the provided configuration for the traffic in the QoS Flow of the given QFI as described in Annex M.

SMF/CUC learns NG-RAN/UPF support of AN-TL/CN-TL based on the feature capability negotiation.

# * * * * Second change * * * *

Annex K (normative):
Port,user plane node, TL management information andexchange

# K.3	TL-Container Information

Table [K.3.X]: TL-Container Information

| TL-Container information | Supported Operations - AN-TL/CN-TL (see NOTE 1) |  | Reference |
| --- | --- | --- | --- |
|  | Talker | Listener |  |
| End Station Parameters of AN-TL/CN-TL |  |  |  |
| List of InterfaceID group(s) |  |  |  |
| >Mac Address | Get | Get | IEEE Std 802.1Q [98], Table 46-3 |
| >InterfaceName (see NOTE 2) | Get | Get | IEEE Std 802.1Q [98], Table 46-3 |
| InterfaceCapabilities (see NOTE 3) |  |  |  |
| >VlanTagCapable (see NOTE 4) | Get | Get | IEEE Std 802.1Q [98], Table 46-3 |
| >Jitter Indicator (see NOTE 4, NOTE 13) | Get | - | Annex M.1 |
| >BufferCapability (see NOTE 5) | Get | - | Annex M.1 |
| TN Stream Parameters |  |  |  |
| >Action Indicator (see NOTE 6) | Set | Set |  |
| TN Stream Identifier (StreamID) |  |  |  |
| >MacAddress | Set | Set | IEEE Std 802.1Q [98], Table 46-1 |
| >UniqueID | Set | Set | IEEE Std 802.1Q [98], Table 46-1 |
| TN Stream Identification Information for mask-and-match (see NOTE 7) |  |  |  |
| >tsnCpeMmIdMsduMaskLength | Set | Set | IEEE Std 802.1CBdb [X], clause 9.1.6.5 |
| >tsnCpeMmIdMsduMask | Set | Set | IEEE Std 802.1CBdb [X], clause 9.1.6.6 |
| >tsnCpeMmIdMsduMatch | Set | Set | IEEE Std 802.1CBdb [X], clause 9.1.6.7 |
| InterfaceConfiguration of End Station |  |  |  |
| List of Interface(s)/Port(s) |  |  |  |
| >InterfaceName (see NOTE 8) | Set | Set | IEEE Std 802.1Q [98],Table 46-3 |
| IEEE802-MacAddresses |  |  |  |
| >>DestinationMacAddress (see NOTE 8) | Set | Set | IEEE Std 802.1Q [98],Table 46-4 |
| IEEE802-VlanTag |  |  |  |
| >>PriorityCodePoint (see NOTE 9) | Set | Set | IEEE Std 802.1Q [98], Table 46-5 |
| >>VlanId (see NOTE 10) | Set | Set | IEEE Std 802.1Q [98], Table 46-5 |
| Parameters to Calculate Gate Control Information |  |  |  |
| List of TN Streams at Interface/Port |  |  |  |
| >UniqueID | Set | - | IEEE Std 802.1Q [98], Table 46-1 |
| >Interval | Set | - | IEEE Std 802.1Q [98], Table 46-8 |
| >MaxFrameSize (see NOTE 11) | Set | - | IEEE Std 802.1Q [98], Table 46-8 |
| >TimeAwareOffset (see NOTE 12) | Set | - | IEEE Std 802.1Q [98], clause 46.2.5.3.5 |
| >Jitter (see NOTE 12) | Set | - | IEEE Std 802.1Q [98], Table 46-9 |
| NOTE 1: Get = Get Response; Set = Set Request; - = not supported. NOTE 2: This parameter is optional and only required if End Station has more than one Interface/Port. It identifies the Interface/Port at the End Station. NOTE 3 Interface Capabilities are identical for all Interfaces/Ports of the End Station. NOTE 4: Possible values are TRUE or FALSE. NOTE 5: Maximum possible buffer duration for a packet with the maximum size of an Ethernet packet (1522 Bytes). NOTE 6: Possible values are ESTABLISH or TERMINATE the TN stream. NOTE 7: The parameters are provided if mask-and-match stream identification is supported and if Action Indicator is set to ESTABLISH. NOTE 8: The parameter is provided if End Station supports multiple Interfaces/Ports. NOTE 9: The parameter is provided if Action Indicator is set to ESTABLISH and if Stream Transformation is performed at AN-TL/CN-TL and if Active Destination MAC and VLAN stream identification is signalled from SMF/CUC to TN CNC and if TN CNC provides it in the status group for the Interface/Port. NOTE 10: The parameter is provided if Action Indicator is set to ESTABLISH and if Stream Transformation is performed at AN-TL/CN-TL and if Active Destination MAC and VLAN stream identification is signalled from SMF/CUC to TN CNC and if TN CNC provides it in the status group for the Interface/Port and End Station VlanTagCapable is TRUE. NOTE 11: Message Length of the Stream including the GTP-U header size. NOTE 12: The parameter is provided if TN Stream is scheduled. NOTE 13: If TRUE, the queuing implementation cannot avoid Jiiter as explained in IEEE Std 802.1Q Annex U.1.1 and U.1.2. NOTE 14: Based on the implementation of SMF/CUC, the parameters shall accordingly be used. |  |  |  |

END OF CHANGES