3GPP TSG-SA WG2 Meeting #157	S2-2306534

# Berlin, Germany, May 22-26, 2023	                                             (revision of S2-305478)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0945 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ki#1 – Removal of Editor´s Notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson? OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 04/07/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Proposal to remove Editor´s Note related to DNN value in the RSD of the VPLMN Specific URSP Rules. Editor´s Note: The HPLMN value DNN selection is FFS. Under the assumption that there may be different DNNs to access e.g. a corporate access at the HPLMN and per VPLMN, there are two options, a) the VPLMN specific URSP Rule contains a DNN in the subscribed DNN list and then the VPLMN knows the mapping to local DNN or b) the VPLMN Specific URSP Rule contains a local DNN value, and then the HPLMN needs to know what the DNN value is in each PLMN. Option b) implies more operation cost for the Home operator, as such Option a) is proposed. In addition, it is also possible that the HPLMN operator wants to route traffic different at the HPLMN or in each VPLMN. As such, the VPLMN specific URSP Rule will route traffic to the HPLMN or VPLMN as of today, i.e. if the LBO is allowed and local policies in the AMF, no need to replace the DNN. Proposal to remove the Editor´s Note on the set up of the PDU Session in LBO using VPLMN specific URSP Rules. Editor´s Note: How to ensure that the enforcement of a VPLMN specific URSP rule sets up a PDU Session routing traffic to the VPLMN is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The first Editor´s Note is removed, by stating that VPLMN specific URSP Rules contain DNN values that are within the subscribed DNN list for this UE and the LBO roaming information indicates that roaming is allowed. The second Editor´s Note is removed, the PCF may set the PCRT for SMF selection management at the AMF to provide a list of DNNs that are candidate for replacement. The PCF provides to the SMF a DNN value to route to enable selection of SMF in the VPLMN, if required, depending on the DNN value. When the UE evaluates URSP Rules in roaming, the VPLMN specific URSP Rules are evaluated first, and next URSP Rules that are applicable to both HPLMN and VPLMN. Align the term of non-VPLMN specific rule used in other places. Clarification on the sentence to make it readable. Clarify the H-PCF need maintain the latest PSI and Tuple and store it in UDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of KI#1 are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.6.2.2.3, 6.2.1.3, 6.1.2.2.1, 6.1.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *First change * * * *

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2], TS 23.502 [3] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Application detection filter: A logic used to detect packets generated by an application based on extended inspection of these packets, e.g. header and/or payload information, as well as dynamics of packet flows. The logic is entirely internal to a UPF, and is out of scope of this specification.

Application identifier: An identifier referring to a specific application detection filter.

Application service provider: A business entity responsible for the application that is being / will be used by a UE, which may be either an AF operator or has an association with the AF operator.

Authorised QoS: The maximum QoS that is authorised for a service data flow. In the case of an aggregation of multiple service data flows within one QoS Flow, the combination of the "Authorised QoS" information of the individual service data flows is the "Authorised QoS" for the QoS Flow. It contains the 5QI and the data rate.

Binding: The association between a service data flow and the QoS Flow transporting that service data flow.

Binding mechanism: The method for creating, modifying and deleting bindings.

Charging control: The process of associating packets, belonging to a service data flow, to a charging key and applying online charging and/or offline charging, as appropriate.

Charging key: Information used by the CHF for rating purposes.

Detected application traffic: An aggregate set of packet flows that are generated by a given application and detected by an application detection filter.

Dynamic PCC Rule: a PCC rule, for which the definition is provided to the SMF by the PCF.

Gating control: The process of blocking or allowing packets, belonging to a service data flow / detected application's traffic, to pass through to the UPF.

Monitoring key: information used by the SMF and PCF for usage monitoring control purposes as a reference to a given set of service data flows or application (s), that all share a common allowed usage on a per UE and DNN basis.

Non-3GPP access network selection information: It consists of ePDG identifier configuration, N3IWF identification and non-3GPP access node selection information, as defined in clause 6.3.6.1 in TS 23.501 [2].

Non-Seamless Offload: A capability of the UE to access the data networks via non-3GPP access (e.g. WLAN radio access) outside of a PDU Session.

Operator-controlled service: A service for which complete PCC rule information, including service data flow filter information, is available in the PCF through configuration and/or dynamic interaction with an AF.

Operating System (OS): Collection of UE software that provides common services for applications.

Operating System Identifier (OSId): An identifier identifying the operating system.

OS specific Application Identifier (OSAppId): An identifier associated with a given application and uniquely identifying the application within the UE for a given operating system.

Packet flow: A specific user data flow from and/or to the UE.

Packet Flow Description (PFD): A set of information enabling the detection of application traffic provided by a 3rd party service provider.

PCC decision: A PCF decision for policy and charging control provided to the SMF (consisting of PCC rules and PDU Session related attributes), a PCF decision for access and mobility related control provided to the AMF, a PCF decision for UE policy information provided to the UE or a PCF decision for background data transfer policy provided to the AF.

PCC rule: A set of information enabling the detection of a service data flow and providing parameters for policy control and/or charging control and/or other control or support information. The possible information is described in clause 6.3.1.

Policy control: The process whereby the PCF indicates to the SMF how to control the QoS Flow. Policy control includes QoS control and/or gating control.

Policy Control Request trigger report: a notification, possibly containing additional information, of an event which occurs that corresponds with a Policy Control Request trigger.

Policy Control Request trigger: defines a condition when the SMF shall interact again with the PCF.

Policy counter: A mechanism within the CHF to track spending applicable to a subscriber.

Policy counter identifier: A reference to a policy counter in the CHF for a subscriber.

Policy counter status: A label whose values are not standardized and that is associated with a policy counter's value relative to the spending limit(s) (the number of possible policy counter status values for a policy counter is one greater than the number of thresholds associated with that policy counter, i.e. policy counter status values describe the status around the thresholds). This is used to convey information relating to subscriber spending from CHF to PCF. Specific labels are configured jointly in CHF and PCF.

Policy Section: A Policy Section is identified by a Policy Section Identifier and consists of one or multiple URSP rule(s) or one or multiple WLANSP rule(s) or non-3GPP access network selection information or a combination of WLANSP rule(s) and non-3GPP access network selection information.

Predefined PCC Rule: a PCC rule that has been provisioned directly into the SMF by the operator.

Redirection: Redirect the detected service traffic to an application server (e.g. redirect to a top-up / service provisioning page).

Service data flow: An aggregate set of packet flows carried through the UPF that matches a service data flow template.

Service data flow filter: A set of packet flow header parameter values/ranges used to identify one or more of the packet flows in the UPF. The possible service data flow filters are defined in clause 6.2.2.2.

Service data flow filter identifier: A scalar that is unique for a specific service data flow (SDF) filter within a PDU Session.

Service data flow template: The set of service data flow filters in a PCC Rule or an application identifier in a PCC rule referring to an application detection filter in the SMF or in the UPF, required for defining a service data flow.

Service identifier: An identifier for a service. The service identifier provides the most detailed identification, specified for flow based charging, of a service data flow. A concrete instance of a service may be identified if additional AF information is available (further details to be found in clause 6.3.1).

Session based service: An end user service requiring application level signalling, which is separated from service rendering.

Spending limit: A spending limit is the usage limit of a policy counter (e.g. monetary, volume, duration) that a subscriber is allowed to consume.

Spending limit report: a notification, containing the current policy counter status generated from the CHF to the PCF.

Subscribed guaranteed bandwidth QoS: The per subscriber, authorized cumulative guaranteed bandwidth QoS which is provided by the UDR to the PCF.

Subscriber category: is a means to group the subscribers into different classes, e.g. gold user, silver user and bronze user.

UE Local Configuration: Information about the association of an application to either a PDU Session or to non-seamless Offload is configured in the Mobile Termination (MT) and in the Terminal Equipment (TE). For example, UE Local Configuration can include operator specific configuration (e.g. operator provided S-NSSAI(s)), or application specific parameters to set up a PDU Session or end user configuration for specific applications.

UE policy information: Policy information preconfigured in the UE and/or provisioned to the UE for access selection (i.e. ANDSP), PDU Session selection (i.e. URSP), V2X communications (i.e. V2XP) and/or ProSe operations (i.e. ProSeP).

Uplink binding verification: The network enforcement of terminal compliance with the negotiated uplink traffic mapping to QoS Flows.

User Preferences On Non-3GPP Access Selection: The list of configuration parameters provided by a layer (e.g. application) above NAS and used by the UE for access network discovery and selection.

VPLMN specific URSP Rules: A VPLMN specific URSP Rule is applicable when the UE is registered in the VPLMN and its equivalent PLMNs. A VPLMN specific URSP Rule may be used to route traffic to the VPLMN, if the V-PCF provides a local DNN to the AMF, using the PCRT on SMF selection management. A VPLMN specific URSP Rule may be used to route traffic to the Home PLMN if the HPLMN operator wants to provide different list of RSDs for the same Traffic Descriptor for a VPLMN and for the HPLMN. When provided, the Time and Location criteria in each of the RSD contain VPLMN values.

# * * * *Next change * * * *

6.6.2.2.3	Provision of URSP Rules to route traffic to the VPLMN

The H-PCF may provision VPLMN specific URSP Rules to UE for the purpose to route traffic to the VPLMN. The H-PCF provides VPLMN specific URSP Rules that contains HPLMN values for Network Slice Selection Policies and the DNN Selection Policies contains values within the list of subscribed DNNs and has LBO Roaming set to allowed in the UE Policy Context Subscription Data in UDR. When provided, the Time and Location criteria in each of the RSD contain VPLMN values.

The H-PCF may use application guidance on URSP determination, received from the V-PCF or retrieved from UDR at the HPLMN, as input to generate new or update existing VPLMN specific URSP Rules, as well as other input data as described in clause 6.2.1.1. The list of parameters provided for application guidance on URSP Rule determination is defined in clause 4.15.6.10 of TS 23.502 [3].

The AF may provide application guidance on URSP Rule determination to the VPLMN or to the HPLMN.

When the AF provides application guidance on URSP Rule determination to the VPLMN, it will target "PLMN ID(s) of inbound roamers", the NEF in the VPLMN authorizes requests based on local configuration using e.g., the AF identifier before storing them in UDR, as defined in in clause 4.15.6.7 of TS 23.502 [3]. The NEF in the VPLMN rejects any request for a GPSI or an External-Group-ID of a different PLMN. The UDR in the VPLMN notifies the V-PCF(s) that has subscribed to the reception of application guidance on URSP determination.

When the AF provides application guidance on URSP Rule determination to the HPLMN, it will target either a GPSI or an External-Group-ID or "any UE" of the HPLMN. The NEF in the HPLMN, may, based on the HPLMN operator local policy, authorize any request for a GPSI or an External-Group-ID and maps it into a SUPI or an Internal-Group-ID via UDM, before storing them in UDR as Application Data. The UDR notifies the H-PCF(s) that has subscribed to the reception of application guidance on URSP determination. The Application guidance on traffic routing may contain the VPLMN ID(s) where the Service Parameters apply.

When the UE Policy Association is established or the V-PCF receives updates on application guidance on URSP determination for a SUPI that has a UE Policy Association established, the V-PCF checks whether application guidance on URSP determination exists and applies for the SUPI then the V-PCF:

-	maps the S-NSSAI of the VPLMN into the S-NSSAI of the HPLMN, using the Configured NSSAI for the Serving PLMN provided by the AMF and stored in PCF; and

-	subscribes to the result of the delivery of UE Policies if it was requested by the AF to the H-PCF, using the event reporting on "Notification on outcome of UE Policies delivery" described in clause 6.1.3.18.

Editor's note:	Whether and how the AMF sends the LBO information (i.e., S-NSSAI and DNN, received as the part of the SM selection data received from the UDM) to the V-PCF when establishing the UE Policy Association is FFS.

The H-PCF generates new or updated URSP Rules using the application guidance on URSP Rule determination where the VPLMN ID included in the Service Parameters is used to indicate to the UE that this URSP Rule applies when the UE is registered in the VPLMN ID. The H-PCF provides RSD values in the URSP Rules that are within the subscribed values defined in the S-NSSAI subscription information defined in Table 6.2-1. The H-PCF provides the list of PSIs applicable per VPLMN ID(s) to the UE. The VPLMN ID(s) corresponds to the VPLMN ID(s) included in the Service Parameters provided by the AF.

Editor's note:	Whether and how URSP Rules can be wild-carded or how to include a group of V-PLMNs is FFS.

How the UE associates applications to PDU Sessions based on URSP Rules follows the procedure described in clause 6.6.2.3.

# * * * * Next change * * * *

6.2.1.3	Policy control subscription information management

The PCF may request subscription information at PDU Session establishment, PDU Session modification, during AM Policy Association Establishment procedure and during the UE Policy Association Establishment procedure.

The PCF may receive notifications on changes in the subscription information. Upon reception of a notification, the PCF shall make the policy control decisions necessary to accommodate the change in the subscription and shall update the SMF and/or the AMF if needed.

NOTE 1:	How the PCF provisions/retrieves information related with policy control subscription data is defined in TS 23.501 [2].

The policy control subscription profile information provided by the UDR during the UE Policy Association Establishment procedure using Nudr service for Data Set "Policy Data" and Data Subset "UE context policy control data" is described in Table 6.2-1:

Table 6.2-1: UE context policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |
| Tracing Requirements | Tracing requirements as defined in TS 32.421 [18] | Optional |
| PEI | The Permanent Equipment Identifier of the UE. | Optional |
| OSId | Identifies the operating system supported by UE. | Optional |
| Indication of UE support for ANDSP | Indicates the UE support for ANDSP. | Optional |
| Indication of URSP Provisioning Support in EPS | Indicates the UE support for URSP Provisioning in EPS. | Optional |
| S-NSSAI subscription information | Contains the list of subscribed S-NSSAIs, its associated subscribed DNNs. For each DNN, it includes the Allowed PDU Session types, the Allowed SSC modes, LBO roaming allowed indication and the ATSSS information (NOTE). | Optional |
| Subscriber spending limits control | Indicates whether the PCF must enforce UE policies based on subscriber spending limits. | Optional |
| NOTE: ATSSS information is defined in TS 23.502 [3] Table 5.2.3.3.1-1 and Indicates whether MA PDU Session establishment is allowed. |  |  |

Editor's note:	It is FFS whether CHF address may be added as an optional parameter to Table 6.2-1.

NOTE 2:	S-NSSAI subscription information can be part of UE context policy control subscription information and Session Management Subscription data/Slice Selection Subscription data. UDR implementation and the provisioning system are responsible for keeping the consistency of this information when both Data Sets are stored in the same UDR. The provisioning system is responsible for keeping the consistency of this information when both Data Sets are stored in different UDRs.

The policy control subscription profile information provided by the UDR at PDU Session establishment, using Nudr service for Data Set "Policy Data" and Data Subset "PDU Session policy control data" is described in Table 6.2-2.

Table 6.2-2: PDU Session policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Allowed services | List of subscriber's allowed service identifiers | Optional |
| Subscriber categories | List of category identifiers associated with the subscriber | Optional |
| Subscribed GBR | Maximum aggregate bitrate that can be provided across all GBR QoS Flows for the DNN and S-NSSAI. | Optional |
| ADC support | Indicates whether application detection and control can be enabled for a subscriber | Optional |
| Subscriber spending limits control | Indicates whether the PCF must enforce session management related policies based on subscriber spending limits | Optional |
| IP index information | Information that identifies the IP Address allocation method during PDU Session establishment | Optional |
| Background Data Transfer Reference ID(s) | Reference ID(s) for Background Data Transfer Policies that apply to the UE. | Optional |
| Local routing indication | Indication on whether AF influence on traffic routing is allowed or not allowed | Optional |
| Subscribed UE-Slice-MBR | Subscribed UE-Slice-MBR value per S-NSSAI | Conditional (NOTE 2) |
| Charging related information | This part defines the charging related information in the policy control subscription profile |  |
| Default charging method | Default charging method for the PDU Session (online / offline) | Optional |
| CHF address | The address of the Charging Function and optionally the associated CHF instance ID and CHF set ID (see clause 6.3.1.0 of TS 23.501 [2]) | Optional |
| Usage monitoring related information | This part includes a list of usage monitoring profiles associated with the subscriber. Each usage monitoring profile is logically associated with a particular operator offer, and includes the following elements |  |
| Monitoring key | An identifier to a usage monitoring control instance that includes one or more PCC rules | Conditional (NOTE 1) |
| Usage monitoring level | Indicates the scope of the usage monitoring instance (PDU Session level or per Service) | Optional |
| Start date | Start date and time when the usage monitoring profile applies | Optional |
| End date | End date and time when the usage monitoring profile applies | Optional |
| Volume limit | Maximum allowed traffic volume | Optional |
| Time limit | Maximum allowed resource time usage | Optional |
| Reset period | Time period to reset the remaining allowed consumed usage for periodic usage monitoring control (postpaid subscriptions) | Optional |
| MPS subscription data | This part defines the MPS subscription information in the policy control subscription profile |  |
| MPS priority | Indicates subscription to MPS priority service; priority applies to all traffic on the PDU Session | Conditional (NOTE 1) |
| IMS signalling priority | Indicates subscription to IMS signalling priority service; priority only applies to IMS signalling traffic | Conditional (NOTE 1) |
| MPS priority level | Relative priority level for multimedia priority services | Conditional (NOTE 1) |
| MCS priority | Indicates subscription to MCS priority service; priority applies to all traffic on the PDU Session | Conditional (NOTE 1) |
| MCS priority level | Relative priority level for MCS services | Conditional (NOTE 1) |
| NOTE 1: The information is mandatory if the specific part is included in the subscription information (e.g. the monitoring key is mandatory if the usage monitoring information part is included). NOTE 2: The information is used in PCF as described in clause 6.2.1.9 when the monitoring of the UE-Slice-MBR for an S-NSSAI is performed at the PCF. There may be a UE-Slice-MBR value for each S-NSSAI, if applicable. |  |  |

NOTE 3:	Subscribed UE-Slice-MBR can be part of the Access and Mobility Subscription data as described in clause 5.2.3.3.1 of TS 23.502 [3] and can be part of the PDU Session policy control subscription information as described in Table 6.2-2. UDR implementation and the provisioning system are responsible for keeping the consistency of this information when both Data Sets are stored in the same UDR. The provisioning system is responsible for keeping the consistency of this information when both Data Sets are stored in different UDRs.

Table 6.2-3: Remaining allowed usage subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Remaining allowed usage related information | This part includes a list of Remaining allowed usage associated with the subscriber. |  |
| Monitoring key | An identifier to a usage monitoring control included one or more PCC rules | Conditional (NOTE 1) |
| Usage monitoring level | Iindicates the scope of the usage monitoring (PDU Session level or service level) | Optional |
| Volume usage | Remaining allowed traffic volume | Optional |
| Time usage | Remaining allowed resource time usage | Optional |
| NOTE 1: The information is mandatory if the specific part is included in the subscription information (e.g. the monitoring key is mandatory if the usage monitoring information part is included). |  |  |

The Allowed services may comprise any number of service identifiers allowed for the subscriber in the PDU Session. The PCF maps those service identifiers into PCC rules according to local configuration and operator policies.

The Subscriber category may comprise any number of identifiers associated with the subscriber (e.g. gold, silver, etc.). Each identifier associates operator defined policies to the subscriber that belong to that category.

The Usage monitoring related information may comprise any number of usage monitoring control instances associated with the subscriber. In each usage monitoring control instance is mandatory to include the Monitoring key. The Reset period only applies to usage monitoring control instances that periodically reset the allowed usage (e.g. daily, monthly, etc.). If the Reset period is not specified, the usage monitoring control instance ends when the allowed data is consumed or when the End date is reached. The usage monitoring related information is used by the PCF instead of the respective information for the subscriber category.

The policy subscription profile may be extended with operator-specific information. Operator-specific extensions may be added both to any specific part of the policy control subscription information (e.g. to the subscriber category part), or as a new optional information block.

Handling of operator specific policy data by the PCF is out of scope of this specification in this release.

The latest list of PSIs and list of PSIs for the VPLMN ID(s) and its content delivered to the UE provided by the UDR during the UE Policy Association Establishment procedure using Nudr service for Data Set "Policy Data" and Data Subset "Policy Set Entry" is described in Table 6.2-4.

Table 6.2-4: Policy Set Entry

| Information name | Description | Category |
| --- | --- | --- |
| Policy Set Entry | The List of PSIs and content for each PSI. Content may be Access Network Discovery & Selection Policy Information or UE Route Selection Policy information or both. The list of PSIs per VPLMN ID(s). | Optional |

The network slice specific policy control information is per S-NSSAI information stored by the UDR and updated by the PCF during PDU Session Establishment or Modification procedure using Nudr service for Data Set "Policy Data" and Data Subset "Network Slice Specific Control Data" is described in Table 6.2-5:

Table 6.2-5: Network slice specific policy control information

| Information name | Description | Category |
| --- | --- | --- |
| Maximum Slice Data Rate for UL (per S-NSSAI) | The maximum uplink data rate for the specific network slice | Optional (NOTE 2) |
| Maximum Slice Data Rate for DL (per S-NSSAI) | The maximum downlink data rate for the specific network slice | Optional (NOTE 2) |
| Remaining Maximum Slice Data Rate for UL (per S-NSSAI) | The remaining maximum uplink data rate for the specific network slice (NOTE 1). | Optional (NOTE 3) |
| Remaining Maximum Slice Data Rate for DL (per S-NSSAI) | The remaining maximum downlink data rate limited for a specific network slice (NOTE 1). | Optional (NOTE 3) |
| NOTE 1: The initial value is set to the Maximum Slice Data Rate for UL/DL value. NOTE 2: The information is only used for limitation of data rate per network slice with assistance of the NWDAF. NOTE 3: The information is only used for limitation of data rate per network slice with PCF based monitoring. |  |  |

The policy control subscription profile information is per SUPI information, provided by the UDR using Nudr service for Data Set "Policy Data" and Data Subset " Access and Mobility policy control data" is described in Table 6.2-6:

Table 6.2-6: Access and Mobility policy control subscription information

| Information name | Description | Category |
| --- | --- | --- |
| Subscriber spending limits control | Indicates whether the PCF must enforce Access and Mobility management related policies based on subscriber spending limits. | Optional |

Editor's note:	It is FFS whether CHF address may be added as an optional parameter to Table 6.2-6.

# * * * * Next change * * * *

6.1.2.2.1	General

The 5GC shall be able to provide policy information from the PCF to the UE. Such UE policy information includes:

1)	Access Network Discovery & Selection Policy (ANDSP): It is used by the UE for selecting non-3GPP accesses and for selection of the N3IWF in the PLMN. The structure and the content of this policy are specified in clause 6.6.1.

2)	UE Route Selection Policy (URSP): This policy is used by the UE to determine if a detected application or a PIN:

-	can be associated to an established PDU Session; or

-	can be offloaded to non-3GPP access outside a PDU Session; or

-	can be routed via a ProSe Layer-3 UE-to-Network Relay outside a PDU session; or

-	multipath transmission via 5G ProSe Layer-3 UE-to-Network Relay outside of a PDU session and over Uu reference point or either path; or

-	can trigger the establishment of a new PDU Session.

The structure and the content of this policy are specified in clause 6.6.2. A URSP rule includes one Traffic descriptor that specifies the matching criteria and one or more of the following components:

2a)	SSC Mode Selection Policy (SSCMSP): This is used by the UE to associate the matching application/PIN with SSC modes.

2b)	Network Slice Selection Policy (NSSP): This is used by the UE to associate the matching application/PIN with S-NSSAI.

2c)	DNN Selection Policy: This is used by the UE to associate the matching application/PIN with DNN.

2d)	PDU Session Type Policy: This is used by the UE to associate the matching application/PIN with a PDU Session Type.

2e)	Non-Seamless Offload Policy: This is used by the UE to determine that the matching application/PIN should be non-seamlessly offloaded to non-3GPP access (i.e. outside of a PDU Session).

2f)	Access Type preference: If the UE needs to establish a PDU Session for the matching application/PIN, this indicates the preferred Access Type (3GPP or non-3GPP or Multi-Access).

NOTE 1:	The Access Type of 3GPP also includes the use of ProSe UE-to-Network Relay access as defined in TS 23.304 [34].

2g)	ProSe Layer-3 UE-to-Network Relay Offload Policy: This is used by the UE to determine if the matching application should be routed via a ProSe Layer-3 UE-to-Network Relay outside of a PDU Session. If this indication is not present the traffic shall not be routed via a ProSe Layer-3 UE-to-Network Relay outside of a PDU Session.

2h)	PDU Session Pair ID: If the UE needs to establish a PDU Session for the matching application/PIN, this indicates PDU Sessions with same PDU Session Pair ID are paired for redundant transmission.

2i)	RSN: If the UE needs to establish a PDU Session for the matching application/PIN, this indicates RSN for redundant transmission.

2j)	ProSe Multi-path Preference: It indicates to UE whether a matching application is preferred to be routed via multipath (i.e. via a PDU Session over Uu reference point and via ProSe Layer-3 UE-to-Network Relay outside of a PDU Session).

3)	V2X Policy (V2XP): This policy provides configuration parameters to the UE for V2X communication over PC5 reference point or over Uu reference point or both. V2X Policies are defined in clause 5.1.2.1 and clause 5.1.3.1 of TS 23.287 [28].

4)	ProSe Policy (ProSeP): This policy provides configuration parameters to the UE for ProSe Direct Discovery, ProSe Direct Communication, ProSe UE-to-Network Relay and Remote UE. ProSe Policies are defined in clauses 5.1.2.1, 5.1.3.1 and 5.1.4.1 of TS 23.304 [34].

The ANDSP and URSP may be pre-configured in the UE or may be provisioned to UE from PCF. The pre-configured policy shall be applied by the UE only when it has not received the same type of policy from PCF.

The methods of configuring V2XP to the UE, including (pre-) configuration and provisioning, and the priority of the same type of parameters acquired from different sources are defined in clause 5.1.1 of TS 23.287 [28].

The methods of configuring ProSeP to the UE, including (pre-)configuration and provisioning, and the priority of the same type of parameters acquired from different sources are defined in clause 5.1.1 of TS 23.304 [34].

The PCF selects the UE policy information applicable for each UE based on local configuration, operator policies taking into consideration the information defined in clause 6.2.1.2 and the PCF determines the URSP Rules for the UE using input from NWDAF as one of the inputs.

In the case of a roaming UE, the V-PCF may retrieve UE policy information from the H-PCF over N24/Npcf. When the UE is roaming and the UE has valid rules from both HPLMN and VPLMN the UE gives priority to the valid ANDSP rules from the VPLMN.

In the case of a roaming UE, the V-PCF may provide guidance on VPLMN specific URSP determination to the H-PCF as defined in clause 4.15.6.10 of TS 23.502 [3]. The H-PCF is required to generate VPLMN specific URSP rule(s) and provide the URSP rules to the UE. This can be triggered by the UE's registration in the VPLMN or it can happen before UE roams into the VPLMN. The URSP Rules received by UE in VPLMN are only applicable when the UE is registered in that VPLMN or its equivalent VPLMNs.

The UE policy information shall be provided from the PCF to the AMF via N15/Namf interface and then from AMF to the UE via the N1 interface as described in clause 4.2.4.3 of TS 23.502 [3]. The AMF shall not change the UE policy information provided by PCF.

The PCF is responsible for delivery of UE policy. If the PCF is notified about UE policy information delivery failure (e.g. because of UE unreachable), the PCF may provide a new trigger "Connectivity state changes" in Policy Control Request Trigger of UE Policy Association to AMF as defined in clause 4.16.12.2 of TS 23.502 [3]. After reception of the Notify message indicating that the UE enters the CM-Connected state, the PCF may retry to deliver the UE policy information.

NOTE 2:	For backward compatibility the PCF may subscribe the "Connectivity state changes (IDLE or CONNECTED)" event in Rel-15 AMF as defined in clause 5.2.2.3 of TS 23.502 [3].

If due to UE Local Configurations, a UE application requests a network connection using Non-Seamless Offload or ProSe Layer-3 UE-to-Network Relay Offload, the UE shall use Non-Seamless Offload for this application without evaluating the URSP rules. Otherwise, the UE shall select the PDU Session or Non-Seamless Offload in the following order:

-	If the UE has an URSP rule (except the URSP rule with the "match all" Traffic descriptor) that matches the application as defined in clause 6.6.2.3, the UE shall perform the association of the application to the corresponding PDU Session or to Non-Seamless Offload or ProSe Layer-3 UE-to-Network Relay Offload according to this rule. In roaming case, the UE firstly evaluates the VPLMN Specific URSP Rules (except the URSP rule with the "match all" Traffic descriptor)  to check that matches the application, if no match is found, the UE evaluates non-VPLMN specific URSP Rules (except the URSP rule with the "match all" Traffic descriptor) that applies to HPLMN and VPLMN; Otherwise,

-	If no URSP rule is applicable for the application (except the URSP rule with the "match all" Traffic descriptor), the UE shall perform the association of the application to a PDU Session according to the applicable UE Local Configurations, if any. If the UE attempts to establish a new PDU Session according to the UE Local Configurations and this PDU Session Establishment request is rejected by the network, then the UE shall perform the association of the application to a PDU Session or to Non-Seamless Offload or ProSe Layer-3 UE-to-Network Relay Offload according to the URSP rule with the "match all" Traffic descriptor, where the URSP rule is either a VPLMN Specific URSP Rule or a non-VPLMN specific URSP Rule considering that a VPLMN Specific URSP Rule is used first in roaming case; Otherwise,

NOTE 3:	It is assumed that the S-NSSAI(s) in the UE Local Configurations are operator-provided S-NSSAI(s). The provision of the S-NSSAI(s) is not specified.

NOTE 4:	The application layer is not allowed to set the S-NSSAI when the UE establishes a PDU Session based on the UE Local Configurations.

NOTE 5:	Any missing information in the UE Local Configurations needed to build the PDU Session Establishment request can be the appropriate corresponding component from the URSP rule with the "match all" Traffic descriptor.

-	If neither the UE Local Configurations nor the URSP rules are applicable for the application (except the URSP rule with the "match all" Traffic descriptor), the UE shall perform the association of the application to a PDU Session or to Non-Seamless Offload or ProSe Layer-3 UE-to-Network Relay Offload according to the URSP rule with the "match all" Traffic descriptor, where the URSP rule is either a VPLMN Specific URSP Rule or a non-VPLMN specific URSP Rule considering that a VPLMN Specific URSP Rule is used first in roaming case.

For the existing PDU Session(s), the UE shall examine the URSP rules within the UE policy information in order to determine whether the existing PDU Session(s) (if any) are maintained or not. If not, then the UE may initiate a PDU Session release procedure for the PDU Session(s) that cannot be maintained.

If there are multiple IPv6 prefixes within the PDU Session, then the IPv6 multi-homed routing rules, described in clause 5.8.2.2.2 in TS 23.501 [2], on the UE shall be used to select which IPv6 prefix to route the traffic of the application.

NOTE 5:	For the case that an application cannot be associated to any PDU Session, the UE can inform the application that association of the application to PDU Session fails.

The PCF may subscribe to analytics on "WLAN performance" from NWDAF following the procedures and services described in TS 23.288 [24]. When the PCF gets a notification from the NWDAF, the PCF may try to update WLANSP rules.

The PCF may use Spending Limits information from the CHF to decide whether to install, update or delete URSP rules, as defined in clause 6.1.1.4.

6.1.2.2.2	Distribution of the policies to UE

The UE policy control enables the PCF to provide UE access selection related policy information, PDU Session related policy information and V2X Policy information to the UE, i.e. UE policies, that includes Access network discovery & selection policy (ANDSP) or UE Route Selection Policy (URSP) or V2X Policy (V2XP) or ProSe Policy (ProSeP) or their combinations using Npcf and Namf service operations.

The PCF may be triggered to provide the UE policy information during UE Policy Association Establishment and UE Policy Association Modification procedures as defined in clause 4.16.11 and clause 4.16.12 of TS 23.502 [3].

NOTE 1:	The PCF can install a PCC Rule and activate start and stop of application detection in the SMF. When the same PCF is selected for SM policy association control and UE policy association control, the reporting of start and stop of an application can trigger the installation or update of a URSP rule in the UE to send the application traffic to the PDU Session as defined in the URSP rule.

NOTE 2:	The PCF can subscribe to the UDR on service specific information change, which will be taken into consideration by the PCF to determine the updated V2XP and ProSeP as defined in clause 4.15.6.7 of TS 23.502 [3].

Operator defined policies in the PCF may depend on input data such as UE location, time of day, information provided by other NFs, etc. as defined in clause 6.2.1.2.

The PCF includes the UE policy information delivered to the UE into a Policy Section identified by a Policy Section Identifier (PSI). The PCF may divide the UE policy information into different Policy Sections, each one identified by a PSI. Each Policy Section provides a list of self-contained UE policy information to the UE, via AMF. The PCF ensures that a Policy Section is under a predefined size limit, known by the PCF.

NOTE 3:	The size limit to allow the policy information to be delivered using NAS transport is specified in TS 29.507 [13]. The size limit is configured in the PCF.

A list of self-contained UE policy information implies that:

-	when the PCF delivers URSP rules to the UE, the PCF provides the list of URSP rules in the order of precedence and without splitting a URSP rule across Policy Sections;

-	when the PCF delivers V2XP to the UE, the PCF provides the list of V2XP in the order of precedence and without splitting a V2XP across Policy Sections;

-	when the PCF delivers ProSeP to the UE, the PCF provides the list of ProSeP in the order of precedence and without splitting a ProSeP across Policy Sections;

-	when the PCF delivers WLANSP rules, the list of WLANSP rules are provided in the order of priority and without splitting a WLANSP rule across Policy Sections;

-	when the PCF delivers the non-3GPP access network selection information, the whole list of non-3GPP access network selection information (as defined in clause 6.6.1.1) is provided in one Policy Section.

It is up to PCF decision how to divide the UE policy information into Policy Sections as long as the requirements for the predefined size limit and the self-contained content (described above) are fulfilled.

NOTE 4:	The Policy Section list can be different per user. One PSI and its corresponding content can be the same for one or more users.

NOTE 5:	The PCF may, for example, assign the URSP as one whole Policy Section, or it may subdivide the information in the URSP into multiple Policy Sections by assigning one or several URSP rules to each Policy Section.

The PLMN ID is provided to the UE together with UE policy information and it is used to indicate which PLMN a Policy Section list belongs to.

The AMF forwards the UE policy information transparently to the UE. If the (H-)PCF decides to split the UE policies to be sent to the UE, the PCF provides multiple Policy Sections separately to the AMF and then AMF uses UE configuration Update procedure for transparent UE policies delivery procedure to deliver the policies to the UE, this is defined in clauses 4.2.4.3 and 4.16 of TS 23.502 [3].

NOTE 6:	The AMF does not need to understand the content of the UE policy, rather send them to the UE for storage.

The UE shall update the stored UE policy information with the one provided by the PCF as follows (details are specified in TS 24.501 [22]):

-	If the UE has no Policy Sections with the same PSI, the UE stores the Policy Section;

-	If the UE has an existing Policy Section with the same PSI, the UE replaces the stored Policy Section with the received information;

-	The UE removes the stored Policy Section if the received information contains only the PSI.

The UE keeps the received UE policies stored even when registering in another PLMN. The number of UE policies to be kept stored in the UE for PLMNs other than the HPLMN is up to UE implementation. If necessary, e.g. the number of UE policies stored in UE for PLMNs exceeds the maximum value, the UE may remove earlier stored UE policy in UE.

NOTE 7:	For aspects related to URSP rules for an SNPN-enabled UE, please refer to clause 6.6.2.2.2.

The ANDSP for VPLMN, if provided within the UE policy in the UE Configuration Update procedure described in clause 4.2.4.3 of TS 23.502 [3], applies to the equivalent PLMN(s) indicated in the last received list of equivalent PLMNs in Registration Accept.

At Initial Registration or the Registration to 5GS when the UE moves from EPS to 5GS:

-	The UE provides the list of stored PSIs which identify the Policy Sections associated to the home PLMN and the visited PLMN (if the UE is roaming) that are currently stored in the UE. If USIM is changed, the UE does not provide any PSI. If no policies are stored in the UE for the home PLMN, the UE does not provide any PSI associated to the home PLMN. If the UE is roaming and has policies for the home PLMN but no associated policies for the visited PLMN the UE includes only the list of PSIs associated to the home PLMN.

-	UE may indicate its ANDSP support to the PCF. If it is received, the PCF shall take it into account for the determination on whether to provide the ANDSP to the UE. The PCF does not provide ANDSP rules to the UE if the UE does not indicate support for ANDSP.

-	UE may indicate its capability of support to report URSP rule enforcement to network to the PCF. If it is received, the PCF shall take it into account as described in clause 6.6.2.4.

NOTE 8:	In the roaming scenario, during AMF relocation with V-PCF change, if the H-PCF does not provide the Indication of UE support for ANDSP, then the behaviour of V-PCF to determine whether to provide ANDSP rules to the UE is implementation specific.

-	The UE may also provide the OSId.

-	The UE may indicate whether it supports the URSP Provisioning when attached in EPS. The PCF for the UE stores it at the UDR, see clause 6.2.1.3.

The UE may trigger an Initial registration with the list of stored PSIs to request a synchronization for example if the UE powers up without USIM being changed.

During Initial Registration, the (H-)PCF retrieves the list of PSIs and its content stored in the (H-)UDR for this SUPI while the V-PCF (in the roaming scenario) retrieves the list of PSIs and its content stored in the V-UDR for the PLMN ID of this UE (alternatively, the V-PCF can have this information configured locally).

NOTE 9:	The PSI list and content stored/configured for a PLMN ID can be structured according to e.g. location areas (e.g. TAs, PRAs). The V-PCF can then provide PSIs and its content only if they correspond to the current UE location.

The PCF provides to the UE the tuple (PLMN ID, list of PSIs associated with the PLMN ID) and the Policy Sections containing URSP Rules. In roaming scenarios, the H-PCF provides this information via V-PCF.

In the roaming scenario, the V-PCF shall also forward any UE provided PSIs that are associated to the home PLMN to the H-PCF.

When the PCF (i.e. the (H-)PCF as well as the V-PCF) receives a list of PSIs associated to the PLMN of the PCF from the UE, the PCF compares the list of PSIs provided by the UE and the list of PSIs retrieved from the UDR. In addition, the PCF checks whether the list of PSIs provided by the UE or its content needs to be updated according to operator policies, e.g. change of Location and/or time. If the two lists of PSIs are different or an update is necessary according to operator policies (which includes the case that the UE did not provide a list of PSIs associated to the PLMN of the PCF), the PCF provides the changes in the list of PSIs or the corresponding content to the AMF which forwards them to the UE.

The (H-)PCF maintains the latest list of PSIs and the Tuple (if any), delivered to each UE as part of the information related to the Policy Association until the UE policy association termination request is received from the AMF. Then the (H-)PCF stores the latest list of PSIs, its contents, and the Tuple (if any) in the (H-)UDR using the Nudr_DM_Update including DataSet "Policy Data" and Data Subset "Policy Set Entry".

The (H-)PCF may use the PEI provided by the AMF and/or the OSId provided by the UE, to determine the operating system of the UE.

If the PEI, the OSId or the indication of UE support for ANDSP is available to the (H-)PCF, the (H-)PCF stores them in the (H-)UDR using Nudr_DM_Create including DataSet "Policy Data" and Data Subset "UE context policy control data" when such information is received from the UE in the UE Policy Container.

If the (H-)PCF is not able to determine the operating system of the UE, and if the (H-)PCF requires to deliver URSP rules that contain Application descriptors as Traffic Descriptors, then the Traffic Descriptors of such URSP rules include multiple instances of Application descriptors each associated to supported UE operating systems by the network operator implementation.

If the (H-)PCF determines the operating system of the UE and if the (H-)PCF requires to deliver URSP rules that contain Application descriptors as Traffic Descriptors, then the Traffic Descriptors of such URSP rules include the Application descriptors associated with the operating system determined by the PCF.

NOTE 10:	If the PCF does not take into account the received PEI and/or OSId then the PCF can send URSP rules containing application traffic descriptors associated to multiple operating systems.

After Registration procedure, the UE may perform UE triggered V2X Policy Provisioning procedure to request V2XP from the PCF as specified in clause 6.2.4 of TS 23.287 [28].

After Registration procedure, the UE may perform UE triggered 5G ProSe Policy Provisioning procedure to request ProSeP from the PCF as specified in clause 6.2.4 of TS 23.304 [34].

# * * * * End of changes * * * *