SA WG2 Meeting #157	S2-2306593

22 - 26 May, 2023, Berlin, Germany

Source:	Interdigital

Title:	Update on 6.7.1 Procedures for service exposure to SL Positioning Client UE

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL/Rel-18

Abstract of the contribution: This contribution proposes update of procedures for service exposure to client UE over PC5 for TS 23.586v0.3.0.

# 1.	Discussion

For service exposure to SL positioning client UE, the following needs to be updated.

(1) On discovery of target UE and SL reference UE for SL positioning and UEs for Ranging.

Client UE needs to discover target UE and SL reference UE in proximity of target UE. But, existing direct discovery is not clear how to discover SL reference UE in proximity of target UE.

When client UE needs to discover UEs for Ranging, how to discover UEs for Ranging is not clear.

(2) Selection of LMF or SL Positioning Server UE

Client UE sends a SL positioning service request to the discovered UE but when multiple UEs are discovered, client UE select a UE to send SL positioning service request.

For selection of UEs for better SL positioning operation, UE’s status (i.e. whether UE has NAS connection or not) should be known.

(3) parameter provisioning for SL positioning service request.

Last meeting, it was included that proximity information of UE1 and UE2 for ranging and SL Reference UE’s information for SL positioning are informed during discovery. But based on decision of discovery, that information cannot be shared during discovery. Therefore, they are moved to be informed over PC5 connection setup.

# 2.	Text proposal

It is proposed to agree the following changes to TS 23.586 (all texts are new):

# * * * * First change * * * *

### 6.7.1	Procedures for service exposure to SL Positioning Client UE

#### 6.7.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.7.1-1.	Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.2 with following updates:

- When SL positioning service is triggered, the client UE tries to discover the target UE and/or a SL Reference UE. When Model B discovery used, in the Solicitation message, target UE’s user info is included and role of target UE and role of SL Reference UE are indicated in the RSPP metadata. Only the target UE or  SL Reference UE(s) in proximity of the target UE may respond.

- When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2. When Model B discovery used, in the Solicitation message, UE1’s user info and UE2’s user info are included and role of target UE is indicated in the RSPP metadata.

During discovery for SL positioning service, the information of SL reference UE is acquired.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes PC5 connection(s) with the discovered UE(s).

3.	The SL Positioning Client UE receives UE’s status information indicating that UE has NAS connection or no NAS connection from the discovered UE(s).

The SL Positioning Client UE may receive list of SL Reference UE(s) from target UE over the PC5 connection.

For Ranging service, if UE1 and UE2 are not in proximity, UE1 and/or UE2 may informe that over the PC5 connection to the client UE.

4.	Based on UE’s status information, the SL Positioning Client UE downselects UE(s) to perform SL Positioning Service Request according and select a UE to receive SL Positioning Service Request. The SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE and info of SL Reference UE for SL positioning, and info of SL Positioning Client UE, info of UE1 and info of UE2 for Ranging.

4. Authorization check of the client UE for the service request may be performed via SL Positioning Server UE or 5GC or internally by UE1 or target UE.

5. The UE which received SL Positioning Service Request selects LMF or SL Positioning Server UE based on its status whether UE has NAS connection. Ranging/SL Positioning operation is performed as described in clause 6.8 among target UE, SL Reference UE and SL Positioning Server UE or LMF.

6.	The Ranging/SL Positioning result is provided to the SL Positioning client UE by UE1.

# * * * * End of change * * * *